3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2201993r02

# Elbonia, April 6 – 12, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0510 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on historical data and analytics storage via MFAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Huawei, Hisilicon, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In clause 6.2B.3, MFAF is not a consumer of DataManagement_subscribe service and not used by DCCF or NWDAF to process data, thus, MFAF should not be in figure 6.2B.3-1 nor description of step 6. In addition, the description in step 4, the ADRF sends Nadrf_DataManagement_StorageRequest Response, is not corresponding with the figure 6.2B.3-1. - It is described in clause 6.2B.2 as follows, which means that MFAF can be one of consumers of ADRF. “The procedure depicted in figure 6.2B.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.” However, MFAF is not contained in example consumers in the first row of Table 10.1-1. - DCCF provides ADRF ID to MFAF for historical data and/or analytics storage via MFAF but it is not clear how to use ADRF ID in clause 9.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete MFAF from figure 6.2B.3-1 and step 6 in clause 6.2B.3. - Change “Nadrf_DataManagement_StorageRequest” to “Nadrf_DataManagement_StorageSubscriptionRequest” in step 4 in clause 6.2B.3. - Add explanation related to how to use ADRF ID in 9.2.2; - Add MFAF as an example consumer in Table 10.1-1; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Ambiguous descriptions and figure in clause 6.2B.3. - Unclear on why “ADRF ID” provided in Nmfaf_3daDataManagement_Configure service; - Inconsistent on consumers of Nadrf_DataManagement_StorageRequest Service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3, 9.2.2, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.2B.3	Historical Data and Analytics Storage via Notifications

The procedure depicted in figure 6.2B.3-1 is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF. The consumer requests the ADRF to initiate a subscription for data and/or analytics. Data and/or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

Figure 6.2B.3-1: Historical Data and Analytics Storage via Notifications

1a-d.	Based on provisioning or based on reception of a DataManagement subscription request (e.g. see clause 6.2.6.3.2), the DCCF or the NWDAF determines that notifications are to be stored in an ADRF.

2a-b.	The DCCF or the NWDAF determines the ADRF where data and/or analytics needs to be stored, and requests that the ADRF subscribes to receive notifications. The determination may be made based on configuration or information supplied by the data consumer as described in clauses 6.1.4 and 6.2.6.3. The request to the ADRF specifies the data and/or analytics to which the ADRF will subscribe by invoking the Nadrf_DataManagement_StorageSubscriptionRequest service operation.

3.	[Optional] The ADRF may, based on implementation, determines whether the same data and/or analytics is already stored or being stored, based on the information sent in step 2 by the consumer.

4.	[Optional] If the data and/or analytics is already stored and/or being stored in the ADRF, the ADRF sends  Nadrf_DataManagement_StorageSubscriptionRequest Response message to the consumer indicating that data and/or analytics is stored.

5a-b.	ADRF subscribes to the DCCF or the NWDAF to receive notifications, providing its notification endpoint address and a notification correlation ID.

6.	The DCCF, the MFAF or the NWDAF sends Analytics or Data notifications containing the notification correlation ID provided by the ADRF to ADRF notification endpoint address. The ADRF stores the notifications.

7a-b.	The DCCF or the NWDAF determines that notifications no longer need to be stored in the ADRF.

8a-b.	The DCCF or the NWDAF requests that the ADRF unsubscribes to receive notifications.

9a-b.	The ADRF sends a request to the DCCF or the NWDAF to unsubscribe to data notifications.

The NWDAF may interact with the Data Source, and the DCCF may interact with the Data Source and/or MFAF. Delivery notifications from the DCCF/MFAF or NWDAF to the ADRF are subsequently halted.

# * * * * Next change * * * *

### 9.2.2	Nmfaf_3daDataManagement_Configure service operation

Service operation name: Nmfaf_3daDataManagement_Configure

Description: The consumer configures or reconfigures the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the out-bound data or analytics.

Inputs, Required: Data Consumer or Analytics Consumer Information.

"Data Consumer or Analytics Consumer Information" contains for each notification endpoint, the consumer provided Notification Target Address (+ Analytics Consumer Notification Correlation ID) or other endpoint addresses if provisioned on the DCCF to be used by the MFAF when sending notifications.

Inputs, Optional: Formatting Instructions, Processing Instructions, MFAF Notification Information, Transaction Reference Id, ADRF ID.

"MFAF Notification Information" is used to identify Event Notifications received from a Data Source and comprises the MFAF Notification Target Address (+ MFAF Notification Correlation ID). If a Data Source is already supplying the data to the MFAF, the MFAF Notification Information previously provided by the MFAF and used by the DCCF to obtain data from a Data Source is provided as an Input. If a new subscription to a Data Source is needed, the MFAF Notification Information is not specified as an Input and the MFAF provides Notification Information as an output. The MFAF Notification Information may subsequently be used by the DCCF when subscribing to a Data Source. “ADRF ID” is used to identify the ADRF when DCCF requests that the Messaging Framework to store historical data and analytics in the ADRF via Nadrf_DataManagement_StorageRequest. When a Notification Correlation ID is provided, MFAF shall use the Nmfaf_3caDataManagement_Notify service with the notification correlation ID to send the data or analytics to the ADRF.

Outputs Required: Operation execution result indication.

Outputs, Optional: MFAF Notification Information, Transaction Reference Id.

# * * * * Next change * * * *

## 10.1	General

Table 10.1-1 shows the ADRF services and ADRF service operations.

ADRF service operations may be used to store data or analytics in the ADRF, retrieve data or analytics from an ADRF, or delete data or analytics from an ADRF.

NOTE:	Storage of ML models in ADRF is not specified in this Release of the specification.

Table 10.1-1: NF services provided by ADRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadrf_DataManagement | StorageRequest | Request / Response | DCCF, NWDAF, MFAF |
|  | StorageSubscriptionRequest | Request / Response | DCCF, NWDAF |
|  | StorageSubscriptionRemoval | Request / Response | DCCF, NWDAF |
|  | RetrievalRequest | Request / Response | DCCF, NWDAF |
|  | RetrievalSubscribe | Subscribe / Notify | DCCF, NWDAF |
|  | RetrievalUnsubscribe |  | DCCF, NWDAF |
|  | RetrievalNotify |  | DCCF, NWDAF |
|  | Delete | Request / Response | DCCF, NWDAF |

# * * * * End of changes * * * *