3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202346r04

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for KI#6 on ranging and sidelink positioning service exposure to UE

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: the KI#6 is captured in TR 23.700-86 to study the ranging and sidelink positioning service exposure to UE, thus it proposes a solution to enable such service exposure.

1. Introduction

In SA2#149 meeting, it is approved to separate the ranging and sidelink positioning service exposure into two KIs, i.e. Key Issue #6 for Ranging/Sidelink positioning service exposure to a UE, and Key Issue #7 for Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning. This contribution proposes the end-to-end procedures for Key Issue #6.

2. Discussion

The followings discuss two potential cases:

- Case1: two UEs, the ranging UE consumes the ranging/sidelink positioning service.

- Case2: three UEs, the 3rd party UE consumes the ranging/sidelink positioning service.

2.1 Case1: UE involved in SL ranging consumes the ranging/sidelink positioning service

RAN

UE1

5GC

UE2

1. SL Ranging execution

2. Local calculation

RAN

UE1

5GC

UE2

1. SL Ranging execution

2. Local calculation

Figure 2.1-1: UE involved in SL ranging consumes the ranging/sidelink positioning service

In this case, two UEs are involved in the ranging/sidelink positioning. The UE triggers ranging service, performs the ranging/sidelink positioning operation, and calculates the result locally, i.e. Ranging/Sidelink positioning service is exposed to one of the two UEs.

2.2 Case2: 3rd party UE consumes the ranging/sidelink positioning service.

1a. 3rd party UE requests via PC5

3rd party UE

UE 1

UE 2

RAN

5GC

1b. 3rd party UE requests via 5GC

3. Ranging result exposure

2. SL Ranging execution

1a. 3rd party UE requests via PC5

3rd party UE

UE 1

UE 2

RAN

5GC

1b. 3rd party UE requests via 5GC

3. Ranging result exposure

2. SL Ranging execution

Figure 2.2-1: 3rd party UE consumes the ranging/sidelink positioning service

In this case, three UEs are involved in the ranging/sidelink positioning, in which a 3rd party UE asks for the ranging/sidelink positioning for the other two UEs (i.e., UE1 and UE2). The 3rd party UE may request the 5GC (e.g. LMF) via Uu interface to perform ranging/sidelink positioning between UE1 and UE2. If the 3rd party UE is in the sidelink coverage with one of the requested UEs (e.g., UE1), the 3rd party UE may alternatively request UE1 via PC5 interface to perform ranging/sidelink positioning with UE2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 6 |  |  |  |
| x | X |  |  |  |

# * * * * Second change * * * *

6.x	Solution #x: Solution for KI#6 on Ranging/SL positioning service exposure to UE

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the subclauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

6.x.1	General

The solutions address the ranging and sidelink positioning service exposure to UE for Key Issue #6.

The followings give two potential cases and the corresponding solutions:

- Case1: two UEs, the UE involved in Ranging/SL positioning consumes the Ranging/SL positioning service.

- Case2: three UEs, the 3rd party UE consumes the Ranging/SL positioning service.

6.x.2	Functional descriptions

Ranging/SL positioning refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

The consumer can be the UE involved in the Ranging/SL positioning service, or a 3rd party UE. The UE/3rd party UE may perform the Ranging/SL positioning operation directly via PC5 interface and use the result locally, without involvement of 5GC, or the 3rd party UE (e.g. out of SL coverage of either UE subject to Ranging/SL positioning)  requests  Ranging/Sidelink positioning service with the involvement of 5GC (e.g. AMF, LMF, etc.).

6.x.3	Procedures

6.x.3.1	UE involved in SL ranging consumes the Ranging/SL positioning service

RAN

UE1

5GC

UE2

1. SL Ranging execution

2. Local calculation

RAN

UE1

5GC

UE2

1. SL Ranging execution

2. Local calculation

Figure 6.x.3.1-1: UE involved in SL ranging consumes the Ranging/SL positioning service

In this case, two UEs are involved in the Ranging/SL positioning operation. The UE triggers ranging service, performs the Ranging/SL positioning operation, and makes the calculation locally, i.e. Ranging/SL positioning service is exposed to one of the two UEs.

UE2

NG-RAN

AMF

LMF

–UE1

2. Ranging (and SL positioning)

1. Ranging service triggers

3. Calculation

UE2

NG-RAN

AMF

LMF

–UE1

2. Ranging (and SL positioning)

1. Ranging service triggers

3. Calculation

Figure 6.x.3.1-2: Procedure of ranging UE consumes the Ranging/SL positioning service

- The application layer of the UE1 triggers the request for Ranging/SL positioning.

2.  The UE1 initiates the Ranging/SL positioning operation with the UE2. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE 1: the role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

Editor’s Note: the security and privacy consideration for the Ranging/SL positioning operation should be studied by SA3.

3. The UE1 calculates the result and uses it locally.

6.x.3.2	3rd party UE consumes the Ranging/SL positioning service.

6.x.3.2.1	General

1a. 3rd party UE requests via PC5

3rd party UE

UE 1

UE 2

RAN

5GC

1b. 3rd party UE requests via 5GC

3. Ranging result exposure

2. SL Ranging execution

1a. 3rd party UE requests via PC5

3rd party UE

UE 1

UE 2

RAN

5GC

1b. 3rd party UE requests via 5GC

3. Ranging result exposure

2. SL Ranging execution

Figure 6.x.3.2.1-1: 3rd party UE consumes the Ranging/SL positioning service

In this case, three UEs are involved in the Ranging/SL positioning, in which a 3rd party UE asks for the Ranging/sSL positioning for the other two UEs (i.e., UE1 and UE2). The 3rd party UE may request via Uu interface to perform Ranging/SL positioning operation between UE1 and UE2 with the involvement of 5GC. If the 3rd party UE is in the sidelink coverage with one of the requested UEs (e.g., UE1), the 3rd party UE may alternatively request UE1 via PC5 interface to perform Ranging/SL positioning with UE2.

When 5GC is involved, there are network based Ranging/Sidelink positioning in which the Ranging/Sidelink positioning estimation result calculation is performed at LMF and UE based Ranging/Sidelink positioning in which the Ranging/Sidelink positioning estimation result calculation is performed at the UE.

6.x.3.2.2	Service exposure to the 3rd party UE via PC5

–UE2

NG-RAN

AMF

LMF

–UE1

3. Calculation

3rd party UE

1. SL ranging request (SL ranging measurement)

4. SL ranging response (result)

2. Ranging (and SL positioning) procedure

–UE2

NG-RAN

AMF

LMF

–UE1

3. Calculation

3rd party UE

1. SL ranging request (SL ranging measurement)

4. SL ranging response (result)

2. Ranging (and SL positioning) procedure

Figure 6.x.3.2.2-1: Service exposure to the 3rd party UE via PC5

- The application layer of the 3rd party UE triggers the request for the Ranging/SL positioning, and the 3rd party UE requests the UE1 to perform the ranging/sidelink positioning operation with UE2.

2. The UE1 initiates the Ranging/SL positioning operation with the UE2. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE 2: the role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

Editor’s Note: the security and privacy consideration for the Ranging/SL positioning operation should be studied by SA3.

3. The  UE1 makes the calculation locally.

4. The  UE2 returns the Ranging/SL positioning result to the 3rd party UE via the PC5 interface.

6.x.3.2.3	Service exposure to the 3rd party UE via 5GC/Uu (network based)

3rd party UE

NG-RAN

AMF

LMF

GMLC

2. UL NAS TRANSPORT (Ranging/SL positioning requirement, UE1 ID, UE2 ID)

NEF

AF

UE1

UE2

5. SL measurement request (Ranging/SL positioning demand, UE1 ID,  UE2 ID)

6. UE Ranging with other UEs

10. DL NAS TRANSPORT (Ranging/SL positioning result)

7. SL measurement report (UE1 ID, UE2 ID, Ranging/SL positioning measurement/result)

4. Ranging request (Ranging/SL positioningrequirement,  UE1 ID, UE2 ID)

8. SL Ranging calculation

9. Ranging response (Ranging/SL positioning result)

1. UE Triggered Service Request

3. LMF selection

3rd party UE

NG-RAN

AMF

LMF

GMLC

2. UL NAS TRANSPORT (Ranging/SL positioning requirement, UE1 ID, UE2 ID)

NEF

AF

UE1

UE2

5. SL measurement request (Ranging/SL positioning demand, UE1 ID,  UE2 ID)

6. UE Ranging with other UEs

10. DL NAS TRANSPORT (Ranging/SL positioning result)

7. SL measurement report (UE1 ID, UE2 ID, Ranging/SL positioning measurement/result)

4. Ranging request (Ranging/SL positioningrequirement,  UE1 ID, UE2 ID)

8. SL Ranging calculation

9. Ranging response (Ranging/SL positioning result)

1. UE Triggered Service Request

3. LMF selection

Figure 6.x.3.2.3-1: Service exposure to the 3rd party UE via 5GC/Uu (network based)

1. The application layer of the 3rd party UE triggers the Ranging/SL positioning, and then requests the network to perform the Ranging/SL positioning between the UE1 and UE2. If the 3rd party UE in CM-IDLE state, it firstly triggers the service request procedure.

2. The 3rd party UE requests the network to perform the Ranging/SL positioning operation between the UE1 and UE2. The Ranging/SL positioning requirement may include the requirement regarding Ranging/SL positioning demand (i.e. the distance and/or the direction between two UEs or more UEs), delay, accuracy, result feedback mode (e.g. immediately, periodically or at a later time).

3. The AMF selects an appropriated LMF, details see TS 23.273 [xx].

4. If the 3rd party UE is authorized to request the Ranging/SL positioning information between the UE1 and UE2 based on the subscription of 3rd party UE, the AMF requests the LMF for the Ranging/SL positioning between theUE1 and UE2.

5. The LMF selects and triggers the UE2 to perform Ranging/SL positioning with the UE1. The measurement request message may include the Ranging/SL positioning demand, UE1 ID, UE2 ID, etc.

Editor's Note: It is FFS that why and how the LMF selects the UE2 (or UE1) to perform Ranging/SL positioning operation.

6. The UE2 performs theRanging/SL positioning operation with the UE1. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocationis used.

NOTE 3: the role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

Editor’s Note: the security and privacy consideration for the Ranging/SL positioning operation should be studied by SA3.

7. The UE2 reports the Ranging/SL positioning measurements to the LMF for calculation. When the Ranging/SL positioning calculation is performed by UE, the Ranging/SL positioning result is returned.

8. The LMF makes calculation based on the Ranging/SL positioning measurements.

9. The LMF returns the Ranging/SL positioning result to the AMF.

10. The AMF returns the Ranging/SL positioning result to the 3rd party UE via a NAS message.

6.x.4	Impacts on services, entities, and interfaces

UE:

-	sends Ranging/SL positioning requirement to another UE for Ranging/SL positioning measurement or to the LMF for Ranging/SL positioning result.

-	receives Ranging/SL positioning measurement from another UE or Ranging/SL positioning result from the LMF (via the AMF).

-	receives Ranging/SL positioning measurement request from the LMF, performs Ranging/SL positioning measurement, and reports the Ranging/SL positioning measurement to the LMF.

NG-RAN:

-	allocates PC5 radio resource to UE for Ranging/SL positioning operation.

AMF:

-	selects LMF and routes the Ranging/SL positioning requirement to the LMF.

-	routes the signalling (e.g. LPP messages for SL ranging) between the UE and the LMF.

-	Support check the 3rd party UE is authorized to request the Ranging/SL positioning information between the UE1 and UE2.

LMF:

-	requests and controls the UE to perform Ranging/SL positioning measurement.

-	receives the Ranging/SL positioning measurement from the UE and makes calculation.

-	returns the Ranging/SL positioning result to the UE via AMF.

# * * * * End of changes * * * *