SA WG2 Meeting #150e	S2-2202770r01

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	Revision of Solution#1 in TR 23.700-62 v0.1.0

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: It provides revision of Solution#1 in TR 23.700-62 v0.1.0.

1 Discussion

The paper provides some revisions for Solution #1 in TR 23.700-62 v0.1.0.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************Start of change ****************************

6.1	Solution #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF

6.1.1	Description

The solution introduces the service based UPF event exposure framework to support registration, deregistration and discovery via NRF. The following Figure 6.1.2-1 depicts the service-based interface Nupf introduced in the 5G system architecture.

The solution for registering UPF in NRF is based on the option in the existing solution described in clause 6.3.3.2 of TS 23.501 [2] and clause 4.17 of TS 23.502 [3], whereby the UPF registers directly with the NRF and hence does not use N4 for registering to NRF.

NOTE 1:	As described in TS 23.501 [2], the NRF can alternatively be configured by OAM with information on the available UPF(s) or the UPF instance(s) may register its/their NF profile(s) in the NRF.

Figure 6.1.1-1: 5G system architecture with service based UPF

NOTE 2:	Figure 6.1.1-1 shows an example of UPF with Nupf service. In the context of this solution the UPF is in the role of consumer of NRF services, i.e. this solution is about how UPF can register its NF profile in NRF with related Nupf service information and does not describe services provided by the UPF itself.

6.1.2	Procedures

6.1.2.1	UPF Event Exposure service Registration

The following Figure 6.1.2.1-1 depicts the UPF Event Exposure service Registration procedure.

Figure 6.1.2.1-1: UPF Event Exposure service Registration procedure

1.	The UPF sends the Nnrf_NFManagement_NFRegister Request message to NRF to inform the NRF of its NF profile when the NF service consumer becomes operative for the first time. The existing UPF NF profile parameters include e.g. S-NSSAI(s) and the associated NSI ID(s), DNN(s), Area of interest, IP range, information about the location of the UPF (operator specific information, e.g., geographical location, data centre), UPF Service Area (TAI List), DNAI, as described in TS 29.510 [6].In addition, to support UPF Event Exposure Service, also Event Exposure Service Name is provided with the UPF NF profile.

2.	The NRF stores the UPF profile and marks the UPF Event Exposure service as available.

3.	The NRF acknowledge UPF Registration is accepted via Nnrf_NFManagement_NFRegister response.

6.1.2.2	UPF Event Exposure service Update

The following Figure 6.1.2.2-1 depicts the UPF Event Exposure service Update procedure.

Figure 6.1.2.2-1: UPF Event Exposure service Update procedure

1.	UPF sends Nnrf_NFManagement_NFUpdate Request message (the updated NF profile of NF service consumer) to NRF to inform the NRF of its updated UPF profile.

2.	The NRF updates the NF profile of UPF instance.

3.	The NRF acknowledge UPF Update is accepted via Nnrf_NFManagement_NFUpdate response.

6.1.2.3	UPF Event Exposure service Deregistration

The following Figure 6.1.2.3-1 depicts the UPF Event Exposure service Deregistration procedure.

Figure 6.1.2.3-1: UPF Event Exposure service Deregistration procedure

1.	UPF sends Nnrf_NFManagement_NFDeregister Request message to NRF to inform the NRF of its unavailability.

2.	The NRF marks the UPF unavailable. NRF may remove the NF profile of UPF according to NF management policy.

3.	The NRF acknowledge NF Deregistration is accepted via Nnrf_NFManagement_NFDeregister response.

6.1.2.4	UPF Event Exposure service Discovery

The following Figure 6.1.2.4-1 depicts the UPF Event Exposure service Discovery procedure.

Figure 6.1.3.4-1: UPF Event Exposure service Discovery procedure

1.	Service consumer NF that requires UPF services invokes Nnrf_NFDiscovery_Request message to NRF with the intent to discover UPF. The input may include e.g. UPF service name, TAI, NF type (i.e. UPF), S-NSSAI, DNN, Area of interest, DNAI, as described in TS 29.510 [6].

2.	The NRF authorizes the Nnrf_NFDiscovery_Request and based on the UPF profile the NRF determines if the service consumer of the UPF is allowed to discover the UPF.

3.	If allowed, the NRF determines a set of UPFs matching the input parameters included in the Nnrf_NFDiscovery_Request to the service consumer of the UPF via Nnrf_NFDiscovery_Request_Response. The output includes one or more UPF instances, and for each UPF instance it includes UPF NF profile.

6.1.2.5	UPF Selection for a UPF Event Exposure Service Request

6.1.2.5.1	Procedure of UPF selection by the NF targeting PDU session or UE with information of IP address

If the consumer NF targets a specific PDU session or UE with information of IP address, it can find the UPF in two ways.

Figure 6.1.2.5.1-1: Procedure of UPF selection by the NF targeting PDU session or UE with information of IP address

1.	If the consumer NF is an AF/NEF, it can use the UE address to discover the PCF from the BSF. Then the PCF can response the SMF list for the PDU sessions to the AF/NEF.

2.	The AF/NEF interacts with the SMF that is responded by the PCF in step 1 to find the appropriate UPF over the Nsmf_EventExposure_Subscribe service operation providing NF type, NF instance ID, FQDN or IP address of NF, PLMN ID.

3.	The SMF responds Nsmf_EventExposure_Notify with the list of UPFs.

Figure 6.1.2.5.1-2: Procedure of UPF locating by the NF targeting PDU session or UE through UPF registration information in NRF

0.	The UPF sends its IP range to the NRF during the UPF Event Exposure service registration procedure as described in clause 6.1.2.1.

1.	If the consumer NF is an AF/NEF, the AF/NEF issues an Nnrf_NFDiscovery_Request service operation to find the appropriate UPF providing its NF type, NF instance ID, FQDN or IP address of NF, PLMN ID, UE IP address range.

2.	The NRF responds Nnrf_NFDiscovery_Request with the list of all UPFs that currently meet the AF/NEF request.

6.1.2.5.2	Procedure of UPF selection by the NF targeting PDU session or UE with information of SUPI, S-NSSAI and DNN

Figure 6.1.2.5.2-1: Procedure of UPF locating by the NF targeting PDU sessions of a certain UE with information of SUPI, S-NSSAI and DNN

1.	If the consumer NF is an NWDAF, the NWDAF issues an Nnrf_NFDiscovery_Request service operation to find the UDM providing the NF type, NF instance ID, UE ID (SUPI).

2.	The NRF responds Nnrf_NFDiscovery_Request with the UDM that currently meet the NWDAF request.

3.	The NWDAF issues an Nudm_EventExposure_Subscribe service operation to find the SMF from UDM providing NF type, NF instance ID, UE ID (SUPI), S-NSSAI, DNN.

4.	The UDM find the serving SMF for the specific PDU or UE providing SUPI, S-NSSAI, and DNN, as described in TS 23.502 [3].

[Optional] The UDM may find the SMF via the AMF. The UDM stores the AMF information for a specific UE.

5.	The UDM responds the requested SMF over Nudm_EventExposure_Notify service operation to the NWDAF.

6.	The NWDAF finds the appropriate UPF from the SMF that responded by the UDM over the Nsmf_EventExposure_Subscribe service operation providing NF type, NF instance ID, SUPI, S-NSSAI, and DNN.

7.	The SMF responds Nsmf_EventExposure_Notify with the requested UPF.

6.1.2.5.3	Procedure of UPF selection by the NF with information of S-NSSAI, DNN or DNAI

Figure 6.1.2.5.3-1: Procedure of UPF selection by the NF with information of S-NSSAI, DNN or DNAI

0.	The UPF sends its S-NSSAI, DNN or DNAI to the NRF during the UPF Event Exposure service registration procedure as described in clause 6.1.2.1.

1.	If the consumer NF is an NWDAF or PCF or NEF, the NWDAF/PCF/NEF issues an Nnrf_NFDiscovery_Request service operation to find the appropriate UPF providing its NF type, NF instance ID, S-NSSAI, DNN, DNAI, Area of interest.

2.	The NRF responds Nnrf_NFDiscovery_Request with the list of all UPFs that currently meet the NWDAF/PCF/NEF request.

**End of change**