3GPP TSG-SA2 Meeting #150E	S2-2202045r01

# Elbonia,  April 06 – 12, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0512 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inputs update for Nnwdaf Notify services for missing elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | R17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.2.6.1.1, it describes The input parameters of Nnwdaf_DataManagement_Notify or Ndccf_DataManagement_Notify, or Nmfaf_3caDataManagement_Notify, or Nadrf_DataManagement_RetrievalNotify service operation shall contain the Notification Correlation ID, and the generated bulked data when the fetch flag = false. When the fetch flag = true the notifications will contain the Notification Correlation ID, the Fetch Correlation ID and a target address where the generated bulked data may be retrieved. In the case of unsuccessful bulked data generation, the notification will contain an indication of an unsuccessful bulked data generation, optionally with expired bulked data deadline. However, in clause 7, only the Nnwdaf_DataManagement_Notify service has the unsuccessful bulked data generation and expired bulked data deadline in Inputs Optional section. For the other three notify services, these two elements is missing in corresponding clauses. This CR is to complete the Inputs Optional for services Ndccf_DataManagement_Notifiy, Nmfaf_3caDataManagement_Notify and Nadrf_DataManagement_RetrevalNotify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add unsuccessful bulked data generation and expired bulked data deadline in input optional for services Ndccf_dataManagement_Notify, Nmfaf_3caDataMangement_Notify, Nadrf_DataMangement_RetirevalNotify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing input elements for unsucessful bulked data generation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4, 9.3.2, 10.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

### 8.2.4	Ndccf_DataManagement_Notify service operation

Service operation name: Ndccf_DataManagement_Notify

Description: Provides the previously subscribed Data or Analytics, or notification of availability of previously subscribed Data or Analytics to the NF Consumer when data delivery is via the DCCF.

Inputs, Required: Notification Correlation Information.

Inputs, Optional: Data or Analytics, Fetch Instructions, Unsuccessful bulked data generation, Expired bulked data deadline

Fetch Instructions indicate whether the data or analytics are to be fetched by the Consumer. If the data or analytics are to be fetched, the fetch instructions include an address from which the data may be fetched and one or more Fetch Correlation IDs.

NOTE:	Data or Analytics provided in notifications are processed and formatted according to the Processing and Formatting Instructions provided by the Consumer in Ndccf_DataManagement_Subscribe.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * 2nd Change * * *

### 9.3.2	Nmfaf_3caDataManagement_Notify service operation

Service operation name: Nmfaf_3caDataManagement_Notify

Description: Provides data or analytics or notification of availability of data or analytics to notification endpoints.

Inputs, Required: Notification Correlation Information.

Inputs, Optional: Data or Analytics, Fetch Instructions, Unsuccessful bulked data generation, Expired bulked data deadline

Fetch Instructions indicate whether the data or analytics are to be fetched by the Consumer. If the data or analytics are to be fetched, the fetch instructions include an address from which the data may be fetched and one or more Fetch Correlation IDs.

NOTE:	Data or Analytics provided in notifications can be processed and formatted according to the Processing and Formatting Instructions provided by the Consumer.

Outputs, Required: None.

Outputs, Optional: None.

* * * 3rd Change * * *

### 10.2.8	Nadrf_DataManagement_RetrievalNotify service operation

Service operation name: Nadrf_DataManagement_RetrievalNotify

Description: This service operation provides consumers with either data or analytics from an ADRF, or instructions to fetch the data or analytics from an ADRF. The notifications are provided to consumers that have subscribed using the Nadrf_DataManagement_RetrievalSubscribe service operation. Historical data or analytics may be retrieved from ADRF storage and data received in the future be sent when obtained by the ADRF.

Inputs, Required: Notification Correlation Information.

Inputs, Optional: Data or Analytics, Fetch Instructions, Unsuccessful bulked data generation, Expired bulked data deadline

Fetch Instructions indicate whether the data or analytics are to be fetched from the ADRF by the Consumer. If the data or analytics are to be fetched, the fetch instructions include an address from which the data may be fetched and one or more Fetch Correlation IDs. Data or Analytics are fetched using the Nadrf_DataManagement_RetrievalRequest service operation.

NOTE:	Data or Analytics provided in notifications from the DCCF are processed and formatted according to the Processing and Formatting Instructions provided by the Consumer in Ndccf_DataManagement_Subscribe.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * *