3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202046

Elbonia, April 6 – 12, 2022

# Title:	Reply LS on Mapped NSSAI

# Response to:	C1-222095 from CT1

# Release:	Rel-17

# Work Item:	5GProtoc17

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 3583

1. Overall Description:

SA2 would like to thank CT1 for their LS on Clarification on Mapped NSSAI.

Specific to the CT1 questions in C1-222095 / S2-2201936 SA2 would like to provide the following feedback:

Question 1: Could SA2 confirm that it is indeed optional for the VPLMN to provide the mapped S-NSSAI for VPLMN S-NSSAI 2?

Answer 1: There are two cases:

- If the HPLMN S-NSSAI value is different comparing to the Serving PLMN S-NSSAI value, the Serving PLMN needs provide the HPLMN S-NSSAI mapped to the Serving PLMN S-NSSAI to the UE.

- If the HPLMN S-NSSAI value is the same as the Serving PLMN S-NSSAI value, the Serving PLMN may provide the HPLMN S-NSSAI to the UE. if the VPLMN S-NSSAI is not provided by the VPLMN, the UE assumes the same S-NSSAI value is used in the VPLMN as in the HPLMN.

Question 2: If the answer to question 1 is yes, could SA2 further clarify the conditions under which it is optional to provide the mapped S-NSSAI.

Answer 2:  See above.

Question 3: If the answer to question 1 is yes, could SA2 further clarify if it is mandatory for the network to provide the mapped S-NSSAI to the UE if the mapped S-NSSAI is known to the network?

Answer 3:  The Serving network decides the mapping of the Serving PLMN S-NSSAI to HPLMN S-NSSAIs based on SLA. There is no such scenario the Serving network doesn’t know the mapping information for a S-NSSAI it provides to the UE in the Configured NSSAI. For the Allowed NSSAI, if the UE is not yet configured with any configured NSSAI and Requests no S-NSSAI, if a Default S-NSSAI value in the UE subscription has not any mapped value in the VPLMN (whether the same or different value as in the HPLMN)  then this is not included in the Allowed NSSAI for the UE (hence all the S-NSSAIs in the Allowed NSSAI have either a mapped value or they are assumed to have the same value as the S-NSSAI of HPLMN). A rejected S-NSSAI for the entire PLMN is expected to be provided without a mapped value if the reason is that there is no mapping available any longer, but if the reason of rejection is a not up to date mapping the HPLMN S-NSSAI can be allowed with the correct mapping. In both cases, and a new configured NSSAI is provided to the UE with the correct mapping for the S-NSSAIs the UE is expected to be able to request in the VPLMN.

Question 4: If the answer to question 3 is no, could SA2 further clarify how the UE determines the mapped S-NSSAI in that case?

Answer:  See above.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E   	May 16 – 20, 2022 				Elbonia