3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202046r04

Elbonia, April 6 – 12, 2022

# Title:	Reply LS on Mapped NSSAI

# Response to:	C1-222095 from CT1

# Release:	Rel-17

# Work Item:	5GProtoc17

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 3583

1. Overall Description:

SA2 would like to thank CT1 for their LS on Clarification on Mapped NSSAI.

Specific to the CT1 questions in C1-222095 / S2-2201936 SA2 would like to provide the following feedback:

Question 1: Could SA2 confirm that it is indeed optional for the VPLMN to provide the mapped S-NSSAI for VPLMN S-NSSAI 2?

Answer 1: Yes, it is optional (or rather conditional).

Question 2: If the answer to question 1 is yes, could SA2 further clarify the conditions under which it is optional to provide the mapped S-NSSAI.

Answer 2: There are two cases in roaming scenarios:

- If the HPLMN S-NSSAI value is different comparing to the Serving PLMN S-NSSAI value, the Serving PLMN needs provide the HPLMN S-NSSAI mapped to the Serving PLMN S-NSSAI to the UE.

- If the HPLMN S-NSSAI value is the same as the Serving PLMN S-NSSAI value, the Serving PLMN may provide the Serving PLMN S-NSSAI to the UE without mapping information i.e. the corresponding mapped HPLMN S-NSSAI value. If the S-NSSAI provided to the UE in the Configured NSSAI or in the Allowed NSSAI does not have a corresponding mapping information (i.e. mapped HPLMN S-NSSAI vallue), the UE assumes the same S-NSSAI value is used in the VPLMN as in the HPLMN.

Question 3: If the answer to question 1 is yes, could SA2 further clarify if it is mandatory for the network to provide the mapped S-NSSAI to the UE if the mapped S-NSSAI is known to the network?

Answer 3:  The Serving network decides the mapping of the Serving PLMN S-NSSAI to HPLMN S-NSSAIs based on Service Level Agreement (SLA). There is no such scenario the Serving network doesn’t know the mapping information for an S-NSSAI it provides to the UE in the Configured NSSAI or Allowed NSSAI. [Robert: I don't think this level of detail is needed for the reply LS.] Hence for all the S-NSSAIs in the Configured NSSAI and Allowed NSSAI either the Serving PLMN will provide a mapped value or the UE can assume that they have the same value as the S-NSSAI of HPLMN. A rejected S-NSSAI for the entire serving PLMN is expected to be provided without a mapped HPLMN S-NSSAI value if the reason of rejection is that there is no mapping info available. However, if the reason of rejection is a not due to unavailable mapping info, the serving PLMN can provide the rejected VPLMN S-NSSAI value with the corresponding mapped HPLMN S-NSSAI value. In both cases, a new configured NSSAI is provided to the UE with the correct mapping for the S-NSSAIs the UE is expected to be able to request in the VPLMN.

Question 4: If the answer to question 3 is no, could SA2 further clarify how the UE determines the mapped S-NSSAI in that case?

Answer 4:  See above.

If CT1 determines that the above described SA2 logic is not what is assumed by CT1 and there is a risk of UE implementations behaving differently, then it is acceptable for SA2 to always provide the HPLMN S-NSSAI values i.e. mandate the logic for 5GC NFs in the serving PLMN.

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E   	May 16 – 20, 2022 				Elbonia