3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202082r01

Elbonia, April 6 – 12, 2022

Source:	China Mobile

Title:	KI#1: Solution for EAS discovery in a VPLMN when roaming

Document for:	Approval

Agenda Item: 	9.11

Work Item / Release:	FS_EDGE_Ph2/Rel-18

Abstract of the contribution: This document proposes a solution to support UE EAS discovery in a VPLMN when UE using a Home Routed PDU Session.

- Discussion

This document proposes a solution to support UE EAS discovery in a VPLMN when UE using a Home Routed PDU Session.

- Proposal

It is proposed to add the following solution to TR 23.700-48.v.0.1.0.

# * * * * First change * * * *

6.0	Solution-Key issue matrix

The solutions in clause 6 can apply to one or more key issues described in clause 5 of this report. Table 6.0-1 describes the relationship between solutions and key issues.

Editor's note:	The table below will be updated with actual content when generating the TR with approved contributions. Page number is automatically updated to ease reference (ctrl-left click to reach the solution).

Table 6.0-1: Solution-Key issue matrix

| Solution |  | Key issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | (page) | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 | KI#7 |
| #:EAS discovery in Home Routed roaming scenario |  | X |  |  |  |  |  |  |

# * * * * Second change * * * *(all new texts)

## 6.X	EAS discovery in Home Routed roaming scenario

### 6.X.1	Description

This solution is for Key Issue #1, which addresses 5GS improvements to support the UE access to an EHE in a VPLMN using a PDU Session with a PSA in the HPLMN including aspects related to:

-	how to authorize the PDU session to support local traffic routing to access an EHE in the VPLMN;

-	how to support Rel-17 edge computing related procedures, such as EAS (re-)discovery, as specified in TS 23.548 [3], clause 6;

The current standards only depict 5GS architecture for non-roaming and LBO roaming scenario supporting with ULCL/BP. For Home Routed roaming scenario, the data flow of the UE needs to pass through the UPF of the VPLMN to the UPF of the HPLMN, the routing is controlled by SMF of the HPLMN, therefore the local traffic routing in the VPLMN cannot be achieved.

This solution proposes several ways to support UE access to VPLMN EHE in HR roaming scenarios according to different scenarios.

When HPLMN has the knowledge of EAS deployment information in VPLMN for specific services, H-SMF brings it to V-SMF during Home Routed PDU session establishment or modification, and V-SMF configure the V-EASDF according to the received EAS deployment information. H-SMF also sends the VPLMN ECS address and local traffic routing indication to V-SMF, and V-SMF performs EAS discovery and local traffic routing with V-EASDF.

If V-AF related information changes during this process, The V-AF can notify the H-AF of the information, and the H-AF configures it to the H-SMF according to the AF requests to influence traffic routing procedure defined in TS 23.502, clause 4.3.6.2. H-SMF will send the new policy information about the HR PDU session to V-SMF, and V-SMF will redo the EAS discovery and local traffic routing in VPLMN according to the updated configuration, e.g. select the UL CL and local DNS server according to the DNAI or update the configuration of V-EASDF.

NOTE: How the V-AF notifies the information to the H-AF is out of scope.

When HPLMN does not have the knowledge of EAS deployment information in VPLMN, after the H-SMF authorizes the V-SMF to perform local traffic routing, V-SMF retrieves the EAS deployment information of the VPLMN from the V-NEF and performs EAS discovery according to the EAS discovery procedure described in 23.548.

In addition, the V-AF can also configure the V-SMF according to the AF influence on traffic routing procedure. In this case, during the HR PDU session establishment, the V-SMF needs to select a PCF in the VPLMN to establish the SM Policy Association. V-PCF may send the VPLMN ECS address obtained from V-AF to V-SMF.

Editor NOTE: It is FFS whether and how charging at VPLMN (reporting by V-UPF to V-SMF to V-CHF) can apply.

### 6.X.2	Procedures

6.X.2.1	Procedure for EAS discovery when HPLMN has the knowledge of EAS deployment information in VPLMN

# Figure 6.X.2.1-1: HPLMN triggered EAS discovery in HR roaming scenario

1.	H-AF creates a new request with carrying V-AF application requirement and invokes a Nnef_TrafficInfluence_Create service operation and influence the traffic as described in TS 23.502 clause 4.3.6.2 step 1-4.

NOTE 1: how V-AF sends application related information to H-AF is in SA6 scope.

Editor Note: Whether the EAS deployment information is contained in AF request is FFS.

2.	5GS establishes a Home Routed PDU Session. During PDU Session establishment, H-SMF retrieve the new PCC rules impacted by the H-AF request for VPLMN from H-PCF, and sends Nsmf_PDUSession_Create response to V-SMF, which also includes an indication to authorize V-SMF to perform local traffic routing in VPLMN, VPLMN ECS address and new PCC rules. The indication and VPLMN ECS address are obtained by H-SMF from H-UDM.

NOTE 2: The new PCC rules are sent from H-SMF to V-SMF over N16.

3.	Step 1-6 in TS 23.548 clause 6.2.3.2.3 are performed. In step 1 of clause 6.2.3.2.3, UL CL will be inserted by V-SMF

NOTE 2: the local DNS server IP address will be configured to UE by V-SMF as described in TS 23.548 clause 6.2.3.2.3 Option C based on the information provided in the AF request in step 1.

6.X.2.2	Procedure for EAS discovery when HPLMN does not have the knowledge of EAS deployment information in VPLMN

# Figure 6.X.2.2-1: VPLMN triggered EAS discovery in HR roaming scenario

1.	EAS deployment Information provisioned from AF via NEF as described in TS 23.548 clause 6.2.3.4.2 .

2-3.	IT is assumed that V-SMF has subscribed to EAS Deployment Information Change Notification from the V-NEF. The V-NEF invokes Nnef_EASDeployment_Notify to the SMF to provide the EAS Deployment Information.

4.	If V-AF related information changes, V-AF creates a new request and invokes a Nnef_TrafficInfluence_Create service operation and influence the traffic as described in TS 23.502 clause 4.3.6.2 step 1-4

5.	UE establish Home Routed PDU Session. During the HR PDU Session establishment, V-SMF needs to establish a SM Policy Association with a PCF in VPLMN.

6.	UE sends DNS Query message to V-UPF.

7. V-UPF forwards the request to V-SMF.

8.	V-SMF decides to do the local traffic routing in VPLMN and sends the request of local traffic routing in VPLMN to H-SMF via Nsmf_PDUSession_Update request, and also sends the capability of supporting EASDF indication to H-SMF.

9.	H-SMF sends the indication to authorize V-SMF to perform local traffic routing in VPLMN via or Nsmf_PDUSession_Update response. The indication is obtained by H-SMF from H-UDM.

10-11.	V-SMF sends Npcf_SMPolicyControl_Update request to V-PCF to retrieve the new policy impacted by the AF request.

12.	After authorized by H-SMF to perform local traffic routing in VPLMN, V-SMF selects V-EASDF.

13. The V-SMF invokes Neasdf_DNSContext_Create Request to the selected V-EASDF.

14.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response.

15a. If V-SMF has received the EAS deployment information in step2-3, V-SMF updates the EASDF address to UE and indicates UE to resend the DNS Query message to EASDF.

16a. Step 7-19 in TS 23.548 clause 6.2.3.2.2 are performed.

15b. If V-SMF did not received the EAS deployment information in step2-3, and V-SMF received the application related information (e.g. DNAI, local DNS server address), V-SMF updates the local DNS server address to UE and indicates UE to resend the DNS Query message to local DNS server.

Editor NOTE: whether the V-SMF can handle the DNS Query (take the role of UE) is FFS

16b. Step 1-6 in TS 23.548 clause 6.2.3.2.3 are performed

# * * * *end of change * * * *