3GPP TSG-SA WG2 Meeting #150E (e-meeting)	S2-220XXXX

# April 6-12, 2022, Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Naf_EventExposure service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2022-03-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clauses 6.8.2 and 6.10.2 in TS 23.288 state that the data collected from AF related to User Data Congestion Analytics and Dispersion Analytics, but they are not listed yet in the Naf service in TS 23.502. The events of User Data Congestion information and Dispersion information which can be subscribed by a NF consumer for the Naf_EventExposure service are missing in Clause 5.2.19.2.1 of TS 23.502. Stage 3 already include it in TS 29.517. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions of events for User Data Congestion information and Dispersion information to Clause 5.2.19.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The events of User Data Congestion information and Dispersion information for the Naf_EventExposure service will missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.19.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

##### 5.2.19.2.1	General

Service description: This service enables consumer NF to subscribe and get notified of events.

The events can be subscribed by a NF consumer are described in TS 23.288 [50].

The following service operations are defined for the Naf_EventExposure service:

-	Naf_EventExposure_Subscribe.

-	Naf_EventExposure_Unsubscribe.

-	Naf_EventExposure_Notify.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	Service Experience information, as defined in clause 6.4.2, TS 23.288 [50].

-	Performance Data information, as defined in clause 6.4.2 and clause 6.14.2, TS 23.288 [50].

-	Collective Behaviour information, as defined in clause 6.5.2 of TS 23.288 [50].

-	UE Mobility information, as defined in clause 6.7.2.2, TS 23.288 [50].

-	UE Communication information, as defined in clause 6.7.3.2, TS 23.288 [50].

-	Exceptions information, as defined in clause 6.7.5.2, TS 23.288 [50].

-	User Data Congestion information, as defined in clause 6.8.2, TS 23.288 [50].

-	UE Data volume Dispersion information, as defined in clause 6.10.2, TS 23.288 [50].

-	DN Performance information, as defined in clause 6.14.2, TS 23.288 [50].

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AF exposure.

Table 5.2.19.2.1-1: Example of Event Filters for AF exposure events

| Event ID for AF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Exceptions information | <Parameter Type = Exception ID, Value = Exception ID1> |
| Service Experience information | <Parameter Type = TAI, Value = TAI1> |
| Service Experience information | <Parameter Type = geographical area, Value = civic address1 or shape1> |
| Collective Behaviour information | <Parameter Type = collective attribute, Value = collective value1> |
| Collective Behaviour information | <Parameter Type = data processing type, Value = data processing type1> |

* * * * End of Changes * * * *