3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202130

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for controlling 5G time synchronization service based on subscription

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS _5TRS _URLLC/ Rel-18

# Introduction

Control of time synchronization service based on UE subscription is important for the operator in managing time critical services such as smart grid or financial services.

This key issue aims at studying how to control 5G time synchronization service based on subscription (i.e. introducing subscription parameter for time synchronization and enforcing it).

# Text proposal

It is proposed to agree the following changes to 23.700-25:

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issue #1 | Key Issue #2 | Key Issue #3 | Key Issue #4 | Key Issue #5 | Key Issue #6 |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions |  |  |  |  |  |  |
| #1 | X |  |  |  |  |  |
| #2 |  |  |  |  |  | X |
| #X |  |  | X |  |  |  |

# * * * * First change * * * *

## 6.X	Solution X: Support for controlling 5G time synchronization service based on subscription.

### 6.X.1	Introduction

The solution enables the operator to control time synchronization service based on UE subscription for time critical services management.

### 6.X.2	General description

The following assumptions are made:

- For specific UEs which the holdover capability is not supported or supported badly, the operator should provide them reference time information continuously.

Editor's note: The previous assumption is not clear since the time reference (indicated through SIB frame boundaries) is always provided to the UE but the actual time does not need to always be provieded.

- UE subscription should be taken into account when TSCTSF chooses a time information provider in 5GS.

- UEs get time synchronization service by receiving 5GS access stratum time or time-synchronized UPF/NW-TT.

Following are the principles for the solution:

- In order to provide continuous time synchronization service for targeted UEs, 5GC should be informed if the time synchronization status of those UEs change. The time status changes can be resulted from some unexpected cases, e.g., time sources failure or UE mobility.

- The AF could send a request to TSCTSF (directly or via NEF) to control the (g)PTP time synchronization service and may target to a set of AF-sessions with a UE or multiple UE(s). In addition, the AF may request time synchronization distribution method, such as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock or as s PTP relay instance. For all the AF-request, TSCTSF should check targeted UE(s) subscription data from the UDM first before it invokes time synchronization service. If necessary, the SMF would check the per UE subscription data again when PDU session establishment.

- The AF may request to use the 5G access stratum timing information for UE(s) or the attached DS-TT(s). When TSCTSF receives the request, it should get the targeted UE subscription data from the UDM to authorize the request. If needed, the AMF is responsible to check the per UE subscription data when it controls 5G-AN to provide 5G access stratum timing information.

- A trusted AF can request a stringent/resilient time synchronization service for targeted UEs or DS-TTs if the UEs subscription supports.

- UE could request a time synchronization service with gNB or UPF itself for specific time service requirements, e.g., UE can request a stringent time synchronization error budget, which may be different from those directly from gNB via SIB or UPF from gPTP.

Editor's note: Need for UE to be aware of the need for time synchronization and need for support to support a related UE request is FFS.

Editor’s Note: the solution assumes that operator’s deployment ensures that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30]. It is FFS if the serving NG-RAN nodes do not support Rel-17 propagation delay compensation.

### 6.X.3	Procedures

Below is the flow chart for UE requesting time synchronization service based on subscription.

AMF

PCF

TSCTSF

BSF

4.Ntsctsf_TimeSynchronization_ASTICreate/Update/Delete request

RAN

2.NAS_TimeSynchronization_Create/Update/Delete request

UDM

3.UE_time subscription data request/response

5.UE_time subscription data request/response

6.Nbsf_Managemet_Subscribe/Notify

7.Npcf_AMPolicyAuthorization_Create/Update request

9.Npcf_AMPolicyAuthorization_Create/Update response

11.TimeSynchronization_ASTICreate/Update/Delete Result

10.Ntsctsf_TimeSynchronization_Create/Update/Delete Response

1. AM Policy Association Establishment

8. AM Policy Association Modification initiated by the PCF

UE

Figure 6.X.3-1 UE requesting time synchronization service based on subscription

- AM policy association establishment procedure is executed when UE registered to the 5G network.

- When triggered by application layer, UE sends UL NAS transport message to AMF to request time synchronization service. The parameters included in the request are associated with UE specific requirements, e.g., error budget.

- The AMF authorizes the request based on UE time synchronisation service related subscription data, i.e. UE has valid subscription data for time synchronization service.

- The AMF invokes the service operation provided by the TSCTSF. AMF discovers and selects the TSCTSC based on local configuration or via NRF query. The service operation request includes the time synchronization service request from the UE.

- When receiving the UE time synchronization service request, TSCTSF may retrieve the UE time synchronization service related subscription data from UDM to check whether the UE is authorized to request the time synchronization service for the requested error budget.

- If the 5G access stratum time distribution parameters in UDR are associated with a DNN/S-NSSAI for the PCF for the UE may discover the PCF for the PDU Session using SUPI and (DNN, S-NSSAI) as parameters.

- TSCTSF determines the PCF that handles the AM Policy Association of the targeted UE with SUPI (received in step 4) as an input parameter.

- If the TSCTSF sends multiple time synchronization error budgets for a given UE, the PCF would pick the most stringent budget. The PCF takes a policy decision and then it may initiate an AM Policy Association Modification procedure. As part of this, the 5G access stratum time distribution indication and the Uu time synchronization error budget are provided to NG-RAN. Based on this, NG-RAN provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

- The PCF of the UE replies to the TSCTSF with the result of Npcf_AMPolicyAuthenorization operation.

- The TSCTSF responds to the AMF request in step 4.

- The AMF informs the UE about the result of the operation in step 2.

### 6.X.4	Impacts on services, entities and interfaces

- TSCTSF

- Update the 5G time distribution indication based on UE subscription.

- Authorize time synchronization service request based on UE subscription.

- UE

- Support to request time synchronization service per UE

- AMF

- Authorize time synchronization service request based on UE subscription.

- Support to forward UE time synchronization service to targeted TSCTSF.

# * * * * End of changes * * * *