3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202137

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New solution on support of Credentials Holder scenarios over untrusted non-3GPP access in SNPN

Document for:	Approval

Agenda Item:	9.4

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract: This contribution would like to propose a new solution on support of Credentials Holder scenarios over untrusted non-3GPP access in SNPN.

1. Introduction/Discussion

Clause 5.30.2.8 and Annex D, clause D.3 of TS 23.501 [3] specify how the UE can access SNPN services via a PLMN. Solution #2 defines how the UE can access SNPN services via Untrusted non-3GPP access network using the credentials owned by the SNPN.

This solution defines how the UE can access SNPN via Untrusted non-3GPP access network by using credentials owned by Credentials Holder separate from the SNPN.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-08.

# * * * * First change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| X |  | X |  |  |  |  |

# * * * * Second change * * * *

6.X	Solution #X: Support of Credentials Holder scenarios over untrusted non-3GPP access in SNPN

6.X.1	Introduction

Editor's Note:	This clause lists the key issue(s) addressed by this solution, and briefly the main principles of the solution.

This solution aims at addressing key Issue #2 about support of Non-3GPP access for SNPN. In particular, this solution mainly focuses on how to support functionalities defined in R17 eNPN such as accessing SNPN using credentials owned by Credentials Holder separate from the SNPN over untrusted non-3GPP access. For UE accessing SNPN using credentials owned by the SNPN, Solution #2 specified in clause 6.2 can be applied.

6.X.2	Functional Description

Editor's Note:	This clause further details the solution principles and any assumptions made.

When UE accesses to a SNPN using credentials owned by Credentials Holder separate from the SNPN, it shall select a N3IWF in the SNPN that supports connecting to the Credentials Holder belonging to a group identified by GINs. Therefore, clause 6.3.6.2a of TS 23.501[3] can be applied with following clarifications and additions:

- The UE is configured with a GIN that identifies a group the CH belongs to.

- The FQDN constructed by the UE consists of GIN that identifies a group the CH belongs to and the Visited Country FQDN, indicating the query is for SNPN and performing a DNS query for the resulting FQDN.

The example of N3IWF FQDN consisting of GIN is shown below:

n3iwf.5gc.GIN999123456789ABCDE.snpnid999123456789ABCDE.mnc012.mcc345.pub.3gppnetwork.org

6.X.3	Procedures

Editor's Note: This clause describes procedures and information flows for the solution.

6.X.4	Impacts on services, entities, and interfaces

Editor's Note: This clause lists impacts to services, entities, and interfaces.

UE impact:

-	Ability to construct an FQDN consist of GINs used for selecting a preferred SNPN that supports connecting with Credentials Holder.

# * * * * End of changes * * * *