SA WG2 Meeting #S2-150e(e-meeting)	S2-2202215r03

06 - 12 April 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson, Huawei, HiSilicon

Title:	Solution for configuration

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: Introduce one solution for FS_VMR KI#1 and KI#4.

# 1. Introduction

This paper proposes one solution for one KI#1 and KI#4

# 2. Proposal

This paper proposes the following updates to TR 23.700-05 clause 6.

* Start of change *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”.

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[x]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[y]	3GPP TS 38.401: "NG-RAN; Architecture description".

* Second change *

## 6.x	Solution #X: Subscription data and O&M based mobile IAB-node configuration

### 6.x.1	Introduction

The solution provides methods for mobile IAB-node configuration and 5GC access control based on O&M and subscription. This includes roaming scenario.

### 6.x.2	Functional Description

The solution is based on the support of existing IAB architecture as specified in TS 23.501 [2] with the following high-level description:

- For the IAB-node connects to 5GS, the access control and mobility handling is in 5GC based on subscription data in the network for the IAB-UE (i.e., IAB-MT). In case of roaming, the local policy in serving AMF may further enforce it based on mobility restriction functionality.

- For the IAB-node configuration, it is supported by the O&M using either backhaul IP layer by implementation or using IP provided by the IAB-UE’s (or IAB-MT’s) PDU session via 5G network as described in TS 38.401 [y]. In case of roaming, it’s the serving network O&M provides the configuration information to the IAB-node.

NOTE: Coordination with RAN WGs is needed to decide if there is extra configuration support needed for IAB-node due to mobility.

Editor’s NOTE: It’s FFS if extra O&M procedures impact in SA5 is needed.

### 6.x.3	Procedures

The registration request procedure in clause 4.2 of TS 23.502 [x] in Rel-17 can be used by the IAB-UE to access the 5GS.

Optionally the PDU session establishment procedure in clause 4.3 of TS 23.502 [x] may be used to provide configuration information. In case of roaming, the serving network provides the configuration information to the IAB-node.

### 6.x.4	Impacts on services, entities and interfaces

AMF:

-	none.

gNB:

-	none.

* End of changes *