SA WG2 Meeting #S2-150e(e-meeting)	S2-2202217r01

06 - 12 April 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	Solution for user location

Document for:	Approval

Agenda Item:	9.9

Work Item / Release:	FS_VMR

Abstract of the contribution: Introduce one solution for FS_VMR KI#6.

1. Introduction

This paper proposes one solution for one KI#6

2. Proposal

This paper proposes the following updates to TR 23.700-05 clause 6.

* Start of change *

6.x	Solution #X: User location information based on IAB-UE

6.x.1	Introduction

The solution provides a way to represent the User Location Information, ULI, of UEs connected to the mobile IAB-node based on IAB-UE (i.e., IAB-MT).

6.x.2	Functional Description

When mobile IAB-mode moves to a new IAB-donor gNB, the cell ID/TAC may change as shown in figure 6.x.2-1 below.

During the mobility of the IAB-UE part, the new ULI for the IAB-UE is based a cell/TAC managed by the new IAB-donor gNB and it is provided to CN. The ULI of the IAB-UE is always corresponding to a geographic area.

When UEs connected to the IAB-node are reconfigured to the new IAB-donor gNB, with the same or different cell/TAC which is covered by the IAB-node and managed by the new IAB-donor gNB, the ULI of the IAB-UE can be used to represent the ULI of the connected UEs, in addition to the ULI of the connected UE, e.g., the cell y is considered as coverage extension of the cell x in figure 6.X.2-1.

NOTE: the cell ID/TAC handling during IAB-node mobility needs to the coordinated with the work in RAN WGs.

Figure 6.X.2-1: Cell/TAC for IAB-UE and UE connected to IAB-node

6.x.3	Procedures

6.x.3.1	User Location Information on NGAP

There is no new procedure for the additional ULI.

The current ULI information on NGAP and user location information on CN related interfaces are extended to support the additional ULI related to the IAB-MT for UEs connected to IAB-node.

Editor's Note: It is FFS how the AMF distinguishes between the ULI of the connected UE and the ULI of the IAB-MT.

Editor's Note: It is FFS how the AMF utilizes the additional ULI, such as Mobility Management, considering the user location information includes ULI of the connected UE and ULI of the IAB-MT.

6.x.4	Impacts on services, entities and interfaces

AMF:

-	Support additional ULI besides current ULI and provide the additional ULI to other NFs.

gNB:

-	Support the formulation of additional ULI for UE connected to IAB-node besides the current ULI.

* End of changes *