SA WG2 Meeting #S2-150e(e-meeting)	S2-2202219

06 - 12 April 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	Solution for KI#4: Introduction of PDU Set information and QoS parameters

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XMR / Rel-18

Abstract of the contribution: solution for PDU Set integrated packet handling by introducing PDU Set information and QoS parameters

# 1. Introduction

This paper proposes solution for KI#4 that addresses following aspects:

-	Which types of PDU Set (e.g. frame/video slice) shall be supported for PDU Set integrated packet handling by 5G network;

NOTE 1: Coordination with SA WG4 is possibly needed.

-	What information should be provided to the 5GS regarding PDU Set for integrated packet handling, and how such information should be provided;

-	How the 5GS identifies that a PDU belongs to a specific PDU Set;

NOTE 2: It is assumed that some header information necessary for the identification of PDUs and it is assumed that the corresponding information is not encrypted.

-	Whether and how to enhance the QoS model and policy control for PDU Set integrated packet handling;

-	Whether and how to perform the PDU Set integrated packet handling in the UE, RAN and/or UPF;

-	What information needs to be provided to the RAN and/or UPF to support PDU Set integrated packet handling in both the downlink and uplink direction.

-	Whether and how provide information to the UE for PDU Set integrated packet handling.

NOTE 3: Coordination with RAN WGs may be needed.

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 6.

* Start of change *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[7]              RFC 3550 RTP: A Transport Protocol for Real-Time Applications

[8]              RFC 8285 A General Mechanism for RTP Header Extensions

* Next change *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

PSDB	PDU-Set Delay Budget

PSER	PDU-Set Error Rate

* Next change *

## 6.X	Solution #X: Solution to KI#4: Introduction of PDU Set information and QoS parameters

### 6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

### 6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

In this solution the application layer has the capabilities to provide new information describing characteristics of a ‘PDU-Set’. The characteristics are encoded in a generic, media-type-agnostic manner. This to relieve the 5GS from the burden of having media knowledge of applications that are not under 3GPP control.

### 6.X.2.1 Solution to identify a PDU-Set on N6/in UPF

To make use of the PDU-Set concept in 5GS, it is needed to provide means for the 5GS to identify the packets constituting a PDU-Set at N6 interface/UPF. Given that the number of PDUs constituting a PDU Set changes dynamically, this solution uses an in-band signaling for that purpose.

For the case when no e2e encryption is used, the solution is outlined below:

- It is assumed that the application layer provides the relevant information about the Information Unit to the lower layers that will enable lower layers to insert information aiding at identification of the PDU-Set at N6.

  - To ensure that UPF only attempts to detect PDU-Sets in packet flows where such are known to be present, the ‘PDU-Set’ presence indication can be provided to UPF from the SMF. The SMF receives it as new information in the PCC rule. PCF can receive it, if available, from AF using existing procedures enhanced with that new information.

- Based on the understanding that segmentation into PDU-Set is applicable for real-time media, it is assumed that RTP is used. Consequently, in this solution RTP extension header (see [7] RFC 3550 section 3.5.1 and [8] RFC 8285) is used to convey PDU-Set specific information to 5GS.

- The following information is used to enable filtering of the PDU-Sets in the UPF :

  - PDU-Set Sequence number – new information added to the RTP extension header

- UPF is configured with PDR that enables inspection RTP extension header. To reduce the UPF from packet inspection complexity, it is required to standardize an RTP configuration that enables a fixed starting position of the PDU-Set RTP extension header. RTP header extension inspection is a step following the PDR matching process and therefore does not impact the PDR matching process.

### 6.X.2.2 New information associated with PDU-Set

The concept of a PDU-Set enables enhancements to efficient resource management in 5GS, e.g. in NG-RAN.

One such example enables cell capacity increase. In this example NG-RAN may take a decision to not deliver any PDU of a given PDU-Set when NG-RAN can assess that not all PDUs constituting that PDU set are feasible to be delivered within a required time while it is known that only if PDUs are delivered the PDU-Set is used by the receiving application layer entity. To enable such enhancement following PDU-Set associated information is provided in the RTP extension header.

- Size of the PDU-Set (number of bytes) enabling NG-RAN to assess whether delivery of the PDU-Set is feasible. If re-ordering of PDUs prior to N6 is assumed as possible, the ‘size of the PDU-Set information’ needs to be included in the RTP extension header of each packet.

- Indication whether a PDU-Set is only used by receiving application layer entity if all PDUs constituting that PDU-Set are successfully received within the PDU-Set Delay Budget (for definition of PDU-Set Delay Budget please see 6.X.2.3).

Rationale for (1) & (2): NG-RAN may take a decision to not deliver any PDU of a given PDU-Set when NG-RAN can assess that not all PDUs constituting that PDU set are feasible to be delivered within a required time while it is known that only if PDUs are delivered the PDU-Set is used by the receiving application layer entity

- PDU-Set inter-dependencies, e.g. Late PDU-Set delivery indication.

Rationale for (3): Dependent on the type of content carried by a PDU-Set within a QoS Flow, a late PDU-Set delivery, i.e. delivery non-compliant with the required latency may be still beneficial from the application perspective. Hence it enables NG-RAN to improve the resource usage while considering benefits for the QoE.

Given that the new PDU-Set associated information described in bullets 1..3 above is PDU-Set specific and can dynamically change between consecutive PDU-Sets, it is not feasible to provide it as a (semi-)static information type in NGAP, e.g. in a QoS profile of the QoS Flow on which the packet flows consisting of PDU-Sets is mapped on. Hence in this solution this new PDU-Set associated information is provided to NG-RAN in-band, i.e. encoded in the GTP-U extension header by the UPF.

To enhance NG-RAN with information required to configure CDRX for UE power savings, see S2-2202220, following additional new assistance information is proposed to be added:

- PDU-Set periodicity

- PDU -Set jitter range

This assistance information is assumed to be static/semi-static and thus can be provided by the AF to 5GS and in NGAP to NG-RAN.

### 6.X.2.3 New QoS parameters associated with PDU-Set

An application layer instance can produce units of information that can be used by another application layer instance, e.g. to construct a usable information and one example of such information unit can be a video frame. Dependent on its size and the MTU of the transport network, that information unit may need to be segmented and transferred in multiple transport units, e.g. multiple IP packets. When all segments are received, the receiving application layer instance uses the information unit. Hence the QoE is dependent on the reception of the information unit rather than individual segments constituting it. Therefore, the forwarding treatment described by the QoS parameters needs to be associated with the information unit.

Following the discussion above, SA2#149E agreed (subject to confirmation by SA4) a concept corresponding to information unit:

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or video slice for XRM Services), which are of same importance at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

Based on the discussion above, we first introduce new ‘PDU-Set’ level QoS parameters (edited in text below in bold format).

PDU-Set Delay Budget (PSDB)

The PDU-Set Delay Budget (PSDB) defines an upper bound for the time that a PDU-Set may be delayed between the UE and the N6 termination point at the UPF. PSDB applies to the DL PDU-Set received by the UPF over the N6 interface, and to the UL PDU-Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL. In the case of 3GPP access, the PSDB is used to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). For GBR QoS Flows using the Delay-critical resource type, a PDU-Set delayed more than PSDB is counted as lost if the QoS Flow is not exceeding the GFBR. For GBR QoS Flows with GBR resource type not exceeding GFBR, [X] percent of the PDU-Sets shall not experience a delay exceeding the 5QI's PSDB.

Editor’s note: The percentage of the PDU-Sets that shall not experience a delay exceeding the 5QI's PSDB for GBR QoS Flows with GBR resource type is FFS.

The 5G Access Network PDU-Set Delay Budget (5G-AN PSDB) is determined by subtracting a static value for the Core Network PDU-Set Delay Budget (CN PSDB), which represents the delay between any N6 termination point at the UPF (for any UPF that may possibly be selected for the PDU Session) and the 5G-AN from a given PSDB.

NOTE 1:	For a standardized 5QI, a static value for the CN PSDB can be specified .

NOTE 2:	For a non-standardized 5QI, the static value for the CN PSDB is homogeneously configured in the network.

For GBR QoS Flows using the Delay-critical resource type, in order to obtain a more accurate delay budget PSDB available for the NG-RAN, a dynamic value for the CN PSDB, which represents the delay between the UPF terminating N6 for the QoS Flow and the 5G-AN, can be used. If used for a QoS Flow, the NG-RAN shall apply the dynamic value for the CN PSDB instead of the static value for the CN PSDB (which is only related to the 5QI). Different dynamic value for CN PSDB may be configured per uplink and downlink direction.

NOTE 3:	The configuration of transport network on CN tunnel can be different per UL and DL, which can be different value for CN PSDB per UL and DL.

NOTE 4:	It is expected that the UPF deployment ensures that the dynamic value for the CN PSDB is not larger than the static value for the CN PSDB. This avoids that the functionality that is based on the 5G-AN PSDB (e.g. Maximum PDU-Set size, NG-RAN scheduler) has to handle an unexpected value.

The dynamic value for the CN PSDB of a Delay-critical GBR 5QI may be configured in the network in two ways:

-	Configured in each NG-RAN node, based on a variety of inputs such as different IP address(es) or TEID range of UPF terminating the N3 tunnel and based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF, etc;

-	Configured in the SMF, based on different combinations of PSA UPF to NG-RAN under consideration of any potential I-UPF. The dynamic value for the CN PSDB for a particular QoS Flow shall be signalled to NG-RAN (during PDU Session Establishment, PDU Session Modification, Xn/N2 handover and the Service Request procedures) when the QoS Flow is established or the dynamic value for the CN PSDB of a QoS Flow changes, e.g. when an I-UPF is inserted by the SMF.

If the NG-RAN node is configured locally with a dynamic value for the CN PSDB for a Delay-critical GBR 5QI, and receives a different value via N2 signalling for a QoS Flow with the same 5QI, local configuration in RAN node determines which value takes precedence.

Services using a GBR QoS Flow and sending at a rate smaller than or equal to the GFBR can in general assume that congestion related packet drops will not occur.

NOTE 5:	Exceptions (e.g. transient link outages) can always occur in a radio access system which may then lead to congestion related packet drops. Packets surviving congestion related packet dropping may still be subject to non-congestion related packet losses (see PSER below).

Services using Non-GBR QoS Flows should be prepared to experience congestion-related PDU-Set drops and delays. In uncongested scenarios, [X] percent of the PDU-Sets should not experience a delay exceeding the 5QI's PSDB.

Editor’s note: The percentage of the PDU-Sets that shall not experience a delay exceeding the 5QI's PSDB for non-GBR QoS Flows is FFS.

The PSDB for Non-GBR and GBR resource types denotes a "soft upper bound" in the sense that an "expired" PDU-Set that has exceeded the PSDB, does not need to be discarded and is not added to the PSER. However, for a Delay-critical GBR resource type, PDU-Set delayed more than the PSDB are added to the PSER and can be discarded or delivered depending on local decision.

PDU-Set Error Rate (PSER)

The PDU-Set Error Rate (PSER) defines an upper bound for the rate of PDU-Sets (e.g. set of IP packets constituting a PDU-Set) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but where all of the PDUs in the PDU-Set are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PSER is the same in UL and DL. For GBR QoS Flows with Delay-critical GBR resource type, a PDU-Set which is delayed more than PSDB is counted as lost, and included in the PSER unless the QoS Flow is exceeding the GFBR.

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

* End of changes *