SA WG2 Meeting #S2-150e(e-meeting)	S2-2202220r02

06 - 12 April 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Ericsson

Title:	Solution for KI#8 Enhancements to power savings for XR

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract of the contribution: This contribution proposes addition of traffic information that is useful for the RAN to enhance power management with CDRX.

# 1. Introduction

This paper proposes solution for KI#8: Enhancements to power savings for XR services. In this KI the following areas of study were agreed [S2-2201812] by SA2#149E:

“For this Key Issue the following areas should be studied:

- Which information (e.g., XR/media traffic characteristics, traffic pattern and statistics), if any, is needed by the RAN from the CN and/or the UE to enhance power management (i.e., CDRX)? If needed, how such information is collected?

NOTE:	This Key issue requires collaboration with RAN WGs.”

In this context this paper proposes traffic information that can be used by the RAN to enhance CDRX and also how to make this information available to the RAN.

# 2. Proposal

This paper proposes the following updates to TR 23.700-60 clause 6.

* Start of change *

## 6.X	Solution #X: Support for PDU Set information for CDRX enhancements

### 6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

This solution addresses KI#8: Enhancements to power savings for XR services.

### 6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

This solution assumes that there are means to support PDU Set identification at the UPF and at the RAN.

The following information is useful for the NG-RAN to enhance and configure CDRX: Burst periodicity, were a burst is a combination of one or more PDU Sets.

#### Burst periodicity

The Burst periodicity can be used to enhance CDRX at the NG-RAN, such that the CDRX cycle length is matched to the inter-arrival time of the PDU Sets. A PDU Set can be, for instance, a video frame with periodicity 1/(60 fps). Thus, more power could be saved at the UE while ensuring a short traffic delay, since the UE awake time could follow closely the traffic arrival. We note that the periodicity is a parameter already supported in one of the NGAP information elements (IEs) in TSCAI, associated with a TSC QoS flow [TS 38.413, TS 23.501]. However, this periodicity is defined per packet burst rather than PDU Set burst. We propose to introduce a new attribute for the periodicity per PDU Set basis. This attribute contains a list of periodicity values, corresponding to different PDU Sets (e.g. for video and audio flows), in case multiple flows are multiplexed in the same QoS flow. This attribute is included in the NGAP PDU Session Management procedures and associated with the applicable QoS Flow.

Editor's Note: The use of multiple periodicity values in the same QoS Flow can result in a complex periodic pattern. It is FFS how such a complex periodic pattern can be used by RAN for configuration of the CDRX.

### 6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's Note: This clause captures impacts on existing 3GPP nodes and functional elements.

* End of changes *