3GPP TSG SA2 Meeting #150E	S2-2202235r13

# E-Meeting, 6th – 12th April 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3597 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of last visited TAI requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH |  |  |  |  |  | Date: |  |  | 2022-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) Clause 5.4.11.6 is referring to a "UE accessing a radio cell", but generally "accessing a cell" is used for situations where the UE establishes an RRC connection (cf. "unified access control"). It would be better to use "entering a cell" or "visiting a cell" which also applies to the situation where the UE remains in idle mode. 2) The above CR also introduced the wording "for the RPLMN or equivalent to the RPLMN", but generally TS 23.501 is referring to "RPLMN or an equivalent PLMN". 3) In the last sentence of this clause, When indicating a last visited TAI in a Registration Update, a UE may indicate any TAI supported in a radio cell for the RPLMN or equivalent to the RPLMN for the last UE access prior to the Registration Update that is part of the UE Registration Area. it is not clear to which part of the sentence the word "that" is referring to. It is proposed to reword the sentence, taking above items 1 and 2 into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1+2) Terminology is corrected. 3) Sentence is reworded. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Based on the unclear wording, the UE may report a wrong last visited TAI to the AMF (e.g., the last TAI where the UE was in connected mode instead of a TAI where the UE was in idle mode), possibly resulting in a non-optimum new TAI list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.11.6	Support of Mobility Registration Update

A moving radio cell for NR satellite access may indicate support for one or more TACs for each PLMN. A UE that is registered with a PLMN may camp on a radio cell and does not need to perform a Mobility Registration Update procedure as long as at least one supported TAC for the RPLMN or an equivalent PLMN is part of the UE Registration Area. A UE shall perform a Mobility Registration Update procedure when camping on a radio cell where none of the supported TACs for the RPLMN or equivalent to the RPLMN are part of the UE Registration Area.

For NR satellite access the concept of "last visited TAI" is extended such that:

a)	if the cell on which the UE is camped is broadcasting one or more TAIs of the UE's Registration Area, then the UE may use any one of those TAIs to determine the last visited TAI.

b)	If the set of broadcast TAIs is changed while the UE is camping on the cell, then this can affect the set of TAIs of the UE's Registration Area from which the UE determines the last visited TAI (according to item a).