3GPP TSG SA2 Meeting #150E	S2-2202237r02

# E-Meeting, 6th – 12th April 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3697 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IoT NTN: Correction of last visited TAI requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_SAT_ARCH_EPS |  |  |  |  |  | Date: |  |  | 2022-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1) With the current definition of the last visited TAI, if the last visited cell is satellite cell broadcasting TAIs for the RPLMN or an equivalent PLMN, the UE could select any of these TAIs, even if none of these TAIs belong to the UE's Tracking Area List. So an essential condition is missing. 2) CR 23.401-3683r1 (S2-2201332) replaced "accessing a cell" with "entering a cell" in one place in clause 4.13.6. There is another place in the same paragraph where the same correction applies. 3) The above CR also introduced the wording "RPLMN or equivalent to the RPLMN" and "RPLMN or equivalent RPLMN", but generally TS 23.401 is referring to "RPLMN or an equivalent PLMN". |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Missing condition is added. 2+3) wrong terminology is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Based on the wrong condition, the UE may report a wrong last visited TAI to the MME, possibly resulting in a non-optimum new TAI list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********** FIRST CHANGE *************

### 4.13.6	Support of Tracking Area Update

A moving cell for NB-IoT, LTE-M or WB-E-UTRAN satellite access may indicate support for one or more Tracking Areas Codes (TACs) for each PLMN (see clause 4.13.7). A UE that is registered with a PLMN may camp on a cell and does not need to perform a Tracking Area Update procedure for mobility reasons as long as at least one supported TAC for the RPLMN or an equivalent PLMN indicated in the cell is part of the UE's Tracking Area List. A UE shall perform a Tracking Area Update procedure when camping on a cell where none of the supported TACs for the RPLMN or equivalent PLMN(s) indicated in the cell are part of the UE's Tracking Area List.

When indicating a last visited TAI in an Attach Request or a TAU Request, if the last visited cell of the UE's Tracking Area List is a cell for satellite access broadcasting

-	two or more TAIs of the UE's Tracking Area List, the UE may indicate any of these TAIs as last visited TAI; or

-	one TAI of the UE's Tracking Area List only, the UE shall indicate this TAI as last visited TAI.

NOTE:	The last visited cell of the UE's Tracking Area List can be the current cell, e.g., if the Tracking Area Update procedure is not initiated for mobility reasons.

-