SA WG2 Meeting #150E (e-meeting)	S2- S2-2202274r02

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI #5, New Sol: Access to localized service by using roaming architecture

Document for:	Approval

Agenda Item:	9.4 Study on enhanced support of Non-Public Networks phase 2

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a new solution to Key Issue #5.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-08 on FS_eNPN_Ph2.

* * * * Start of 1st Change * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 |  | X |  |  |  |  |
| 3 |  | X |  |  |  |  |
| X |  |  |  |  | X |  |

* * * * Start of 2nd Change (All new text) * * * *

6.X	Solution #X: Access to localized service by using roaming architecture

6.X.1	Introduction

This solution addresses Key Issue #5: Enabling access to localized services via a specific hosting network. The basic principle of this solution is reusing existing roaming architecture and LADN mechanism. This solution does not resolves how the UE triggers, discovers and selects hosting network, which is in the scope of Key Issue #4.

6.X.2	Functional Description

In this solution, it is assumed that a UE's home network is a PLMN and hosting network is a PNI-NPN.

The following figure 6.X.2-1 and figure 6.X.2-2 depict the proposed architecture, which are reusing 5G System roaming architecture in case of local breakout and home routed scenario. When a UE registers to the selected hosting network and receives localized services provided by the hosting network, architecture in figure 6.X.2-1 is used. If the UE also need to get services from the home network, the UE may establish home routed PDU session as shown in figure 6.X.2.2. Whether the UE receives a specific service from the hosting network or home network is determined based on subscription of the UE by establishing a local breakout session, or a home routed session. The UE just follows the existing PDU Session Establishment procedure based on URSP rule provided by the home network.

Figure 6.X.2-1. Architecture when a UE uses localized services provided by the hosting network

Figure 6.X.2-2. Architecture when a UE uses services provided by the home network

When a UE registers to the hosting network and has the subscription to a specific LADN DNN for local service or a wild card DNN, the AMF provides LADN information (i.e. LADN DNN and LADN service area) to the UE according to the existing LADN mechanism. The UE uses the LADN information to access localized services provided by the hosting network.

NOTE:	When hosting network operator configures LADN information in the AMF, the operator can provide associated validity time of LADN DNN so that the AMF provides LADN information when the validity time is satisfied.

Editor's note:	It is FFS whether and how to support the scenario that PNI-NPN is in the HPLMN.

Editor's note:	It is FFS how the UE discovers and selects PNI-NPN (including whether and how to support CAG) for local services.

6.X.3	Procedures

Figure 6.X.3-1. Registration in the hosting network

The Registration procedure in TS 23.502 [4] is used with following modifications:

-	When a UE performs initial Registration procedure, Registration type indicates that the UE is accessing the network for localized services. Based on this information the AMF determine to provide LADN information to the UE.

-	The AMF indicates to the UDM that the UE is registering for localized services. The UDM use the indication whether to accept UE registration.

6.X.4	Impacts on services, entities, and interfaces

UE:

-	The UE indicates that the UE is accessing the network for localized services during the Registration procedure.

AMF:

-	The AMF indicates to the UDM that the UE is registering for localized services.

UDM:

-	The UDM determines whether to accept registration taking into account the indication that UE is registering for localized services.