SA WG2 Meeting #150E (e-meeting)	S2-2202275r01

February 14 – 25, 2022, Elbonia	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	KI #5, New Sol: Delaying UDM Registration until non-3GPP access switching completes

Document for:	Approval

Agenda Item:	9.3 Study on Access Traffic Steering, Switching and Splitting support in the 5GS; Phase 3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes a new solution to Key Issue #5.

Discussion

Proposal

It is proposed to agree the following changes into TR 23.700-53 on FS_ATSSS_Ph3.

* * * * Start of 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution # | Solution Title | Key Issue(s) |
| --- | --- | --- |
| #1 | New steering mode - Redundancy steering mode with packet loss rate | #3 |
| #2 | Support non-3GPP access leg of MA-PDU Session with PDN connection in EPC | #6 |
| #3 | MP-DCCP based Steering Functionality | #2 (DCCP) |
| #X | Delaying UDM Registration until non-3GPP access switching completes | #5 |

* * * * Start of 1st Change (All new text) * * * *

6.X	Solution #X: Delaying UDM Registration until non-3GPP access switching completes

6.X.1	Introduction

This solution addresses Key Issue #5 "Switching traffic of an MA PDU Session between two non-3GPP access paths" by allowing two simultaneous registration over two non-3GPP accesses. In this solution, in order to minimize overall system impact, the AMF delays UDM Registration until access switching is completed.

6.X.2	High-level Description

When a UE establishes MA PDU Session, the SMF may indicate to the UE whether non-3GPP access switching is supported. Based on this indication, the UE may determine to change non-3GPP access leg. Whether and when to switch non-3GPP access is determined by the UE. If the UE determines to switch access, the UE performs registration over the new non-3GPP access with new registration type to indicate the registration is for switching non-3GPP access. The AMF follows normal registration procedure as described in TS 23.502 [3] but does not perform UDM registration. After the Registration procedure is completed, the UE sends PDU Session Establishment message to add new non-3GPP access leg to the existing MA PDU Session. After the access leg over the new non-3GPP access is established, the UE and UPF starts sending traffic over the new non-3GPP access leg and stops sending traffic over the old non-3GPP access leg. The AMF triggers AN release procedure over the old non-3GPP access and performs UDM Registration.

6.X.2	Procedures

This procedure assumes that the UE is registered over untrusted non-3GPP access first and then switches trusted non-3GPP access.

Figure 6.X.2-1. Overall non-3GPP access switching procedure

1.	The UE is registered over 3GPP access and untrusted non-3GPP access and established MA PDU Session. During the MA PDU Session Establishment, the UE indicates whether it supports non-3GPP access switching in the PDU Session Establishment Request message. The AMF also indicates whether it supports non-3GPP access switching to the SMF. Considering the received capabilities of the UE, AMF and SMF capability, the SMF indicates whether non-3GPP access switching is supported to the UE in the PDU Session Establishment Accept message.

2.	The UE registers over trusted non-3GPP access with new Registration type = non-3GPP access switching. The UE includes in the List Of PDU Sessions To Be Activated the MA PDU Sessions that user plane resources are established over untrusted non-3GPP access.

Editor's note:	It is assumed that the selected TNGF supports the S-NSSAI of the established MA PDU Session. Weather and how this is achieved is FFS.

3.	The AMF may perform authentication procedure based on existing procedure.

4.	The AMF sends Initial UE Context Setup Request message to the TNGF.

5.	After the Initial UE Context Setup procedure is completed, the AMF notifies to the SMF that the UE requested non-3GPP access switching.

6-7.	The SMF establishes user plane resources over the trusted non-3GPP access. At this point, there can be three user plane tunnels (i.e. 3GPP access, untrusted non-3GPP access, trusted non-3GPP access) in the UE and UPF. When the SMF establishes user plane resources over the trusted non-3GPP access, the SMF shall set the target access type to "trusted non-3GPP access" so that the AMF delivers the N2 information to the trusted non-3GPP access.

8.	The AMF performs UDM Registration by triggering Nudm_UECM_Registration service operation. The UDM triggers Nudm_UECM_DeregistrationNotify service according to the existing procedure. After this point, all signalling and Reachability procedures are performed over the trusted non-3GPP access.

9.	The AMF performs AN release procedure over the untrusted non-3GPP access. As a result of this procedure, user plane resources over untrusted non-3GPP access of MA PDU is completely released. When the UE and UPF recognize that user plane resources over untrusted non-3GPP access are released, the UE and UPF starts to send traffic over trusted non-3GPP access.

10.	The AMF sends Registration Accept message to the UE. When the UE receives Registration Accept message, the UE considers that the UE is deregistered from untrusted non-3GPP access and registered over trusted non-3GPP access.

6.X.3	Impacts on Existing Nodes and Functionality

UE:

-	Performs Registration with new registration type.

-	Indicates to the SMF that the UE supports non-3GPP access switching during the MA PDU Session Establishment.

-	Temporarily maintains simultaneous parallel user plane tunnel over untrusted 3GPP access and trusted non-3GPP access.

AMF:

-	During the Registration procedure, delay UDM registration until non-3GPP access switching is completed.

-	Indicates to the SMF that the AMF supports non-3GPP access switching during the MA PDU Session Establishment.

-	Notifies to the SMF that the UE requested non-3GPP access switching during the Registration procedure.

-	Performs AN release over old access during the Registration procedure

SMF:

-	Indicates to the UE whether the MA PDU Session supports non-3GPP access switching during the MA PDU Session Establishment.

UPF:

-	Temporarily maintains simultaneous parallel user plane tunnel over untrusted 3GPP access and trusted non-3GPP access.

* * * * End of Changes * * * *