3GPP TSG-SA WG2 Meeting #150E (e-meeting)			S2-2202283r04

April 6 – 12, 2022, Elbonia

Title:	Reply LS on Tx Profile

Response to:	LS from RAN2 on Tx Profile (S2-2201946/R2-2203693)

Release:	Rel-17

Work Item:	NR_SL_enh-Core, eV2XARC_Ph2, 5G_ProSe

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

Name:	LaeYoung Kim

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.287 CR (S2-2202284), TS 23.304 CR (S2-2202285)

1	Overall description

SA2 thanks RAN2 for their LS on Tx Profile in S2-2201946/R2-2203693.

Regarding the questions from RAN2, SA2 would like to provide the following answers.

- Question 1: May an L2 ID associate with multiple Tx profiles, and thus associate with both DRX-based Tx profile and non-DRX based Tx profile with the current SA2 spec? Based on the working assumption above, RAN2 assumes no additional RAN2 work if SA2 confirms it is feasible that an L2 id is only associated with either DRX-based TX profile(s) or non-DRX based TX profile(s).

[SA2 answer] SA2 would like to answer that an L2 ID may associate with both DRX-based Tx profile and non-DRX based Tx profile due to the reason as described below. SA2 could not reach any consensus on how an L2 ID is only associated with either DRX-based TX profile(s) or non-DRX based TX profile(s). However, SA2 would like to share the two approaches discussed so far.

This is because considering the following configurations regarding destination Layer-2 ID for broadcast and groupcast as well as the mechanisms to determine destination Layer-2 ID for groupcast specified in TS 23.287 for V2X services and TS 23.304 for ProSe services, it is hard to confirm that an L2 id is only associated with one NR Tx Profile, i.e. either DRX-based TX profile or non-DRX based TX profile. For example, two V2X service types can be mapped to an L2 ID where NR Tx Profile for one V2X service type is DRX-based TX profile while NR Tx Profile for the other V2X service type is non-DRX based TX profile.

In case there are different types of NR Tx Profile associated with an L2 ID and provided by the upper layer, the AS layer can disable SL DRX for the L2 ID. How the AS layer operates in this case is up to RAN2.

The two approaches discussed by SA2 for Question 1 are the following but none agreed :

- The upper layer provides either DRX-based TX Profile or non-DRX based TX Profile for an L2 ID. To do this, the upper layer maintains all the associated NR Tx Profile(s) for a certain DST L2 ID. When there is a need to pass the NR Tx Profile to the AS layer and multiple NR Tx Profiles are associated with the DST L2 ID, the upper layer passes non-DRX based TX Profile to the AS layer. Therefore, from the AS layer perspective, an L2 ID is only associated with either DRX-based TX Profile or non-DRX based TX Profile. This approach did not reach consensus in SA2 because it breaks the assumption made for LTE Tx Profile by SA2 and RAN2 that the contents of TX profile are transparent by V2X layer which means the upper layer does not look into the contents of Tx Profile (please see R2-1809166/S2-185513 about the assumption). Anyhow, SA2 would like to share this information with RAN2 for their discussion.

- Making an L2 ID only associated with either DRX-based TX Profile or non-DRX based TX Profile feasible with some assumption or expectation by adding something like the following NOTE:

NOTE:	It is assumed that destination Layer-2 ID(s) mapped from V2X service types associated with DRX-based NR Tx Profile do not overlap with destination Layer-2 ID(s) mapped from V2X service types associated with non-DRX based NR Tx Profile.

This approach also did not reach consensus in SA2 because this assumption or expectation may be not realistic.

- Question 2: For Groupcast, whether the mapping from L2 id to TX profile is feasible in the gNB (like what we did in LTE for broadcast, i.e. the mapping from Destination L2 ID to Tx Profile is configured in the gNB)? Based on the working assumption above, RAN2 assumes no additional RAN2 work if SA2 confirms it is feasible.

[SA2 answer] First, for Broadcast, the mapping from destination Layer-2 ID to NR Tx Profile can be configured in the NG-RAN like for LTE PC5 broadcast.

For Groupcast, there is the case using the destination Layer-2 ID mapped to service (i.e. V2X service type for V2X and ProSe service for ProSe). For this case, configuring the mapping from destination Layer-2 ID to NR Tx Profile in the NG-RAN is considered feasible.

There is also the case that a destination Layer-2 ID is generated with the group identifier information provided by the application layer. In this case, configuring the mapping from destination Layer-2 ID to NR Tx Profile in the NG-RAN is considered unfeasible.

SA2 understanding is that the decision on how the NG-RAN can know the mapping from destination Layer-2 ID to NR Tx Profile for Groupcast is within RAN2 remit and would like to request RAN2 to provide feedback to SA2 when this aspect is decided.

SA2 agreed the attached TS 23.287 CR and TS 23.304 CR to clarify PC5 DRX operations per the RAN2 agreements in S2-2201946/R2-2203693 and the SA2 answers above.

The attached TS 23.287 CR also clarifies the following aspect:

- the upper layer does not provide NR Tx Profile to the AS layer when there is no NR Tx Profile mapped for the relevant service. In this case, the AS layer can consider that SL DRX is not supported. How the AS layer operates in this case is up to RAN2.

Regarding the following RAN2 agreement, SA2 discussed whether the mapping of V2X service types to NR Tx Profiles for transmitting and receiving DCR message needs to be configured in the UE. This can be used for the case, e.g. that Rel-17 V2X UE intends to perform unicast mode communication with Rel-16 V2X UEs so that Rel-17 UE can receive DCR message sent by Rel-16 V2X UEs based on Non-DRX based NR Tx Profile.

SA2 would like to request RAN2 to provide feedback to SA2 on this aspect.

4. The default SL DRX configuration for BC/GC [(including at least DRX cycle, start offset and on-duration timer)] can be used for both BC-based and UC-based DCR message.

2. Actions:

To RAN2

ACTION:  	SA2 kindly asks RAN2 to take the above information into account for further work and to provide feedbacks on for Groupcast, how the NG-RAN can know the mapping from destination Layer-2 ID to NR Tx Profile based on the cases presented.

SA2 kindly asks RAN2 to provide feedback regarding RAN2 agreement#4 related aspect described above.

3. Dates of Next SA2 Meetings:

SA2#151E	May 16 – 20, 2022	Electronic meeting

SA2#152	August 22 – 26, 2022	Goteborg, Sweden