3GPP TSG-SA WG2 Meeting #150E (e-meeting)			S2-2202283r05

April 6 – 12, 2022, Elbonia

Title:	Reply LS on Tx Profile

Response to:	LS from RAN2 on Tx Profile (S2-2201946/R2-2203693)

Release:	Rel-17

Work Item:	NR_SL_enh-Core, eV2XARC_Ph2, 5G_ProSe

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

#### Name:	LaeYoung Kim

E-mail Address:	laeyoung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.287 CR (S2-2202284), TS 23.304 CR (S2-2202285)

# 1	Overall description

SA2 thanks RAN2 for their LS on Tx Profile in S2-2201946/R2-2203693.

Regarding the questions from RAN2, SA2 would like to provide the following answers.

- Question 1: May an L2 ID associate with multiple Tx profiles, and thus associate with both DRX-based Tx profile and non-DRX based Tx profile with the current SA2 spec? Based on the working assumption above, RAN2 assumes no additional RAN2 work if SA2 confirms it is feasible that an L2 id is only associated with either DRX-based TX profile(s) or non-DRX based TX profile(s).

[SA2 answer] SA2 would like to answer that an L2 ID may associate with both DRX-based Tx profile and non-DRX based Tx profile due to the reason as described below.

Considering the configurations of destination Layer-2 ID for broadcast and groupcast as well as the mechanisms to determine destination Layer-2 ID for groupcast specified in TS 23.287 for V2X services and TS 23.304 for ProSe services, it is impossible to limit that an L2 id is only associated with one NR Tx Profile, i.e. either DRX-based TX profile or non-DRX based TX profile. For example, two V2X service types can be mapped to an L2 ID where NR Tx Profile for one V2X service type is DRX-based TX profile while NR Tx Profile for the other V2X service type is non-DRX based TX profile. 3GPP has no control of the contents of these mappings.

SA2 had discussed several potential approaches to pass only one type of Tx Profile to AS layer but could not agree any solution, because those approaches require either V2X/ProSe layer to understand the Tx Profile contents, or mandating some deployment assumptions that is out of 3GPP's control.

It is assumed that the V2X/ProSe layer would maintain the active list of services and corresponding NR Tx Profiles associated with a certain Dst L2 ID. In case there are different types of NR Tx Profile associated with an L2 ID and provided by the upper layer, the AS layer can disable SL DRX for the L2 ID. How the AS layer operates in this case is up to RAN2.

- Question 2: For Groupcast, whether the mapping from L2 id to TX profile is feasible in the gNB (like what we did in LTE for broadcast, i.e. the mapping from Destination L2 ID to Tx Profile is configured in the gNB)? Based on the working assumption above, RAN2 assumes no additional RAN2 work if SA2 confirms it is feasible.

[SA2 answer] First, for Broadcast, the mapping from destination Layer-2 ID to NR Tx Profile can be configured in the NG-RAN like for LTE PC5 broadcast.

For Groupcast, there is the case using the destination Layer-2 ID mapped to service (i.e. V2X service type for V2X and ProSe service for ProSe). For this case, configuring the mapping from destination Layer-2 ID to NR Tx Profile in the NG-RAN is considered feasible.

There is also the case that a destination Layer-2 ID is generated with the group identifier information provided by the application layer. In this case, configuring the mapping from destination Layer-2 ID to NR Tx Profile in the NG-RAN is considered unfeasible.

SA2 understanding is that the decision on how the NG-RAN can know the mapping from destination Layer-2 ID to NR Tx Profile for Groupcast is within RAN2 remit and would like to request RAN2 to provide feedback to SA2 when this aspect is decided.

In addition to the above, SA2 would like to inform RAN2 that the attached TS 23.287 CR and TS 23.304 CR were agreed to clarify PC5 DRX operations per the RAN2 agreements in S2-2201946/R2-2203693 and the SA2 answers above. SA2 would like to seek feedback from RAN2 regarding the following aspect of the CR

:

- the upper layer does not provide NR Tx Profile to the AS layer when there is no NR Tx Profile mapped for the relevant service. In this case, the AS layer can consider that SL DRX is not supported. How the AS layer operates in this case is up to RAN2.

SA2 Question 1: Would this behaviour be compliant with RAN2's assumption, or would AS layer always expect a NR Tx Profile from upper layer?

Regarding the following RAN2 agreement, SA2 discussed

how the use of "default SL DRX configuration" is controlled for DCR message.

SA2 Question 2: Would the use of "default SL DRX configuration" also require the NR Tx Profile?

4. The default SL DRX configuration for BC/GC [(including at least DRX cycle, start offset and on-duration timer)] can be used for both BC-based and UC-based DCR message.

SA2 would like to request RAN2 to provide feedback on this aspect.

# 2. Actions:

To RAN2

ACTION:  	SA2 kindly asks RAN2 to take the above information into account for further work and to provide feedbacks.

SA2 kindly asks RAN2 to provide feedbacks on the two questions listed above.

# 3. Dates of Next SA2 Meetings:

SA2#151E	May 16 – 20, 2022	Electronic meeting

SA2#152	August 22 – 26, 2022	Goteborg, Sweden