3GPP SA WG2 Meeting #150E	S2-2202299

e-meeting, April 06-12, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1 Solution proposal 5GS network timing synchronization status and reporting – Solution proposal

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes a solution for the key issue #1 “5GS network timing synchronization status and reporting” to the TR 23.700-25.

# Discussion

The FS_5TRS_URLLC SID has a KI#1 for 5GS network timing synchronization status and reporting agreed in S2-2201762 with the following objectives:

-	Study how RAN and 5GC learn about 5GS network timing synchronization status to be able to inform UEs (e.g. application running in the UE), devices attached to the UE (i.e. that receive time information from 5GS) and AFs.

-	Study how to report 5GS network timing synchronization status (such as divergence from UTC and 5GS network timing source degradation) to UEs (e.g. application running in the UE), devices attached to the UE (i.e. that receive time information from 5GS) and 3rd party applications (AFs):

- 	Study if additional information needs to be provided to UEs and AFs to inform about 5GS network timing synchronization status.

To address them, the following solution is proposed for TR 23.700-25.

# Proposal

It is proposed to add the following solution to TR 23.700-25.

*** Start of changes (all new text) ***

## 6.x	Solution #x: 5GC learning and reporting network timing synchronization status

### 6.x.1	Introduction

This solution is proposed to solve Key Issue #1: 5GS network timing synchronization status and reporting.

In this key issue, the 5GS has a synchronization plane that synchronizes the 5G network functions (e.g., UPFs and RAN nodes) to a common time reference. The synchronization plane may have different sources of time/frequency like GNSS signal, Synchronous Ethernet (SyncE), PTP transport network, PPS input, etc. Thus, the following assumptions are considered for the synchronization plane:

-	NG-RAN is time synchronized with an external clock using transport network synchronization protocols or using a local GNSS receiver.

-	NG-RAN is frequency synchronized (i.e., synchronization) with an external clock using transport network syntonization methods or using a local GNSS receiver.

NOTE: 	The syntonization aspect is included in the assumptions because it may have an important role when the primary time reference source is lost but still the base station has a frequency reference (e.g., SyncE), then the holdover period can be longer. This is taking for example the long interruption failure scenarios considered in ITU-T G.8271.1 Appendix V

-	NG-RAN can detect network timing synchronization degradation/improvement or timing synchronization failures locally, e.g., based on information provided by the transport network time or frequency synchronization methods or based on information provided by the local GNSS receiver. The details of how NG-RAN detects timing synchronization degradation/improvement or timing synchronization failures are beyond the scope of 3GPP.

-	UPF/NW-TT is time synchronized with an external clock using transport network-based time synchronization protocols if UPF/NW-TT is involved in providing time information to UEs/DS-TTs.

-	UPF can detect network timing synchronization degradation/improvement or timing synchronization failures locally, e.g., based on information provided by the transport network time synchronization protocols. The details of how the UPF detects timing synchronization degradation/improvement or timing synchronization failures are beyond the scope of 3GPP.

This solution addresses the following scenarios:

-	5GS distributes time information to UEs using access stratum signaling.

-	5GS distributes time information to UEs using PTP or gPTP messages. If the 5GS is acting as a GM for the PTP instance, the network timing synchronization status reporting to UE/DS-TT or AF can complement the description of the GM the PTP messages already include (e.g., Announce messages) and detail the status of the 5G clock distributed via u-plane messages. If the 5GS is not the GM, the network timing synchronization status reporting to UE/DS-TT can assist determining the relative error between the 5G clock realizations at the DS-TT side and the NW-TT side used for (g)PTP timestamping operation.

Editor's note: Whether external gPTP clock is in scope is FFS. How to etermine the relative error between the 5G clock realizations at the DS-TT side and the NW-TT side used for (g)PTP timestamping operation is FFS.

From UE/DS-TT perspective, the 5GS time information received can be the primary time source the UE/DS-TT is consuming or can be a back-up time source alternative to a time source already present at the UE/DS-TT side. For both cases, this solution proposes enablers to allow the 5GC to retrieve 5GS network timing synchronization status from NG-RAN nodes and UPF/NW-TT (if needed) and report this information to subscribed UEs and AFs.

### 6.x.2	Functional Description

#### 6.x.2.1	Functional Description for 5GC learning network timing synchronization status

The following principles are proposed to enable the 5GC to learn the network timing synchronization status at the NG-RAN and UPF/NW-TT:

-	The TSCTSF can retrieve and store timing synchronization status from NG-RAN and UPF/NW-TT.

-	The NG-RAN timing synchronization status and the UPF/NW-TT timing synchronization status provide information for different time synchronization processes the UE/DS-TT(s) may have configured. The TSCTSF checks the time synchronization distribution method the target UE(s) have configured and determines the network functions to subscribe:

-	The NG-RAN timing synchronization status informs the synchronization performance of the time distribution process the gNB and UE execute at Uu interface using access stratum signaling.

-	On top of the access stratum time distribution method, if the UE/DS-TT receives 5G clock via (g)PTP (with the GM behind UPF/NW-TT) or the UPF/NW-TT performs timestamping operation of (g)PTP messages, the UPF/NW-TT timing synchronization status informs the synchronization performance of the time distribution process between the UPF/NW-TT and the UE/DS-TT. When UPF/NW-TT acts as PTP GM, GM quality attributes (e.g., clockClass, offsetScaledLogVariance, and ClockAccuracy) already present in PTP Announce messages may be enough and the TSCTSF can avoid subscribing to UPF/NW-TT timing synchronization status.

Editor's note:	How TSCTSF can learn the UPF/NW-TT time synch status is FFS.

-	For TSCTSF subscription to NG-RAN timing synchronization status, two reporting alternatives via AMF are possible as follows:

- Alternative 1 for NG-RAN Time Sync Status reporting: The TSCTSF is responsible for determining the impacted UE(s) based on their location and the NG-RAN timing synchronization status reports received from AMF.

- Alternative 2 for NG-RAN Time Sync Status reporting: The AMF is responsible for determining the impacted UE(s) based on their location and the NG-RAN timing synchronization status reports received. Per impacted UE, the AMF forwards the notification to the TSCTSF subscribed to it.

-	For TSCTSF subscription to UPF/NW-TT timing synchronization status, three reporting alternatives are possible as follows:

- Alternative 1 for UPF/NW-TT Time Sync Status reporting: Using UPF event exposure service operation. The TSCTSF can be a new consumer of the service and node level signalling is used between UPF and TSCTSF.

- Alternative 2 for UPF/NW-TT Time Sync Status reporting: Using N4 Node Level procedures between the UPF and the SMF to report the update from the UPF. The TSCTSF subscribes to this information at the SMF on a per UE level or node level (between UPF and SMF or between SMF and TSCTSF). If UE level signalling is preferred, the SMF is responsible for determining the impacted UE(s) and notify the TSCTSF per UE level basis using PDU Session information available.

- Alternative 3 for UPF/NW-TT Time Sync Status reporting: Using UMIC to forward the status update from the UPF to the TSCTSF. Node level signalling is used between UPF and TSCTSF.

-	The TSCTSF can determine time source degradation/failure/recovery events using the event flags and/or comparing numeric attributes included within the network timing synchronization status update received from NG-RAN and UPF/NW-TT.

-	The TSCTSF can use the primary source quality attributes included within the NG-RAN and UPF/NW-TT timing synchronization status update to compute a new 5GS error budget for the time synchronization service offered to UE(s) to assist time synchronization enforcement.

Editor's note:  It is FFS what this means because the 5GS error budget comes from the AF and cannot be changed unilaterally by the TSCTSF.

This depends on the progress of the related key issue #3.

#### 6.x.2.2	Functional Description for network timing synchronization status information

The following attributes are proposed for composing network timing synchronization status at the NG-RAN and UPF/NW-TT, as described in Table 6.x.2.2:

Table 6.x.2.1: Information element contained in network timing synchronization status

| Information Name | Description | Category |
| --- | --- | --- |
| Network function identification | Information to identify the network function (i.e., a NG-RAN cell or a UPF/NW-TT). | Mandatory |
| Synchronization state | Indicates the state of the network element synchronization, represented by the values: locked, holdover, freerun. | Mandatory |
| Network function synchronization performance | Indicates the synchronization performance the network element can have based on its implementation, that is, independent of the primary time reference source. | Optional |
| Primary source | Description of the primary source the network element uses for synchronization. Attributes: Type, Source Identity, Quality, Lock State. | Mandatory |
| Primary source events | Defines a list of events regarding the primary source notified from the network element. List of events: Switch, Primary Source Degradation, Preferred Source Recovery, etc. | Optional |

Editor's note: Which of this information is needed and how it is used is FFS.

For each information element mentioned in Table 6.x.2.2, the detailed description is as follows:

1) Network function identification:

- For a NG-RAN node the needed granularity is a cell ID (i.e., NG-RAN Cell Global ID (NCGI)).

- For a UPF/NW-TT node the needed granularity is the UPF ID.

2) Synchronization state:

- Locked: clock is in the locked mode, as defined in ITU-T G.810.

- Holdover: clock is in the holdover mode, as defined in ITU-T G.810.

- Freerun: clock isn't locked to an input reference, and is not in the holdover mode, as defined in ITU-T G.810.

3) Network function synchronization performance: This can be represented by performance/fault management flags already present in the OAM synchronization plane monitoring or other attributes that describe the expected node performance (e.g., uncertainty at the network function, frequency stability, etc).

4) Primary source. This may contain:

- Source type: SyncE, PTP, GNSS.

- Source identity. To identify the GM, values per type of source:

- SyncE: local-port-number.

- PTP: grandmasterClockIdentity.

- GNSS: Satellite constellation (e.g., GPS, GLONASS, GALILEO, BEIDOU).

- Quality. Clock source quality description, values per type of source:

- SyncE: SSM clock quality-level.

- PTP: clockClass, ClockAccuracy, offsetScaledLogvariance.

- GNSS: satellites-tracked, gnss-rx-time-error.

- Lock state. Indicates the status of the primary source, values per type of source:

- SyncE: Parent, OK, NOK, Disabled.

- PTP: Parent, OK, NOK, Disabled.

- GNSS: synchronized, acquiring-sync, antenna-disconnected, initializing, antenna-short-circuit.

Note: 	The lock state values indicate: Parent: the signal from this source is currently used as a synchronization reference; OK: Indicates that the signal from this source can be potentially used as a synchronization reference; NOK: Indicates that the signal from this source cannot be used as a synchronization reference; Disabled: Indicates that source connection is not available.

5) Primary source event. The list of events may contain:

- Primary source switch: Change of primary source due to a failure. If this flag is active, the primary source information should be updated in the generated report.

- Primary source degradation: If this flag is active, the quality of the primary source or performance attributes should be updated in the generated report.

- Primary source recovery: If this flag is active, the preferred primary source that was in disabled or in NOK state is available again (OK state).

#### 6.x.2.3	Functional Description for AF requested network timing synchronization status

The following capabilities are proposed for AF requesting network timing synchronization status:

-	The AF requests network timing synchronization status monitoring from the NEF or TSCTSF (if it is a trusted AF). The request may contain:

-	Targets of the monitoring and filtering information (e.g., UE IDs, spatial validity, DNN/S-NSSAI).

-	The subscription events or information elements it wants to be notified (e.g., all information elements listed in Table 6.x.2.2 or a subset of the information elements).

-	Request for UE side reporting (i.e., the UE(s) that are targets of the monitoring receive also the network timing synchronization status report) and reporting criterion (e.g., as soon as new information is available, when the UE is reachable).

-	AF Identification.

Editor's note: Whether a new request for network timing synchronization status monitoring is needed or whether the existing rquest for time synchronization is reused is FFS”

-	To determine the targets of the monitoring, AF request for network timing synchronization status provides the filter information. Four criteria to determine the targets of the network timing synchronization status exposure framework can be considered as summarized in Table 6.x.2.3 based on UE identities, known location, or connectivity.

Table 6.x.2.3: Criteria alternatives for network timing synchronization status exposure

| Criteria | Spatial validity | All UEs within the spatial validity | All UEs connected to the DNN/S-NSSAI | A UE or group of UEs |
| --- | --- | --- | --- | --- |
| Target of Monitoring Reporting | an optional SUPI or any UE | any UE | any UE | one or more SUPI(s) or Internal Group Identifier(s) |
| Monitoring Filter information | spatial validity; | Spatial validity | DNN; S-NSSAI; |  |

-	If AF’s request includes geographical area filter(s), the AF can provide them in spatial validity format (e.g., a civic address or shapes), or area(s) of interest. If the AF provides spatial validity, the NEF/TSCTSF maps the spatial validity to validity area(s) (based on pre-configuration). Later, the TSCTSF or PCF determines the area(s) of interest based on validity area(s). The TSCTSF or AMF subscriptions use area of interest that may be the same as spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

-	AMF or TSCTSF uses Location reports or UE presence in Area of Interest services at the AMF to identify the UE(s) to which the AF request with geographical area filter(s) applies, following the description of clause 6.x.2.1.

-	If AF’s request includes DNN/S-NSSAI filters, service events related to PDU Sessions (e.g., establishment or release) can be used at the SMF or TSCTSF to identify the UE(s) to which the request applies.

#### 6.x.2.4	Functional Description for network timing synchronization status reporting to UE(s)

The following capabilities are proposed for UE(s) receiving network timing synchronization status:

-	Based on TSCTSF policies or the received AF’s network timing synchronization status request as described in clause 6.x.2.3 and the time synchronization service the UE has configured, the TSCTSF is responsible for subscribing to network timing synchronization status as described in clause 6.x.2.1 (i.e., to the serving NG-RAN via AMF and if needed to the UPF/NW-TT via SMF or UPF directly).

-	The TSCTSF gathers the status from the network functions and generates the network timing synchronization status report to be forwarded to the UE/DS-TT(s). Depending on the time distribution method the UE has configured, the TSCTSF may need to compound a report using the input received from the serving NG-RAN node and the UPF/NW-TT or group the information coming from the NG-RAN node and UPF/NW-TT node into one report to indicate to the UE/DS-TT the performance of the gNB and the UPF/NW-TT related to the 5G clock.

Editor's note: How the TSCTSF can create a single report based on RAN and UPF time synch status and what the report would contain is FFS.

-	When access stratum time distribution method is configured for the UE, the TSCTSF could configure the NG-RAN node (via AMF) to generate and send the NG-RAN timing synchronization status report to the UE(s) via SIB9/RRC signalling. The report shall be made available to DS-TT by the UE.

-	When (g)PTP time distribution method is configured for the UE/DS-TT, the DS-TT is the receptor of the network timing synchronization status report. TSCTSF could provide network timing synchronization status reports to the DS-TT via the UE using NAS signalling to transport the report, e.g., extending Port Management Information Container (PMIC).

Editor's note: Whether there is a need for additional information to be signaled to DS-TT in case of (g)PTP based time distribution (and how the DS-TT should use it) or if the PTP information that can already be signaled to DS-TT is sufficient is FFS.

-	If the DS-TT is the last recipient of the network timing synchronization status reports (if only access stratum time distribution method is configured), it is up to the DS-TT implementation the determination of how the time source status update impacts the total degradation of the timing service the DS-TT is running.

### 6.x.3	Procedures

#### 6.x.3.1	Procedure for AF requested network timing synchronization status

An overall procedure for AF requested network timing synchronization status is illustrated in Figure 6.x.3.1-1.

Figure 6.x.3.1-1: Procedure for AF requested network timing synchronization status

1.	The AF sends network timing synchronization status request to the NEF or TSCTSF. The request may include targets of the monitoring and filtering information (as described in Table 6.x.2.3), the subscription events or information elements it wants to be notified, request for UE side reporting and reporting criterion, AF Identification.

2.	If the request is received at the NEF, it checks whether the AF is authorized to send the request and forwards the request to the TSCTSF.

If spatial validity is included in AF’s request, NEF or TSCTSF may determine the validity area(s).

Based on the filters the request contains, the TSCTSF determines the target(s) UE(s) for the request.

3.	The TSCTSF determines the AMF(s) and the UPF/NW-TT nodes (if applicable) that needs to initiate network timing synchronization status subscription considering the time synchronization service the target UE(s) have configured in the 5GS.

To determine the serving AMF(s), the TSCTSF can use different methods such as:

-	Method 1: NRF services (Nnrf_NFDiscovery) or UDM UE Context Management services (Nudm_UECM_Get) to discover the AMF of the target TAI(s) or UE(s). This method enables the TSCTSF to directly interact with the AMF to retrieve NG-RAN Time Sync Status reports at UE or NG-RAN node level.

-	Method 2: AM policy procedures via PCF. The TSCTSF can discover the PCF for the UE using the BSF and interact with the AMF via PCF AM Policy services. This method only enables the TSCTSF to retrieve NG-RAN Time Sync Status reports at UE level.

To determine the serving UPF/NW-TT(s) the TSCTSF can use the PTP instance information already configured for the (g)PTP service provided to the UE(s). That is, the TSCTSF can reuse the time synchronization capability exchange via UMIC with the UPF/NW-TT (e.g., attribute List of UEs associated with the User-Plane Node ID in 23.502 Table 5.2.6.25.8-1).

4.	The TSCTSF request NG-RAN timing synchronization status subscription to the AMF. Two alternatives are possible (as described in 6.x.2.1):

-	Alternative 1 for NG-RAN Time Sync Status reporting: TSCTSF subscribes for receiving NG-RAN timing synchronization status at the AMF on a per RAN node level. The AMF forwards the received status report from the NG-RAN node to the TSCTSF. The TSCTSF is responsible for determining the impacted UE(s) based on their location.

-	Alternative 2 for NG-RAN Time Sync Status reporting: TSCTSF subscribes for receiving NG-RAN timing synchronization status at the AMF on a per UE level. The AMF is responsible for determining the impacted UE(s) based on the NG-RAN node level report received and UE(s) location. Per impacted UE, the AMF forwards the NG-RAN time synchronization status report to the TSCTSF subscribed to it.

If (g)PTP time distribution is configured for a UE, the TSCTSF request UPF/NW-TT timing synchronization status subscription to the SMF (at UE level or node level) or directly at the UPF/NW-TT (node level), as described in 6.x.2.1.

5.	If AF’s request contains a spatial validity filter and the TSCTSF subscribes to node level information at the AMF in step 4, the TSCTSF is responsible for determining if a NG-RAN timing synchronization status update received at node level signalling impacts the UE(s) matching AF’s request condition. To support this, the TSCTSF subscribes to location services at the AMF (e.g., Namf_EventExposure service for events like Location Report (TAI, Cell ID) or UE moving in or out of a subscribed "Area Of Interest").

If AF’s request contains a spatial validity filter and (g)PTP time distribution is configured for a UE, the TSCTSF is responsible of determining the UE(s) matching AF’s request condition based in their location and the serving UPF/NW-TT for the PTP instance.

6.	The TSCTSF responds the AF.

7.	The AMF request network timing synchronization status subscription to the NG-RAN node.

8.	The AMF subscribes to UE(s) location services reporting from NG-RAN node.

9.	The NG-RAN node detects a primary source event (e.g., degradation, failure, recovery).

10.	The NG-RAN node notifies the AMF providing a NG-RAN timing synchronization status report.

11.	The AMF forwards the NG-RAN timing synchronization status report to the TSCTSF. If TSCTSF subscription is at NG-RAN node level (alternative 1 for NG-RAN Time Sync Status reporting), the AMF can directly forward the report received from NG-RAN node. If TSCTSF subscription is at UE level (alternative 2 for NG-RAN Time Sync Status reporting), the AMF determines the UE(s) impacted by the status update received from NG-RAN before notifying the TSCTSF (e.g., based on UE locations or UE identities).

12.	The TSCTSF stores the network timing synchronization status update received and notifies the subscribed AF via exposure framework.

Figure 6.x.3.1-1 illustrates the NG-RAN timing synchronization status subscription example, if the TSCTSF requires UPF/NW-TT timing synchronization status (determined in step 3), a similar signalling exchange between UPF/NW-TT, SMF, and TSCTSF is required to subscribe to UPF/NW-TT updates (directly or via the SMF), as described in 6.x.2.1.

#### 6.x.3.2	Procedure for UE provisioning network timing synchronization status

An overall procedure for UE provisioned network timing synchronization status is illustrated in Figure 6.x.3.2-1.

Figure 6.x.3.2-1: Procedure for UE provisioning network timing synchronization status

1.	The AF sends network timing synchronization status request to the NEF or TSCTSF. The subscription configuration of NG-RAN nodes and UPF/NW-TT (if applicable) is performed as described in clause 6.x.3.1.

2.	The NG-RAN node detects a primary source event (e.g., degradation, failure, recovery).

3.	The NG-RAN node notifies the AMF providing a NG-RAN timing synchronization status report. The TSCTSF may have already configured the NG-RAN node to provide updates to the UE directly via SIB9/RRC signalling, in that case, steps 5 and 6 can be skipped.

4.	The AMF forwards the NG-RAN timing synchronization status report to the TSCTSF. If TSCTSF subscription is at NG-RAN node level (alternative 1 for NG-RAN Time Sync Status reporting), the AMF can directly forward the report received from NG-RAN node. If TSCTSF subscription is at UE level (alternative 2 for NG-RAN Time Sync Status reporting), the AMF determines the UE(s) impacted by the status update received from NG-RAN before notifying the TSCTSF (e.g., based on UE locations or UE identities).

5.	The TSCTSF determines the UE(s) to notify and the method to use to forward the network timing synchronization status notification.

If UE is using (g)PTP time distribution and there is an available UPF/NW-TT timing synchronization status report, the TSCTSF may include this information when determining the content to send to the UE.

Editor's note: Need to inform the UE about UPF time synch issues and how a UE could make use of this information is FFS.

Editor's note: Need to remove a UE from the PTP instance in case of time synchronization degradation is FFS.-

6.	The TSCTSF can initiate a network triggered procedure to forward the notification to the UE via AMF or can command the serving NG-RAN node to notify the UE via SIB9/RRC messages.

7.	The serving NG-RAN node forwards the network timing synchronization status to the UE.

Figure 6.x.3.2-1 illustrates the NG-RAN timing synchronization status notification example, for UPF/NW-TT timing synchronization status notification, a similar signaling exchange is required between UPF/NW-TT, SMF, and TSCTSF to retrieve the timing synchronization. For reporting UPF/NW-TT timing synchronization status report to the UE/DS-TT, the TSCTSF may use PMIC.

### 6.x.4	Impacts on services, entities and interfaces

-	UE:

-	Support receiving network timing synchronization status information.

-	Support performing a registration request when RAN network timing synchronization status information changes while the UE is in CM-IDLE, if requested by TSCTSF or AMF.

-	DS-TT:

-	Support receiving network timing synchronization status information.

-	NG-RAN:

-	Indicate NG-RAN network timing synchronization status to UEs via SIB9 or RRC signalling.

-	Report NG-RAN network timing synchronization status to AMF.

-	AMF:

-	Subscribe for network timing synchronization status reports from NG-RAN nodes.

-	Report NG-RAN timing synchronization status to the TSCTSF (at UE level or NG-RAN node level).

-	SMF:

-	If SMF is involved in UPF/NW-TT timing sync status reporting, subscribe for network timing synchronization status reports from UPF/NW-TT nodes.

-	Report UPF/NW-TT timing synchronization status to the TSCTSF (at UE level or UPF node level).

-	TSCTSF:

-	Receive network timing synchronization status information from NG-RAN (via AMF) and UPF/NW-TT (via SMF or directly).

-	Discover serving AMF for a UE.

-	Subscribe for receiving UE’s location, UE’s presence in Area of Interest, and reachability information from AMF.

-	Subscribe for receiving UE’s PDU Session events from SMF.

-	Support for requesting UEs to perform a Registration request if the UE detects a change in RAN network timing synchronization status information and the UE is in CM-IDLE.

-	Mapping from spatial validity to validity area(s) (based on pre-configuration).

-	Determination of area of interest based on the validity area(s).

-	Inform AFs about network timing synchronization status.

-	Initiate network-triggered procedures to reach the UE if is in CM-IDLE.

-	Inform UE(s) about network timing synchronization status using PMIC or configuration of NG-RAN node to notify the UE via RRC.

-	UPF:

-	Report transport network timing synchronization status to TSCTSF (directly or via SMF).

-	AF:

-	Request for network timing synchronization status.

-	Support receiving network timing synchronization status information.

*** End of changes ***