3GPP SA WG2 Meeting #150E	S2-2202300r01

e-meeting, April 06-12, 2022

Source:	Nokia, Nokia Shanghai Bell, CATT, Samsung, ZTE

Title:	KI#2 Solution proposal Time synchronization service enhancements – Solution proposal

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract of the contribution: This paper proposes a solution for the key issue #2 “Time synchronization service enhancements” to the TR 23.700-25.

# Discussion

The FS_5TRS_URLLC SID has a KI#2 for time synchronization service enhancements agreed in S2-2201762 with the following objectives:

-	How to enable AFs to request time synchronization service in a specific coverage area and whether and how to enforce the coverage area.

To address them, the following solution is proposed for TR 23.700-25.

# Proposal

It is proposed to add the following solution to TR 23.700-25.

*** Start of changes (all new text) ***

## 6.x	Solution #x: Geographical area filters for time synchronization service

### 6.x.1	Introduction

This solution is proposed to solve Key Issue #2: Time synchronization service enhancements. In this Key Issue, the coverage area is taken into account for the configuration of the time synchronization service. Therefore, this solution addresses the time synchronization service scenarios already supported in 5G Release-17 based on access stratum or (g)PTP time distribution methods.

This solution proposes to use Tracking Areas (TAs) identified by a list of Tracking Area Identities (TAI) to describe a specific geographical area (a so-called Requested Coverage Area) where an AF requests to enable a time synchronization (TS) service. Furthermore, in order to enforce that the requested TS service is enabled for UEs (a specific UE or a group of UEs) only in that Requested Coverage Area, the proposed solution exploits the 5GS functionality (provided by the AMF) of tracking and reporting “UE mobility on Area of Interest” events (clause 5.3.4.4 of TS 23.501). An Area of Interest for each AMF is represented by the TA(s), wherein the Area of Interest is identical to the Requested Coverage Area or the Area of Interest is a TA subset of the Requested Coverage Area.

### 6.x.2	Functional Description

The existing time synchronization exposure procedures (clause 4.15.9 of TS 23.502 [3]) are enhanced.

The following principles are proposed to enable the use of coverage area for time synchronization service operation:

-	An AF may optionally include geographical area filter(s) (i.e., Requested Coverage Area) within the timing synchronization service request.

-	The AF may provide as geographical area filter(s) spatial validity information (e.g., a civic address or shapes), or area(s) of interest (AoI), or a TAI list (if it is an AF within the operator’s domain, it will have TA(s) configured).

- 	If the AF provides spatial validity, the NEF/TSCTSF maps the spatial validity to validity area(s) (based on pre-configuration). Later, the TSCTSF or PCF determines the area(s) of interest based on validity area(s). The TSCTSF or AMF subscriptions use area of interest that may be the same as spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

-	If the AF provides a AoI, the NEF/TSCTSF maps the information to a list of TAs.

-	TSCTSF uses Location or UE presence in Area of Interest services at the AMF Event Exposure service  (Namf_EventExposure operations) to identify the UE(s) to which the AF request with a Requested Coverage Area applies. The TSCTSF subscribes to this information at the AMF(s) and stores it.

-	Requested Coverage Area will be used at the TSCTSF for triggering time synchronization service activation/modification/deactivation with the following differences: (1) the TSCTSF discovers the AMF(s) serving in the Tracking Areas (TAs) comprising the Requested Coverage Area; (2) the TSCTSF subscribes (using the Namf_EventExposure_Subscribe service operation specified in clause 5.2.2.3.2 of TS 23.502 [3]) to the UE mobility on Area of Interest event notification service from the AMF(s) to be notified the UE(s) presence in an Area of Interest for each AMF (e.g. UE location is IN or OUT an Area of Interest); (3) the TSCTSF determines (based on the notification from the AMF(s)) whether or not the targeted UE(s) are inside the Requested Coverage Area; and (4) the TSCTSF proceeds with the time synchronization service activation/deactivation only after the TSCTSF has determined that the targeted UE(s) has moved in/out the Requested Coverage Area. The subscription to reports of UE presence in the Area of Interest procedure is described in clause 5.3.4.4 of TS 23.501 [2].

-	For access stratum distribution activation/deactivation, the TSCTSF will enable/disable ReferenceTimeInformation delivery to the UE at the serving NG-RAN node. For (g)PTP distribution activation/deactivation, the TSCTSF will modify the PTP instance configuration by means of sending a PMIC to the impacted UE/DS-TT and UMIC to the UPF/NW-TT, as described in clause K.2.2 of TS 23.501 [2].

Editor's note: Whether to enable/disable 5G access stratum time distribution for UE(s) based on RAN report of AoI is FFS.

### 6.x.3	Procedures

#### 6.x.3.1	Procedure for AF requested (g)PTP timing synchronization with geographical area filters

An overall procedure for AF providing coverage area filter(s) for (g)PTP based time synchronization service is illustrated in Figure 6.x.3.1-1.

Figure 6.x.3.1-1: (g)PTP time distribution configuration with geographical area filters

1.	The AF creates a time synchronization service configuration for a PTP instance by invoking Nnef_TimeSynchronization_ConfigCreate service operation. The request includes the parameters as described in table 4.15.9.3.1 in TS 23.502 [3] and optionally a spatial validity condition (i.e., Requested Coverage Area) in the format of spatial validity, or area(s) of interest, or list of TA(s).

2.	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_TimeSynchronization_ConfigCreate service operation with the corresponding TSCTSF, with the parameters as received from the AF.

The AF that is part of operator's trust domain may invoke the services directly with TSCTSF.

3.	If the AF provided spatial validity, the TSCTSF maps the information to areas of validity based on pre-configuration, then the TSCTSF determines area(s) of interest based on validity area(s) and the corresponding list of TA(s).

4.	The TSCTSF discovers the AMF(s), serving in the Tracking Areas (TAs) that comprise the spatial validity condition, using the NRF discovery service (Nnrf_NFDiscovery_Request) with the TAs.

5.	The spatial validity condition (or the area(s) of interest condition) for the UE(s) is resolved at the TSCTSF. In order to do that, the TSCTSF subscribes to the AMF to receive notifications about the UE presence in an area of interest (or UE location) using Namf_EventExposure operation. The subscribed area of interest may be the same as the spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

The AMF(s) track the UE’s location to determine the UE’s presence in an Area of Interest as described in clause 4.15.4.2 of TS 23.502 [3]. Further, the AMF(s) notify the TSCTSF about the UE(s) presence (IN, OUT, or UNKNOWN) in the Area of Interest(s).

6. 	According to the UE location and spatial validity condition, the TSCTSF determines whether to active time synchronization service:

-  If the UE location is in an Area of Interest, the TSCTSF determines to active time synchronization service;

- 	If the UE location is out of an Area of Interest, the TSCTSF determines not to active time synchronization service, or determines to hold a while according to Temporal Validity Condition (if this parameter is available, and current time is within validity time period);

7.	The TSCTSF responds with the Ntsctsf_TimeSynchronization_ConfigCreate response.

8.	The NEF responds with the Nnef_TimeSynchronization_ConfigCreate response.

9. 	The TSCTSF uses the procedures described in clause K.2.2 of TS 23.501 [2] to configure and initialize the PTP instance in the DS-TT(s) and NW-TT.

10.	The TSCTSF uses the procedure in clause 4.15.9.4 of TS 23.502 [3] to manage the 5G access stratum time distribution for the UEs that are part of the impacted PTP instance.

11. 	UE moves in/out of the configured area of interest.

12. NG-RAN determines the UE presence in the area of interest has changed and notifies the AMF.

13.	The AMF detects the subscription change related event occurs and it sends the event report by means of Namf_EventExposure_Notify message to the TSCTSF.

14. If the TSCTSF receives the UE location change notification for AoI. The TSCTSF may update to activate or deactivate the time synchronization service.

- 	If the UE moves out of an Area of Interest, TSCTSF determines to deactivate time synchronization service and proceeds as Ntsctsf_TimeSynchronization_ConfigDelete was received as described in clause 4.15.9.3.4 TS 23.502 [3];

- 	If the UE moves into an Area of Interest, and according to Temporal Validity Condition (if this parameter is available, and current time is within validity time period), the TSCTSF determines to activate time synchronization service;

-	The TSCTSF keeps the requested TS service unchanged (i.e., active/inactive) when the UE presence in the spatial validity condition becomes UNKNOWN until the TSCSTF determines that the UE is inside/outside the spatial validity condition.

15.	The TSCTSF updates the state of the time synchronization configuration and may notify the NEF (or AF) with the Ntsctsf_TimeSynchronization_ConfigUpdateNotify service operation.

16.	The NEF notifies the AF with the Nnef_TimeSynchronization_ConfigUpdateNotify service operation.

#### 6.x.3.2	Procedure for AF requested access stratum timing synchronization with geographical area filters

An overall procedure for AF providing coverage area filter(s) for access stratum based time synchronization service (refer TS 23.502) is illustrated in Figure 6.x.3.2-1 .

Figure 6.x.3.2-1: 5G access stratum time distribution configuration with geographical area filters

1.	The AF request to influence the 5G access stratum time distribution providing access stratum time distribution parameters to the NEF (together with the AF identifier and potentially further inputs as specified in table 4.15.9.4-1 in TS 23.502 [3]) and optionally a spatial validity condition (i.e., Requested Coverage Area) in the format of spatial validity, or area(s) of interest, or list of TA(s).

2.	The NEF authorizes the request. After successful authorization, the NEF invokes the Ntsctsf_ ASTI_Create service operation with the corresponding TSCTSF.

3.	If the AF provided spatial validity, the TSCTSF maps the information to areas of validity based on pre-configuration, then the TSCTSF determines area(s) of interest based on validity area(s) and the corresponding list of TA(s).

4.	The TSCTSF discovers the AMF(s), serving in the Tracking Areas (TAs) that comprise the spatial validity condition, using the NRF discovery service (Nnrf_NFDiscovery_Request) with the TAs.

5.	The spatial validity condition (or the area(s) of interest condition) is resolved at the TSCTSF. In order to do that, the TSCTSF subscribes to the AMF to receive notifications about change of UE location in an area of interest using Namf_EventExposure operation. The subscribed area of interest may be the same as spatial validity condition or may be a subset of the spatial validity condition (e.g., a list of TAs) based on the latest known UE location.

The AMF(s) track the UE’s location to determine the UE’s presence in an Area of Interest as described in clause 4.15.4.2 of TS 23.502 [3]. Further, the AMF(s) notify the TSCTSF about the UE(s) presence (IN, OUT, or UNKNOWN) in the Area of Interest(s).

6.	According to the UE location and spatial validity condition, the TSCTSF determines whether to active time synchronization service:

-  If the UE location is in an Area of Interest, the TSCTSF determines to active time synchronization service;

- 	If the UE location is out of an Area of Interest, the TSCTSF determines not to active time synchronization service, or determines to hold a while according to Temporal Validity Condition (if this parameter is available, and current time is within validity time period);

7.	The TSCTSF responds the AF with the Ntsctsf_ASTI_Create service operation response.

8.	The NEF informs the AF about the result of the Nnef_ASTI_Create service operation performed in step 1.

9.	The TSCTSF uses the procedures described in clause 4.15.9.4 of TS 23.502 [3] to configure ReferenceTimeInformation delivery to the UE(s) at the serving NG-RAN nodes.

10. 	UE moves in/out of the configured area of interest.

11. 	NG-RAN determines the change of the UE presence in the area of interest has changed and notifies the AMF.

12.	The AMF detects the subscription change related event occurs and it sends the event report by means of Namf_EventExposure_Notify message to the TSCTSF.

13.  If the TSCTSF receives the UE location change notification for AoI. The TSCTSF may update to activate or deactivate the time synchronization service.

- 	If the UE moves out of an Area of Interest, TSCTSF determines to deactivate ASTI time synchronization service as described in clause 4.15.9.4 TS 23.502 [3];

- 	If the UE moves into an Area of Interest, and according to Temporal Validity Condition (if this parameter is available, and current time is within validity time period), the TSCTSF determines to activate ASTI time synchronization service as described in clause 4.15.9.4 TS 23.502 [3];

-	The TSCTSF keeps the requested TS service unchanged (i.e., active/inactive) when the UE presence in the spatial validity condition becomes UNKNOWN until the TSCSTF determines that the UE is inside/outside the spatial validity condition.

14.	The TSCTSF updates the state of the time synchronization configuration and may notify the NEF (or AF) with the Ntsctsf_ASTI_UpdateNotify service operation.

15.	The NEF notifies the AF with the Nnef_ASTI_UpdateNotify service operation.

### 6.x.4	Impacts on services, entities and interfaces

-	TSCTSF:

-	Subscribe for receiving UE’s presence in Area of Interest from AMF.

-	Mapping from spatial validity to validity area(s) (based on pre-configuration).

-	Determination of area of interest based on the validity area(s).

-	Support for time synchronization service configuration conditioned to geographical area filter(s) received from AF.

-	Discovers the related AMF(s) using the NRF’s service operation Nnrf_NFDiscovery_Request.

-	AMF:

-  Tracks a UE’s location to determine the UE’s presence in an Area of Interest using the existing event reporting type, i.e., UE mobility on Area of Interest.

-	Notifies the subscribed TSCTSF about the change of UE’s status (IN, OUT, or UNKNOWN) in the Area of Interest using the (existing) Namf_EventExposure_Notify service operation.

-	AF:

-  Includes the Spatial Validity Condition in AF request for time synchronization service configuration.

*** End of changes ***