3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202346r01

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Solution for KI#6 on ranging and sidelink positioning service exposure to UE

Document for:	Approval

Agenda Item:	9.20

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: the KI#6 is captured in TR 23.700-86 to study the ranging and sidelink positioning service exposure to UE, thus it proposes a solution to enable such service exposure.

1. Introduction

In SA2#149 meeting, it is approved to separate the ranging and sidelink positioning service exposure into two KIs, i.e. Key Issue #6 for Ranging/Sidelink positioning service exposure to a UE, and Key Issue #7 for Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning. This contribution proposes the end-to-end procedures for Key Issue #6.

2. Discussion

The followings discuss two potential cases:

- Case1: two UEs, the ranging UE consumes the ranging/sidelink positioning service.

- Case2: three UEs, the 3rd UE consumes the ranging/sidelink positioning service.

2.1 Case1: UE involved in SL ranging consumes the ranging/sidelink positioning service

RAN

UE1

5GC

UE2

1. SL Ranging execution

2. Local calculation

Figure 2.1-1: UE involved in SL ranging consumes the ranging/sidelink positioning service

In this case, two UEs are involved in the ranging/sidelink positioning. The UE triggers ranging service, performs the ranging/sidelink positioning operation, and calculates the result locally, i.e. Ranging/Sidelink positioning service is exposed to one of the two UEs.

2.2 Case2: 3rd UE consumes the ranging/sidelink positioning service.

1a. 3rd UE requests via PC5

3rd UE

UE 1

UE 2

RAN

5GC

1b. 3rd UE requests via 5GC

3. Ranging result exposure

2. SL Ranging execution

Figure 2.2-1: 3rd UE consumes the ranging/sidelink positioning service

In this case, three UEs are involved in the ranging/sidelink positioning, in which a 3rd UE asks for the ranging/sidelink positioning for the other two UEs (i.e., UE1 and UE2). The 3rd UE may request the 5GC (e.g. LMF) via Uu interface to perform ranging/sidelink positioning between UE1 and UE2. If the 3rd UE is in the sidelink coverage with one of the requested UEs (e.g., UE1), the 3rd UE may alternatively request UE1 via PC5 interface to perform ranging/sidelink positioning with UE2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 6 |  |  |  |
| x | X |  |  |  |

# * * * * Second change * * * *

6.x	Solution #x: Solution for KI#6 on ranging/sidelink positioning service exposure to UE

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the subclauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

6.x.1	General

The solutions address the ranging and sidelink positioning service exposure to UE for Key Issue #6.

The followings give two potential cases and the corresponding solutions:

- Case1: two UEs, the ranging UE consumes the ranging/sidelink positioning service.

- Case2: three UEs, the 3rd UE consumes the ranging/sidelink positioning service.

6.x.2	Functional descriptions

Ranging (and SL positioning) refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

The consumer can be the UE involved in the ranging/sidelink positioning service, or a 3rd UE. The UE/3rd UE may perform the ranging/sidelink positioning directly via PC5 interface and use the result locally, without involvement of 5GC (e.g., LMF), or the 3rd UE (e.g. out of SL coverage) should request the 5GC (e.g. LMF) for the ranging/sidelink positioning service.

6.x.3	Procedures

6.x.3.1	UE involved in SL ranging consumes the ranging/sidelink positioning service

RAN

UE1

5GC

UE2

1. SL Ranging execution

2. Local calculation

Figure 6.x.3.1-1: UE involved in SL ranging consumes the ranging/sidelink positioning service

In this case, two UEs are involved in the ranging/sidelink positioning. The UE triggers ranging service, performs the ranging/sidelink positioning operation, and makes the calculation locally, i.e. Ranging/Sidelink positioning service is exposed to one of the two UEs.

Reference UE

NG-RAN

AMF

LMF

Target UE

2. Ranging (and SL positioning)

1. Ranging service triggers

3. Calculation

Figure 6.x.3.1-2: Procedure of ranging UE consumes the ranging/sidelink positioning service

1.	The application layer of the UE (e.g. Target UE) triggers the request for ranging/sidelink positioning.

2.  The Target UE initiates the ranging/sidelink positioning with the Reference UE. The NG-RAN node is involved if UE is in network coverage and the operator managed radio resource is used.

3. The Target UE calculates the result and uses it locally.

6.x.3.2	3rd UE consumes the ranging/sidelink positioning service.

6.x.3.2.1	General

1a. 3rd UE requests via PC5

3rd UE

UE 1

UE 2

RAN

5GC

1b. 3rd UE requests via 5GC

3. Ranging result exposure

2. SL Ranging execution

Figure 6.x.3.2.1-1: 3rd UE consumes the ranging/sidelink positioning service

In this case, three UEs are involved in the ranging/sidelink positioning, in which a 3rd UE asks for the ranging/sidelink positioning for the other two UEs (i.e., UE1 and UE2). The 3rd UE may request the 5GC (e.g. LMF) via Uu interface to perform ranging/sidelink positioning between UE1 and UE2. If the 3rd UE is in the sidelink coverage with one of the requested UEs (e.g., UE1), the 3rd UE may alternatively request UE1 via PC5 interface to perform ranging/sidelink positioning with UE2.

6.x.3.2.2	Service exposure to the 3rd UE via PC5

Reference UE

NG-RAN

AMF

LMF

Target UE

3. Calculation

3rd-UE

1. SL ranging request (SL ranging measurement)

4. SL ranging response (result)

2. Ranging (and SL positioning) procedure

Figure 6.x.3.2.2-1: Service exposure to the 3rd UE via PC5

1. The application layer of the 3rd UE triggers the request for the ranging/sidelink positioning

2. The 3rd UE requests the UE (e.g. Target UE) to perform the ranging/sidelink positioning with the Reference UE. The NG-RAN node is involved if UE is in network coverage and the operator managed radio resource is used.

3. The Target UE makes the calculation locally.

4. The Target UE returns the ranging/sidelink positioning result to the 3rd UE via the PC5 interface.

6.x.3.2.3	Service exposure to the 3rd UE via 5GC/Uu

3rd UE

NG-RAN

AMF

LMF

GMLC

2. UL NAS TRANSPORT (SL ranging requirement, Reference UE ID, Target UE ID)

NEF

AF

Target UE

Reference UE

5. SL measurement request (ranging/sidelink positioning demand, Target UE ID, Reference UE ID)

6. UE Ranging with other UEs

10. DL NAS TRANSPORT (SL ranging result)

7. SL measurement report (Target UE ID, Reference UE ID, SL ranging measurement/result)

4. Ranging request (SL ranging requirement, Reference UE ID, Target UE ID)

8. SL Ranging calculation

9. Ranging response (SL ranging result)

1. UE Triggered Service Request

3. LMF selection

Figure 6.x.3.2.3-1: Service exposure to the 3rd UE via 5GC/Uu

1. The application layer of the 3rd UE triggers the ranging/sidelink positioning, and then requests the network to perform the ranging/sidelink positioning between the Target UE and Reference UE. If the 3rd UE in CM-IDLE state, it firstly triggers the service request procedure.

2. The 3rd UE requests the network to perform the ranging/sidelink positioning between the Target UE and Reference UE. The SL ranging requirement may include the requirement regarding ranging/sidelink positioning demand (i.e. the distance and/or the direction between two UEs or more UEs), delay, accuracy, result feedback mode (e.g. immediately, periodically or at a later time).

3. The AMF selects an appropriated LMF, details see TS 23.273 [xx].

4. If the 3rd UE is authorized to request the ranging/sidelink positioning information between the Target UE and Reference UE based on the subscription of 3rd UE, the AMF requests the LMF for the ranging/sidelink positioning between the Target UE and Reference UE.

5. The LMF selects and triggers the UE (e.g. Reference UE) to perform ranging/sidelink positioning with the Target UE. The measurement request message may include the ranging/sidelink positioning demand, Target UE ID, Reference UE ID, etc.

Editor's Note: It is FFS that why and how the LMF selects the Reference UE (or Target UE) to perform ranging/sidelink positioning.

6. The Reference UE performs the ranging/sidelink positioning with the Target UE. The NG-RAN node is involved if UE is in network coverage and the operator managed radio resource is used.

7. The Reference UE reports the ranging/sidelink positioning measurements to the LMF for calculation. When the Ranging/SL positioning calculation is performed by UE, the ranging/sidelink positioning result is returned.

8. The LMF makes calculation based on the ranging/sidelink positioning measurements.

9. The LMF returns the ranging/sidelink positioning result to the AMF.

10. The AMF returns the ranging/sidelink positioning result to the 3rd UE via a NAS message.

6.x.4	Impacts on services, entities, and interfaces

UE:

-	sends SL ranging requirement to another UE for SL ranging measurement or to the LMF for SL ranging result.

-	receives SL ranging measurement from another UE or SL ranging result from the LMF (via the AMF).

-	receives SL measurement request from the LMF, performs SL measurement, and reports the SL measurement to the LMF.

NG-RAN:

-	allocates PC5 radio resource to UE for SL ranging.

AMF:

-	selects LMF and routes the SL ranging requirement to the LMF.

-	routes the signalling (e.g. LPP messages for SL ranging) between the UE and the LMF.

LMF:

-	requests and controls the UE to perform SL ranging measurement.

-	receives the SL ranging measurement from the UE and makes calculation.

-	returns the SL ranging result to the UE via AMF.

# * * * * End of changes * * * *