3GPP SA WG2 Meeting #150E	S2-2202349R01

Elbonia, April 06 - 12, 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	New solution on Key issue for WT#2 for Handling Rejected S-NSSAIs in some TAs of RA

Document for:	Discussion/Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3

Abstract of the contribution: This contribution proposes a solution for WT#2.

# 1.	Proposal

It is proposed to accept the following changes to TR 23.700-41

FIRST CHANGE

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | WT#2 |
| X |  |  | x | x |

NEXT CHANGE

## 6.X	Solution #X: Enabling UEs to Request S-NSSAIs not uniformly available

### 6.X.1	Introduction

Editor's note:	This clause lists the key issue(s) addressed by this solution.

This solution applies to KI#XX for WT#2 and KI#3.

### 6.X.2	Functional Description

Editor's note:	This clause outlines solution principles, assumptions and high-level architectures, etc.

The high-level points of the proposed solution are:

-	The AMF creates an RA for the UE as per current mechanisms ensuring the optimal size of the RA with respect to the S-NSSAIs that can be allowed

-	The Allowed NSSAI contains only S-NSSAIs that are allowed in the whole RA

-	The AMF provides a set of conditions to the UE for the S-NSSAIs that valid for the selected RA only under specific condition and therefore cannot be placed in the Allowed NSSAI (here called Conditional S-NSSAI)

-	the conditions may include a list of TAs (not included in the present RA) where the S-NSSAI is allowed

-	the conditions may include an Area of Service (as defined in KI#3) for the S-NSSAI

-	the conditions may include “time-related conditions” (e.g. specific times at which an S-NSSAI is available or the time period in which the S-NSSAI is available) to support the use cases associated with KI#3

-	multiple conditions may be associated to an S-NSSAI (e.g. list of TAs and “time-related conditions”)

NOTE 1: an S-NSSAI that is allowed in the whole RA or part of the RA but only under specific time conditions would be placed in the Conditional S-NSSAI.

-	the conditions may be provided by placing the S-NSSAI in an enhanced existing Rejected NSSAI IE (i.e. the S-NSSAI in the Rejected NSSAI may be associated with conditions)

-	alternatively, the conditions are provided by placing the S-NSSAI in a new Conditionally Allowed NSSAI IE which contains all the S-NSSAIs the UE requested and that cannot be placed in the Allowed NSSAI, but that may be available under different conditions (other TAs, other times, etc.)

NOTE 2: the decision with respect to which IE is used to convey the additional information should be taken in CT1.

-	The AMF provides the conditions to the UE anytime the AMF provides, or updates, the Allowed NSSAI and Rejected NSSAI to the UE (e.g. registration procedure, UCU procedure)

Editor’s Note: how the AMF is provided such condition information, e.g. via OAM or other mechanisms, is FFS.

-	The UE shall not request resources for a Conditional S-NSSAI unless the associated conditions indicate the S-NSSAI is available (e.g. specific TA or time)

-	When the UE determines that a Conditional S-NSSAI is available and that the applications in the UE require connectivity with the Conditional S-NSSAI, the UE may re-register with the network including the Conditional S-NSSAI.

-	in IDLE mode, when the UE performs a Service Request for an S-NSSAI that is no longer available (e.g. the UE has moved outside the TA where the S-NSSAI is available or the time conditions have changed), the request for the resources associated to the S-NSSAI is rejected and an indication is provided to the UE as to why. If the S-NSSAI was in the Allowed NSSAI, the AMF may provide a new Allowed NSSAI, and either a new Rejected NSSAI or Conditionally Allowed NSSAI to the UE.

-	Not all AMFs in a network may support this extension, thus to enable compatibility a UE receiving the additional information shall ignore it, and no capability indication to the AMF is required.

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause describes impacts to existing services, entities and interfaces.

The following impacts have been identified:

- UE:

-	interpreting and processing new information in Rejected NSSAI or new Conditional S-NSSAI IE

- AMF:

-	providing new information in Rejected NSSAI or new Conditional S-NSSAI IE

-	providing rejection cause at Service Request

END OF CHANGES