3GPP TSG-WG SA2 Meeting #150E e-meeting  	S2-2202357

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

# Title:	[DRAFT] LS on response to LS on parameters preconfigured in the UE to receive MBS service

# Response to:	LS (CP-220398) on parameters preconfigured in the UE to receive MBS service from CT

# Release:	Release 17

# Work Item:	5MBS

Source:	[Huawei to be] SA2

To:	CT

Cc:	CT1, CT4, SA4, RAN2

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	Raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

SA2 would like to thank CT for LS on parameters preconfigured in the UE to receive MBS service.

SA2 would like to inform CT that the purpose of pre-configuration is to enable UE to receive the data of MBS service. In TS 23.247 subclause 7.3.1 and 4.2.2, the usages are highlighted along the same line:

subclause 7.3.1

To receive the data of broadcast communication service, the UE is either preconfigured with needed configuration (e.g. USD as defined in TS 26.346 [13]) for the UE to receive MBS service, or provisioned with the configuration of broadcast session on application level (service announcement; the configuration may for instance be performed using SIP signalling, or methods described in TS 26.346 [13]). If the needed configuration is pre-configured, the UE does not need to interact with network.

subclause 4.2.2:

Service announcement: Service announcement is used to distribute information towards UEs about the service required for service reception (e.g., IP multicast address(es)) and possibly other service related parameters (e.g. service start time). This step can occur in parallel or after the MBS session configuration. However, TMGI allocation is required before. The information of the service announcement could be pre-configured at the UE side, see clause 7.3.1.

Subclause 6.11:

The Service Announcement includes the MBS Session ID(s), which is represented by TMGI or a Source Specific IP Multicast Address, for the service. When the MBS Session ID is Source Specific IP Multicast Address, the Service Announcement may include the PLMN ID of the PLMN and NID for an SNPN in which the service is delivered.

…

NOTE 3:	The MBS Service related information, e.g. default PLMN ID, DNN and S-NSSAI can be pre-configured in the UE.

Regarding the questions from CT:

Question 1: Which parameters can be preconfigured in the UE to receive MBS service? Question 2: How are these parameters used by the UE?

SA2 answer:

SA2 defined requirements to allow a preconfiguration of PLMN ID, DNN and S-NSSAI in the UE. This enables a UE to receive a multicast MBS session if such information is not included in the service announcement.

SA2 also assumes that the USD defined in TS 26.346 [13] can be used to preconfigure information for broadcast communication services. Again, this enables a UE to receive a broadcast MBS session if such information is not included in the service announcement.

SA2 considers that possible additional parameters and UE behaviours should be under the remit of CT WGs.

Some potential additional preconfigured parameters could be:

- PLMN ID, TMGI(s) and associated USD(s), and RAN information (e.g., frequencies).

UE could use uses the pre-configured information for the purpose of receiving the MBS data of a certain TMGI by using the associated USD.

2. Actions:

To CT group.

ACTION: 	SA2 asks CT group to take above answers into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #151E   	May 16 – 20, 2022 				Elbonia