3GPP SA WG2 Meeting #150E	S2-2202379R02

Elbonia, April 06 - 12, 2022	(was S2-220xxxx)

Source:	Qualcomm Incorporated

Title:	New solution for KI#2 and KI#3: U2X for support of Broadcast Remote ID and direct DAA via PC5

Document for:	Discussion/Approval

Agenda Item:	9.5

Work Item / Release:	FS_eNS_Ph3

Abstract of the contribution: This contribution proposes a solution for KI#2.

# 1.	Proposal

It is proposed to accept the following changes to TR 23.700-58.

FIRST CHANGE

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP".

[5]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking".

[6]	DEPARTMENT OF TRANSPORTATION Federal Aviation Administration 14 CFR Parts 1, 11, 47, 48, 89, 91, and 107 [Docket No.: FAA-2019-1100; Amdt. Nos. 1-75, 11-63, 47-31, 48-3, 89-1, 91-361, and 107-7] RIN 2120-AL31 Remote Identification of Unmanned Aircraft.

[7]	EASA NPA 2021-14: "Notice of Proposed Amendment 2021-14".

[8]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[9]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[10]	3GPP TR 23.754: "Study on supporting Unmanned Aerial Systems (UAS) connectivity, Identification and tracking".

[X]	ASTM F3411.19 “Standard Specification for Remote ID and Tracking”

[Y]	ASD-STAN prEN 4709-002:2022-03, Aerospace series - Unmanned Aircraft Systems - Part 002: Direct Remote Identification

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 23.256 [5] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] or TS 23.256 [5].

U2X communication: A communication to support UAV-to-Everything (U2X) services leveraging Uu and / or PC5 reference points. U2X services are realized by various types of U2X applications, i.e. UAV-to-UAV (U2U) and UAV-to-Network (U2N).

U2X message: A dedicated messaging type of U2X service, for example Broadcast Remote ID messages.

U2X service type: A type of U2X service, which is identified by e.g. ITS-AID (ITS Application Identifier), PSID (Provider Service Identifier) or AID (Application Identifier).

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BRID	Broadcast Remote Identification

C2	Command and Control

DAA	Detect And Avoid

RID	Remote Identification

TPAE	Third Party Authorized Entity

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

U2X	UAV to everything

U2XP	U2X Policy

NEXT CHANGE

# 6	Solutions

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |  |  |  |
| X |  | x | x |  |  |  |  |
| 2 |  |  |  |  |  |  |  |

NEXT CHANGE

## 6.X	Solution #X: U2X for support of Broadcast Remote ID and direct DAA via PC5

### 6.X.1	Introduction

This solution applies to KI#2 and KI#3.

### 6.X.2	Functional Description

The high-level points of the proposed UAV-to-everything (U2X) solution are:

-	U2X leverages V2X mechanisms as defined in TS 23.287 [9] to support BRID and direct DAA, with the differences described in this solution

-	both LTE PC5 and NR PC5 are supported and the RAT selection is based on the U2XP.

-	Communications modes:

-	Broadcast communication mode is used for BRID

-	Broadcast communication mode is used for DAA to advertise UAV information, and unicast over PC5 may be used between two UAVs for DAA deconfliction

Editor’s Note: unicast over Uu via the U2X AS is FFS due to lack of requirements.

-	When NR PC5 is selected, connection-less groupcast communications may be used for DAA. Application layer managed groupcast are not considered in this release due to lack of clear requirements

-	U2X is supported by an U2X Application Server which interfaces with the operator network via NEF, as in the case of the V2X Application Server

Editor’s Note: whether the same set of services is required or not is FFS.

NOTE 1: multiple deployment scenarios need to be allowed where the U2X AS and the USS serving a UAV are the same or different entities.

-	a U2X Policy (U2XP) is defined to provide configuration parameters to the UE for U2X communication over the PC5 reference point or over the Uu reference point

-	the configuration parameters may be pre-configured in the ME, or configured in the UICC, or preconfigured in the ME and configured in the UICC, or provided/updated by the U2X Application Server via PCF and/or V1 reference point, or provided/updated by the PCF to the UE.

-	the UE shall consider them in the following priority order: provided/updated by the PCF, provided/updated by the U2X Application Server via V1 reference point, configured in the UICC, pre-configured in the ME.

NOTE 2:	a scenario where the U2X Application Server provides the U2XP to the UAVC via means of communication outside the scope of 3GPP (e.g. Internet connection) over U2X1, and the UAVC provides the U2XP to the UAV is also considered. In such case, the UE considers the parameters as being pre-configured in the ME.

-	Similar to V2X, Tx Profiles or NR Tx Profiles are determined based on U2XP mapping ofU2X service types.

-	both UAVs with UICC and UAVs without UICC (i.e. with no subscription to an MNO) are supported.

-	UAVs with no UICC can only perform U2X communications when authorized for “not served by E-UTRA” and “not served by NR”

-	U2X communications parameters from U2X Application Server or PCF may be delivered via UAV-C UE

-	The transmission method between the UAV and UAV-C UE is out of the scope of the specification

-	in addition to existing parameters for V2X, the radio parameters per PC5 RAT (i.e. LTE PC5, NR PC5) can be configured with Geographical Area, Altitude Limitation, and Validity timer

-	this additional information may be needed to enable policing the use of PC5 depending on the specific location of the UAV

-	the definition of DAA/UAV service type is out of scope of 3GPP

-	the use of PC5-based communications for BRID and DAA for UAV with UICC is subjected to successful UUAA authentication/authorization of the UAV as defined in TS 23.256 [5] and authorization via U2XP. However, no specific authorization of the use of PC5 for either BRID or DAA is required by the USS. For UAVs without UICC, the use of PC5-based communications for BRID and DAA is authorized only by U2XP.

Editor’s Note: whether U2X services will be identified by more than one, and which ones, of e.g. ITS-AID (ITS Application Identifier), PSID (Provider Service Identifier) or AID (Application Identifier) is FFS.

The functional architecture is as follows:

Figure 6.X.2-1: Non-roaming 5G System architecture for U2X communication over PC5 and Uu reference points.

The reference points of TS 23.287 [9] apply, with the following differences:

U2X1:	The reference point between the U2X applications in the UE and in the U2X Application Server. This reference point is out of scope of this specification.

U2X5:	The reference point between the U2X applications in the UEs. This reference point is not specified in this release of the specification.

N1:	In addition to the relevant functions defined in TS 23.501 [2] for N1, in the case of U2X Service it is also used to convey the U2X policy and parameters (including service authorization) from AMF to UE and to convey the UE's U2X Capability and PC5 Capability for U2X information from UE to AMF.

N2:	In addition to the relevant functions defined in TS 23.501 [2] for N2, in the case of U2X Service it is also used to convey the U2X policy and parameters (including service authorization) from AMF to NG-RAN.

Use or U2X for BRID:

-	the content of the messages for BRID are defined according to the regional regulations for BRID (e.g. message set of ASTM F3411.19 [X] or ASD-STAN prEN 4709-002 P1 [Y]) and optionally according to regional mean of compliance documents.

Editor’s Note: references to means of compliance will be listed as example when available.

Use of U2X for DAA:

-	the content of the messages for DAA are defined according to the regional regulations for DAA and is out of scope of this specification

### 6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.X.4	Impacts on services, entities and interfaces

The following impacts have been identified:

- UE: In addition to the functions defined in TS 23.501 [2], the UE may support the following functions

-	Report the U2X Capability and PC5 Capability for U2X to 5GC over N1 reference point.

-	Indicate U2X Policy Provisioning Request in UE Policy Container for UE triggered U2X Policy provisioning.

-	Receive the U2X parameters from 5GC over N1 reference point.

-	Procedures for U2X communication over PC5 reference point.

-	Configuration of parameters for U2X communication. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over U2X1 reference point from the U2X Application Server.

- AMF: In addition to the functions defined in TS 23.501 [2], the AMF performs the following functions:

-	Obtain from UDM the subscription information related to U2X and store them as part of the UE context data.

-	Select a PCF supporting U2X Policy/Parameter provisioning and report the PC5 Capability for U2X to the selected PCF.

-	Obtain from PCF the PC5 QoS information related to U2X and store it as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about U2X communication over PC5 reference point.

-	Provision the NG-RAN with PC5 QoS parameters related to U2X communication.

-	PCF: In addition to the functions defined in TS 23.501 [2], the PCF includes the functions described in 23.287 [9] to provision the UE and AMF with necessary parameters in order to use U2X communication

-	UDM: Subscription management for U2X communication over PC5 reference point. The UE subscription data types are extended according to the following table:

| U2X Subscription data | NR U2X Services Authorization | Indicates whether the UE is authorized to use the NR sidelink for U2X services as UAV UE, UAV-C UE, or Authority UE. |
| --- | --- | --- |
|  | LTE U2X Services Authorization | Indicates whether the UE is authorized to use the LTE sidelink for U2X services as UAV UE, UAV-C UE, or Authority UE. |
|  | NR UE-PC5-AMBR | AMBR of UE's NR sidelink (i.e. PC5) communication for U2X services. |
|  | LTE UE-PC5-AMBR | AMBR of UE's LTE sidelink (i.e. PC5) communication for U2X services. |

- U2X Application Server: implements a subset of the V2X AS functionality specified in TS 23.287 [9]:

-	includes AF functionality, and may support at least the following capabilities:

Editor’s Notes: whether any uplink data from the UE or downlink data to the UE is required for U2X service handling is FFS.

-	For U2X service parameters provisioning, the U2X AS provides the 5GC and the UAV UE (possibly via the UAVC) with parameters for U2X communications over PC5 and Uu reference points.

-	UDR: In addition to the functions defined in TS 23.501 [2], the UDR stores U2X service parameters.

-	NRF: In addition to the functions defined in TS 23.501 [2], the NRF performs PCF discovery by considering U2X capability.

-	NEF: for U2X AS, the NEF supports U2X service parameters.

END OF CHANGES