SA WG2 Meeting #150E e-meeting	S2-2202383

Elbonia, 06 – 12 April, 2022	(revision of S2-22xxxxx)

Source:	vivo

Title:	FS_eNA_Ph3, KI#1, New solution for improving NWDAF correctness

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract: This contribution proposes a solution to address how to improve the correctness of the NWDAF analytic.

1.	Introduction

In AI community, there is a concept called ‘accuracy’ which is used to indicate the correctness of an AI/ML model’s predictions. The Accuracy is the percentage of correct predictions in all predictions.

This document provides a solution to address FS_eNA_Ph3 KI#1: how to improve the correctness of the NWDAF analytic with AI/ML model accuracy.

This contribution focuses on the solution to allow the NWDAF (AnLF) collect the label data and calculate AiU (accuracy in use).

2.	Proposal

It is proposed to adopt the following solution into 23.700-81.

# * * * * ALL NEW TEXT * * * *

6.X	Solution #X: Accuracy based NWDAF Analytics Correctness Improvement.

6.X.1	Description

This solution is about KI #1: How to improve correctness of NWDAF analytics to address how to improve correctness of NWDAF analytics.

In AI community, there is a concept called ‘accuracy’ which is used to indicate the correctness of an AI/ML model’s predictions. The Accuracy is the percentage of correct predictions in all predictions. One of methods to determines a prediction is correct is to compare the prediction with the label (data). A set of data can be composed as (x, t), and the ‘x’ is ‘input data’ and ‘t’ is ‘label’ or ‘label data’. If this label is correct, it also is called ‘ground truth’.

For the regression problems, the predictions and label data may be not same but similar, and their difference can be calculated by other methods e.g. Mean Absolute Error (MAE). If this difference is smaller than a threshold, this prediction should be considered as correct.

Figure 1: Accuracy in AI Community

As shown in Figure 2, Accuracy in Training (AiT) and Accuracy in Use (AiU) can be used to measure the correctness of a ML model, the Accuracy in Use should be observed/monitored by the NWDAF/network, which could be a trigger for NWDAF(MTLF) to re-train the ML Model if Accuracy in Use cross a threshold:

Figure 2: Accuracy in Training with Training dataset vs Accuracy in Use with live network dataset

- As shown in left side of the Figure, a NWDAF(MTLF) trained a ML Model with Training dataset (input data, label data) and the Accuracy in Training is to indicate the performance of ML model in training stage by comparing prediction with label data in validation dataset reserved from training dataset.

- The NWDAF(MTLF) deliver the trained ML Model to a NWDAF(AnLF), which will be used by the NWDAF(AnLF) to perform inference with input data from live network.

- As shown in right side of the Figure, Accuracy in Use is to indicate the performance of ML model used in live network by comparing prediction with the observed label data i.e. ground truth from the live network.

Please note that the NWDAF(AnLF) performs inference with live network input data to get the prediction and also retrieves the label data from live network.

- However, due to some reasons, e.g. different data distribution between training data set and data in live network, poor model generalization ability, etc., there may be a gap between Accuracy in Training and Accuracy in Use, and the Accuracy in Use usually is not as good as Accuracy in Training especially as times goes by.

The NWDAF(AnLF) decides to indicate the NWDAF(MTLF) to re-train the AI/ML model e.g. if Accuracy in Use crosses a threshold or the gap between Accuracy in Training and Accuracy in Use crosses a threshold.

In order to improve training data quality e.g. the data distribution, the NWDAF(AnLF) also provides the input data, labelled data, where prediction is regarded as incorrect prediction by comparing with labelled data i.e. ground truth , to help the NWDAF(MTLF) to re-train a better AI/ML Model e.g. with model generalization ability.

Editor’s Note: how the NWDAF(AnLF) determines the prediction is regarded as a incorrect prediction by comparing with the ground truth is FFS.

Similarly, regarding regression, MAE in training and MAE in use is introduced to measure the ML model correctness.

6.X.2	Procedures

Figure 6.x.2-1 depicts the procedure for the proposed solution that the AnLF collects the label data and calculates the AiU.

Figure 6.X.2-1 AnLF collect label data and calculate accuracy in use

Step 1, The NWDAF(AnLF) service consumer subscribes to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3, TS 23.288.

Step 2, The NWDAF(AnLF) subscribes to a NWDAF(MTLF) a trained ML Model by invoking the Nnwdaf_MLModelProvision (an Analytics ID) service operation.

Step 3 and step 4, the MTLF trains the model according to the requirement from AnLF and notifies the AnLF with the ML model information by invoking Nnwdaf_MLModelProvision_Notify (analytic ID, the file address of the trained ML model, Accuracy in Training (AiT) or MAE in Training) service operation.

Step 5~8, AnLF collects the input data from input data providers, e.g. OAM, NFs, makes predictions and sends the predictions to the NWDAF consumer NF.

Step 9 ~ 10, the AnLF collects label data corresponding to predictions from data providers e.g. AF and SMF, and calculates the Accuracy in Use or the MAE in Use, which reflects the performance of ML model used in live network by comparing prediction with the observed labelled data i.e. ground truth from the live network.

Step 11, the AnLF compares the Accuracy in Use with the Accuracy in Training and  or MAE in Use with MAE in Training and decide whether or not to send notification to MTLF to re-train the AI/ML model e.g. if Accuracy in Use cross a threshold or by comparing the between Accuracy in Training and Accuracy in Use.

Step 12 and 13,

The AnLF reuse MLModelProvision service(subscription Correlation ID, analytic ID, AiU or MAE in Use, input data and label data) to modify of the original ML model subscription in step 2, indicating to the MTLF that the Accuracy in Use or MAE in Use is not good as the Accuracy in Training or MAE in Training and the MTLF retrains the model with new data from the AnLF.

Alternatively, the AnLF sends a notification to the MTLF indicating that the Accuracy in Use or MAE in Use is not good as the Accuracy in Training or MAE in Training using a new service operation Nnwdaf_MLModelPerformance_notify (Notification Correlation ID, analytic ID, AiU or MAE in Use, input data and label data), if an Nnwdaf_MLModelPerformance_Subscribe (Notification Target Address (+ Notification Correlation ID), analytic ID, [Reporting Threshold]) is received from the MTLF after step 4. And the MTLF retrains the model with new data from the AnLF.

In order to improve training data quality e.g. the data distribution, the AnLF provides the input data, label data i.e ground truth, where prediction is regarded as incorrect prediction by comparing with labelled data i.e. ground truth , to help the MTLF to re-train a better AI/ML Model e.g. with model generalization ability.

Editor’s Note: how the NWDAF(AnLF) determines the prediction is regarded as a incorrect prediction by comparing with the ground truth is FFS.

Step 14~15, the AnLF notifies the consumer that the Accuracy in Use is not as good as the Accuracy in Training to consequently stop using the provided analytic result by invoking the Nnwdaf_AnalyticInfo_Notify (analytic ID, AiU, Disable notification).

6.X.3	Impacts on services, entities and interfaces

The solution has the following impacts:

NWDAF(AnLF):

- Collects data from corresponding data provider(s) to calculate the Accuracy in Use.

- Sends the notifications to the NWDAF(MTLF) and consumer to inform the Accuracy in Use or MAE in Use has decreased.

NWDAF(MTLF):

- Receives the notification that the Accuracy in Use or MAE in Use has decreased from NWDAF(AnLF) and re-trains the AI/ML Model.

NWDAF service Consumer:

•	Receives the notification that the Accuracy in Use or MAE in Use has decreased from NWDAF(AnLF) and stops using the analytics provided by the NWDAF(AnLF).

# * * * * End of change * * * *