3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202403r07

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update on KI#2: Add Use Cases, Scenarios and Assumptions

Document for:	Approval

Agenda Item:	9.11

Work Item / Release:	FS_EDGE_Ph2 / Rel-18

Abstract: Update on KI#2 to add Use cases, scenarios and assumptions.

# 1. Introduction

In TR 23.700-48, clause 5.2.2 is for use cases and scenarios applicable to KI#2 and clause 5.2.3 is for assumptions.

There are also two Editor’s notes as following:

Editor's note:	This clause will document the use cases and scenarios applicable to KI#2.

Editor's note:	This clause will document assumptions applicable to KI#2, if any. This clause will be removed if left empty.

Based on this, use cases, scenarios and assumptions are proposed.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-48.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[4]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)".

[5]	GSMA OPG.02: "Operator Platform Telco Edge Requirements", https://www.gsma.com/futurenetworks/wp-content/uploads/2021/07/GSMA-OPG-Telco-Edge-Requirements-2021.pdf.

[6]	SP-210583: "Reply LS to GSMA Operator Platform Group on edge computing definition and integration", SA#92e.

[x]	IETF RFC 5681: "TCP Congestion Control".

[y]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

# * * * * Second change * * * *

### 5.2.2	Use cases and scenarios

Observation of the link characteristics is widely used to adjust the transmission behaviour in different scenarios. For example,

-	For some transport layer protocols, e.g. TCP, QUIC, congestion control algorithms can be used to control packets transmission via 5GS based on e.g. observing packet loss and take it as indication of network congestion. For example, TCP congestion control algorithm defined in RFC 5681 [x] uses different ways to adjust congestion window, perform slow start, congestion avoidance, fast retransmit, and fast recovery, which lead to classic “sawtooth” congestion window.

-

-	For video based services, rate adaption can be achieved based on network condition exposed by 5GS.

For Edge computing scenarios, most applications are sensitive to the change of network latency and throughput. When network congestion happens, if the application can be notified in a fast and efficient way (e.g. within a RTT of user packet transmission), these services can react to the change of network congestion status more agilely.

Editor’s Note: Further updates to the Use Case is FFS.

### 5.2.3	Assumptions

Editor's note: This clause will document assumptions applicable to KI#2, if any. This clause will be removed if left empty.

# * * * * End of changes * * * *