3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202477

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	New Solution: AF triggered MBS session management

Document for:	Approval

Agenda Item:	9.18

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: this paper proposes a new solution for AF triggered MBS session management.

1. Introduction

This solution addresses KI#3 “On demand multicast MBS session”.

Instead of UE demands 5GC to use multicast transport described in R17, this solution addresses the case that AF dynamically demands 5GC to use multicast transport for the content delivery, e.g., due to APP in UE interacts with AF for content fetching.

This solution does not address the on-demand cases that described in TR 26.849 (MooD), which is for resource efficiency that LTE RAN is on-demanded to use MBMS frequency for traffic delivery when lots of UE on the same site receive the same content. But NG-RAN has the ability to use both PTP and PTM for multicast data delivery, so whether MooD is needed or not for 5G needs FFS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 MBS session reception in RRC Inactive | 2 MOCN network sharing | 3 On demand multicast MBS session |  |
| 1 | X |  |  |  |
| 2 |  | X |  |  |
| Y |  |  | X |  |

# * * * * Second change (NEW TEXT) * * * *

## 6.Y	Solution #Y: AF triggered MBS session management

### 6.Y.1	Introduction

This solution addresses Key Issue #3.

This solution addresses the case that AF dynamically demands 5GC to use multicast transport for the content delivery, e.g., due to publisher dynamically provides the service and APP in UE interacts with AF for content fetching of the service, and according to the response from 5GC to select unicast mode for content delivery, e.g., UE does not support multicast transport (out of the scope of this study). The service may contain multiple media streams but only part of the media streams is demanded to use multicast transport.

### 6.Y.2	Functional description

It is most popular today that content provider provides video to users via APP in UE, but in unicast transport style. Live stream dramatically grows today, which also uses unicast transport. The video services provided by AF can not only be published by the content provider, but also can be published by users of the content provider dynamically. The consumer will not only be a receiver in today’s video stream services, but also a participant to interact with the services, such as sending message to interact with the live stream salesperson, sending message when watching a video to share with all the viewers.

In most cases, the users visit the portal for requesting the content, and the AF holds most business logic for providing the services, e.g., the location related operations, the user authentication and authorization per the AF instead of the service, etc. And the service, in most cases, will contain multiple media streams that only part of the steams is demanded to be delivered via multicast transport. This business model is consistent for services provided by AF.

In order to use multicast transport for multicast streams of those services, the 5GC needs to identify the multicast data and uses multicast transport for the delivery and do not impact the AF service logic. And in order to not limiting the consumers based on the UE capability, the 5GC and AF needs to prepare unicast mode in case the UE does not support receiving data with multicast transport (this is out of the scope of this study).

Release 17 5MBS uses the model that most service logic is done by 5GC although provisioned by AF (e.g., area restriction per user or per MBS Session, authorization logic no matter whether it can map to R17 5MBS authorization method or not, etc.), which means the AF shall hand over lots of logic to operator. This solution reuses the current unicast business model to provide an alternative solution for content providers.

### 6.Y.3	Procedures

#### 6.Y.3.1	General

Following figure 6.Y.3.1-1 shows the general concept of the solution:

Figure 6.Y.3.1-1: General concept

The AF delivers the multicast data of different multicast sessions to the 5GC, which may be in a tunnel between the AF and the 5GC if the transport network between the AF and 5GC does not support multicast. The AF can send multiple MBS Session data in one tunnel, the 5GC distinguishes the multicast session based on the mapping between the packet filters and the MBS Session ID. The content delivery is requested by the UEs over application layer, and the AF handles the service logic.

The content provider may use a portal to publish the services, which may be dynamically published, e.g., a live stream arranged by a live streamer, a live interview for breaking news, etc. The user may visit the portal via an APP in the UE and request the content of an MBS service, which may be interactive MBS service, i.e., contains multicast service data and unicast service data. Although service announcement for MBS Session is supported, but currently most content providers still want to use web portal, which is consistent as unicast service announcement.

The AF of the content provider sends the flow descriptions per UE, which includes different multicast services, to the 5GC for asking transport for the services just like currently used for unicast services, but the flow descriptions includes multicast information.

#### 6.Y.3.2	MBS Session Configuration procedure

This is an optional procedure, and the purpose to for negotiate the information for establishing the tunnel for multicast data delivery for the AF. The Ingress address of MB-UPF/MBSTF may be preconfigured for an AF or multiple AFs, in this case the procedure is not needed to be performed. This procedure does not create MBS Session, clause 7.1.1.2 and 7.1.1.3 of TS 23.247 [4] can be used for the MBS Session Creation. Considering the multicast content may be dynamically published, the AF wants to have on-demand multicast service delivery, so this procedure only configures the transport for all multicast service data.

The differences between this procedure and the MBS Session Creation procedures in clause 7.1.1.2 and 7.1.1.3 of TS 23.247 [4] are as following:

-	The request from AF does not include MBS Session ID, Service type, QoS requirement, which are related to an MBS Session.

-	The PCC is not involved if deployed.

-	No TMGI allocation procedure in prior.

-	The MB-SMF is discovered and registers per AF.

Figure 6.Y.3.2-1: MBS Session Configuration management procedures

1.	For creating the configuration for MBS services, the AF sends MBS Session Configuration Create Request (AF ID, [Area Info]) towards the NEF/MBSF. For updating the configuration for MBS services, the AF sends MBS Session Configuration Update Request (AF ID, Area Info) towards the NEF/MBSF. For deleting the configuration for MBS services, the AF sends MBS Session Configuration Delete Request (AF ID) towards the NEF/MBSF.

If the AF only provides services in a limited area, the Area Info may be included, which may be a geographic area or civic address information.

2.	The NEF/MBSF authorizes the AF.

3.	If authorization succeeds, the NEF/MBSF first discovers the MB-SMF from the NRF by providing the AF ID, The NRF selects the MB-SMFs configured for the AF and responds with the information of the MB-SMFs. For creation, if no MB-SMF is found, the NEF/MBSF then discovers the MB-SMF by the Area Info, the Area Info is received from step 1, or is locally configured for the AF. For update, if the returned MB-SMFs is not able to cover the Area Info, the NEF/MBSF then discovers the MB-SMF by an appropriate Area Info. The NRF selects the appropriate MB-SMFs according to the Area Info and responds with the information of the MB-SMFs.

4.	For creation and update, the NEF/MBSF selects the MB-SMFs, according to the Area Info if needed, and invokes MBS Session Configuration Create Request (AF ID, [Area Info]) or MBS Session Configuration Update Request (AF ID, Area Info) towards the MB-SMFs. For deletion, the NEF/MBSF invokes MBS Session Configuration Delete Request (AF ID) towards the MB-SMFs.

5.	For creation and update, if the MB-SMF has not registered AF ID to the NRF, the MB-SMF registers the AF ID to the NRF. For deletion, the MB-SMF deregisters the AF ID from the NRF.

6-7. For creation, the MB-SMF selects MB-UPF, according to the received Area Info if received, and may interact with the MB-UPF to create an Ingress address for the AF. For update, the MB-SMF selects MB-UPF that does not serve the AF and may interact with the MB-UPF to create an Ingress address of the AF. For deletion, the MB-SMF may interact with the MB-UPF that serves the AF to release the connection resources.

8.	The MB-SMF responds to the NEF/MBSF, for creation and update, the MB-SMF sends the Ingress address and corresponding Area Info to the NEF.

9-10. For creation and update, if MBSF decides to involve MBSTF, the MBSF interacts with the MBSTF with the received Ingress address, and MBSTF returns Ingress address of the MBSTF. For deletion, if MBSTF is involved in creation, the MBSF interacts with the MBSTF to release the connection resources.

11.	The NEF/MBSF responds to the AF. For creation and update, the NEF/MBSF sends the Ingress address and optional corresponding Area Info to the AF. The Area Info is used by the AF to determine which Ingress address is used to deliver the MBS data based on UE location.

#### 6.Y.3.3	AF triggered MBS Session management procedures with PCC

This procedure is for AF triggered MBS Session Join or Leave when dynamic PCC is deployed.

Figure 6.Y.3.3-1: AF triggered MBS Session management procedures with PCC

1.	The UE established a PDU Session as described in clause 7.2.1.2 of TS 23.247 [4], during the PDU Session Establishment procedure, the UE indicates the MBS capability to SMF. The content provider may use a portal to publish the services, which may be dynamically published, e.g., a live stream arranged by a live streamer, a live interview for breaking news, etc. The user may visit the portal via an APP in the UE and request the content of an MBS service or stops the content of an MBS service, which may be interactive MBS service, i.e., contains multicast service data and unicast service data. The APP sends UE address and the UDP port for receiving the multicast data to the AF.

The AF may perform procedure as described in clause 6.Y.3.2 for creating or removing the tunnel between the AF and some MB-UPFs.

NOTE 1:	In case UE has multiple PDU Sessions, the URSP in the UE needs to make sure that the APP uses the associated PDU Session for unicast traffic delivery.

2-5. The steps are same as described in TS 23.502 [3] clause 4.15.6.6 and 4.15.6.6a with following differences:

-	The name of service operations exposed by NEF is different, and the flow description(s), if included, includes some packet filters that additionally contains IP multicast address to indicate that the data of the unicast flow can be alternatively sent via multicast (in case unicast transport is used, the network can know that it is for a multicast service).

-	The flow description(s) sent to PCF includes some packet filters that additionally contains IP multicast address.

-	If the MBS Session has not been created, the steps 10-17 in clause 7.1.1.2 or steps 10-26 in clause 7.1.1.3 of TS 23.247 [4] are performed to create the MBS Session.

If the AF is in trust domain, the AF can perform step 3, 5, and 16 directly.

6.	The PCF invokes Policy Update Notify towards the SMF, which includes unicast QoS information and flow description(s) that additionally contains IP multicast address (i.e., multicast flow descriptions).

7.	The SMF may initiate PDU Session Modification procedure for updating unicast QoS flows.

8.	MBS Session Join: For multicast flow creation, the steps 2-4 in clause 7.2.1.3 of TS 23.247 [4] are performed with following differences:

-	If the authorization fails, go to step 10 and SMF indicates cause value to the PCF. The AF can use unicast transport for the multicast data delivery.

MBS Session Leave: For multicast flow releasing, the steps 3-6 in clause 7.2.1.4 of TS 23.247 [4] are performed with following differences:

-	If the MB-SMF does not serve the MBS Session any more (i.e., all SMFs unsubscribed the MBS Session Context Status), steps 6-8 in clause 7.1.1.4 or 7-11 in clause 7.1.1.5 of TS 23.247 [4] are performed to delete the MBS Session.

9.	MBS Session Join: For multicast QoS information creation, steps 5-12 described in TS 23.247 [4] clause 7.2.1.3 are performed with following differences:

-	The step 5 is Namf_Communication_N1N2MessageTransfer. If the MBS capability of UE is received and multicast streams are demanded, the SMF includes QoS rules for DL only in the N1 Message Container in step 5, which will include MBS Session ID as a parameter in the QoS rules for DL only. The QoS rules for DL only can be used by the UE to transfer the IP multicast address in the received packets to the UE address, and to transfer the destination UDP port in the received packets to the UE UDP port.

-	If the MBS capability of UE is not received, the SMF does not include MBS Session related information in the N2 SM info send to NG-RAN (i.e., individual delivery is selected), as well as instructs the UPF to perform NATP as described in above bullet for the incoming multicast data of the MBS Session towards the UE (i.e., unicast mode is selected).

-	If unicast QoS parameters are received in step 6, the SMF also update the unicast QoS flows during the PDU Session Modification procedure.

Editor´s note; User consent for this operation and procedures if it is denied are ffs.

MBS Session Leave: For multicast QoS information removal, steps 3-11 and 13 described in TS 23.247 [4] clause 7.2.2.2 are performed with following differences:

-	The step 7 is Namf_Communication_N1N2MessageTransfer, and the SMF also update the unicast QoS flows during the PDU Session Modification procedure if unicast QoS flows need to be updated.

10.	The SMF notifies the PCF.

11.	The PCF Notifies the NEF with cause value related to MBS.

12.	The NEF notifies the AF with the cause value. If the PCF does not support MBS (i.e. no corresponding cause value indicated), the corresponding unicast QoS flow will be established, the AF can use unicast transport for the multicast data delivery.

#### 6.Y.3.4	AF triggered MBS Session management procedures without PCC

This procedure is for AF triggered MBS Session Join or Leave when dynamic PCC is not deployed.

Figure 6.Y.3.4-1: AF triggered MBS Session management procedures without PCC

0.	After the PDU Session Establishment procedure, the SMF create the SMF-NEF connection with the NEF similar as described in TS 23.502 [3] clause 4.25.2 and 5.2.6.15, during which the SMF sends the UE address, i.e., IP address, to the NEF. The difference is that the subscription of the UE includes the NEF ID for MBS, the NIDD information in the request to the NEF is changed to MBS information with same content, i.e., GPSI, AF ID, and additional UE address is included.

1.	Same as steps 1-2 in clause 6.Y.3.3.

2.	The NEF invokes the MBS On-demand Session Create/Update with AF ID and SSM towards the SMF.

3.	Same as steps 7-8 in clause 6.Y.3.3.

4-5. The SMF responds to the NEF. The NEF responds to the AF.

6.	Same as steps 9 in clause 6.Y.3.3.

7-8. The SMF notifies to the NEF. The NEF notifies to the AF.

### 6.Y.4	Impacts on services, entities and interfaces.

UE:

-	May support indicating MBS capability to SMF during PDU Session Establishment.

-	May support NATP performance for incoming multicast data.

NEF/MBSF:

-	Support handling MBS AF Session service to AF (similar as AF Session with QoS service).

-	May support additionally handling UE address during SMF-NEF connection establishment procedure as described in TS 23.502 [3] clause 4.25.2, which is used to find out the serving SMF.

-	May expose MBS 5GC-AF Connection service to AF, discovering MB-SMF from NRF by AF ID and Area Info, and invoke MBS AF Connection service towards MB-SMF.

SMF:

-	Support discovering MB-SMF from NRF by AF ID and/or Area Info.

-	Support receiving AF ID, SSM, and multicast flows information from PCF or NEF and sending them to MB-SMF for multicast flows handling.

-	Support determining delivery method and instructs UPF to perform NATP for multicast data based on UE MBS capability.

-	May support sending UE address during the SMF-NEF connection establishment procedure to NEF after associated PDU Session Establishment.

MB-SMF:

-	Support MBS Session Context Status subscribe with AF ID and SSM, and allocate/deallocate TMGI accordingly.

-	Support register to NRF with AF ID.

-	May support interacting with MB-PCF with multicast flows information.

-	May support handling MBS 5GC-AF Connection service to NEF.

NRF:

-	Support selecting MB-SMF and responding information of MB-SMF queried by AF ID or Area Info.

-	Support MB-SMF registration with AF ID.

PCF:

-	Support sending AF ID, SSM, and multicast flows information to SMF for multicast flows handling.

-	Support sending cause value for handling multicast flows to NEF/AF.

Editor's note:	Additional impacts are FFS.

# * * * * End of changes * * * *