3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202479

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	New Solution: Management PIN and PIN Elements

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes a new solution for management PIN and PIN Elements.

1. Introduction

This solution mainly addresses KI#3 “Management of PIN and PIN Elements” and KI#5 “Authorization for PIN”, i.e., authorization of PEMC management PIN and PEGC providing connectivity to and from 5G network.

This solution also address part of KI#6 “Policy and parameters provisioning for PIN”, i.e., the RAT parameters and Discovery parameters provisioning.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |  |
| Y |  |  | X |  | X | X |  |

# * * * * Third change (NEW TEXT) * * * *

## 6.Y	Solution #Y: Management of PIN and PIN Elements

### 6.Y.1	Description

This solution mainly addresses KI#3 “Management of PIN and PIN Elements” and KI#5 “Authorization for PIN”, i.e., authorization of PEMC management PIN and PEGC providing connectivity to and from 5G network. This solution also address part of KI#6 “Policy and parameters provisioning for PIN”, i.e., the RAT parameters and Discovery parameters provisioning.

The steps in the following call flows depicted as grey line in the figures, which are over application layer, are out of SA2 scope.

### 6.Y.2	Procedures

#### 6.Y.2.1	Management of PIN

Figure 6.Y.2.1-1: Management of PIN

1.	The PEMC interacts with the PIN Management Function (PINMF). The PEMC can discover the PINMF based on DNS query, or preconfigured information.

2.	The PINMF sends PIN Authorization Request (GPSI) to the NEF.

3.	The NEF queries whether the PEMC has subscribed PIN service as a manager for a group or not.

4.	The NEF responds to the PINMF based on the subscription, if the PEMC indicated by the GPSI has subscribed PIN service, then responds success.

#### 6.Y.2.2	Management of PEGC and PINE

Figure 6.Y.2.2-1: Management of PEGC and PINE

1.	The PEGC or PINE may establish direct connection, e.g., the PEGC/PINE powered on with WiFi hotspot, and PEMC scans the bar code on the PINE/PEGC to have the parameters for connecting to the PEGC/PINE.

2.	The PEMC may query device information from PEGC/PINE over application layer. The parameters for visiting the application in the PEGC/PINE may be documented via the bar code.

3.	The PEMC interacts with PINMF over application layer, to indicate adding/deleting/updating the information for the PEGC/PIN.

4-5. The PINMF sends PIN configuration to the PEMC and to the PINE/PEGC via the PEMC over application layer. The configuration may include the PIN name/ID. For PINE, the configuration may include access information to discover/select and visit PEGCs, e.g., the allowed SSIDs, corresponding passwords, and PIN APP visiting parameters of PEGC, the SSID of the PINEs, etc. For PEGC, the configuration may include parameters such as SSID, password, parameters for visiting PINMF, etc.

6.	In case of PEGC, the PEGC may close direct connection with PEMC. The PEGC connects to 5GS.

For PINE management, the procedure stops. For PEGC management, the following steps are performed.

7.	The PEGC registers to the PINMF over application layer using the configured parameters obtained in step 5.

8.	The PINMF sends Create/Update/Delete PIN Parameters Request (GPSI, PIN ID, RAT Info, Discovery Info) to the NEF. The RAT Info includes RAT configuration information, e.g., whether the SSID is hidden or not, activated RAT types (e.g., WiFi, BT), discovery methods (e.g., WiFi for open, BT for restrict), etc. The Discovery Info includes the allowed list or block list of device information (e.g., MAC address, IP address) for authorization of the PINE discovering PEGC.

9.	The NEF queries PIN subscription of the PEGC from UDM/UDR to check whether the PEGC is a group member in a group that also contains the PEMC. If the authorization fails, go to step 16.

10.	The NEF sends Create/Update/Delete PIN Parameters Request (SUPI, PIN ID, RAT Info, Discovery Info) to the UDM/UDR.

11.	The UDM/UDR stores the PIN related information and sends PIN Parameters Delivery (SUPI, PIN ID, RAT Info, Discovery Info) to the AMF.

12.	The AMF sends the PIN ID, RAT Info, and Discovery Info to the PEGC.

13.	If the UE supports PEGC, the UE sends result to the AMF.

14.	If the result from the UE is received, or timed out for receiving the result from UE (i.e., the UE does not support PEGC), the AMF responds to the UDM/UDR.

15-16. The UDM/UDR responds to the NEF. The NEF responds to the PINMF.

17.	The PINMF may update PIN configuration to the PEMC over application layer.

#### 6.Y.2.3	Management of association between PEGC and PINE

Figure 6.Y.2.3-1: Management of association between PEGC and PINE

1.	The PEGC has established PDU Session.

2.	The PINE establishes direct connection with the PEGC using the parameters obtained in procedure described in clause 6.Y.2.2.

3.	The PINE sends relay request over application layer to the PEGC. The parameters for visiting the PEGC over application layer can be obtained over application layer in procedure described in clause 6.Y.2.2.

4.	The PEGC sends relay request over application layer to the PINMF. The parameters for visiting the PINMF over application layer can be obtained over application layer in procedure described in clause 6.Y.2.2.

5.	The PINMF authorizes the request and initiates the relay path management procedures using solution Y addresses to KI#4.

6-7. The PINMF sends relay response to the PEGC over application layer, and PEGC sends the relay response to the PINE over application layer.

#### 6.Y.2.4	Parameters provisioned to PEMC

The parameters provisioned to PEMC is over application layer. The parameters provisioned via application layer may include following:

-	PIN name and PIN ID, as well as the PIN description.

-	Characteristic of PIN, e.g., wearable, smart home, smart office, etc.

-	List of PEGC

>	Device Info, e.g., GPSI, IP address, name, description.

>	Access RAT info, e.g., WiFi open/close, BT open/close, PC5 open/close, SSID hidden, etc.

>	Discovery Info.

>>	Connection parameters, e.g., SSID and Password for WiFi.

>>	Discovery method, e.g., monitor, announcer, open, restrict, etc.

>>	Restrict discovery info, e.g., list of allowed PINE info, list of block PINE info.

>	List of PINE associated with the PEGC.

>>	Service type, e.g., printer, TV, etc.

>>	Allowed peer info or block peer info.

>>	Access RAT info.

>>	Discovery Info.

-	List of PINE not associated with any PEGC

>	Service type, e.g., printer, TV, etc.

>	Access RAT info.

>	Discovery Info.

#### 6.Y.2.5	Parameters provisioned to PINE

The parameters provisioned to PINE is over application layer. The parameters provisioned via application layer may include following:

-	PIN ID.

-	List of PEGC

>	Access RAT info.

>	Discovery Info.

>>	Connection parameters, e.g., SSID and Password for WiFi.

>>	Discovery method, e.g., monitor, announcer, open, restrict, etc.

-	List of PINE allowed to have direct communication

>	Discovery Info.

#### 6.Y.2.6	Parameters provisioned to PEGC

The parameters provisioned to PINE is over 5GC control plane. The parameters provisioned via 5GC control plane may include following:

-	PIN ID.

-	Access RAT info, e.g., WiFi open/close, BT open/close, PC5 open/close, SSID hidden, etc.

-	Discovery Info.

>	Connection parameters, e.g., SSID and Password for WiFi.

>	Discovery method, e.g., monitor, announcer, open, restrict, etc.

>	Restrict discovery info, e.g., list of allowed PINE info, list of block PINE info.

### 6.Y.3	Impacts on Existing Nodes and Functionality

NEF:

-	Support PEMC authorization operation.

-	Support PIN Parameters operations and PEGC authorization.

UDM:

-	Support PIN Parameters operations with NEF and delivery PIN parameters to UE via AMF

UDR:

-	Support PIN subscription for PEMC and PEGC and PIN subscription query.

UE (support PEGC):

-	Support PIN Parameters provisioning over NAS.

Editor's note:	Additional impacts are FFS.

# * * * * End of changes * * * *