3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202480

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	New Solution: Communication of PIN

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes a new solution for communication of PIN.

1. Introduction

This solution mainly addresses KI#4 “Communication of PIN”.

This solution also addresses some part of the KI#6 “Policy and parameters provisioning for PIN”, i.e., UE to UE relay parameter provisioning and QoS parameter provisioning.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: “System architecture for the 5G System (5GS); Stage 2”.

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

[x]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2"

# * * * * Second change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |  |
| Y |  |  |  | X |  | X |  |

# * * * * Third change (NEW TEXT) * * * *

## 6.Y	Solution #Y: Communication of PIN

### 6.Y.1	Description

This solution mainly addresses KI#4 “Communication of PIN”. This solution also addresses some part of the KI#6 “Policy and parameters provisioning for PIN”, i.e., UE to UE relay parameter provisioning and QoS parameter provisioning.

The steps in the following call flows depicted as grey line in the figures are out of SA2 scope.

### 6.Y.2	Procedures

#### 6.Y.2.1	Management of relay path

Figure 6.Y.2.1-1: Management of relay path

1.	The PEGC has established PDU Session that may involve framed routing as described in TS 23.501 [2] clause 5.6.14.

2.	The PINE establishes direct connection with the PEGC using the parameters obtained in procedure described in clause 6.Y.2.2.

3.	The PINE sends relay request over application layer to the PEGC. The parameters for visiting the PEGC over application layer can be obtained over application layer in procedure described in clause 6.Y.2.2.

4.	The PEGC sends relay request over application layer to the PINMF. The parameters for visiting the PINMF over application layer can be obtained over application layer in procedure described in clause 6.Y.2.2.

5.	The PINMF authorizes the request and sends Create/Update/Delete Association Request (GPSI, [UE address, DNN/S-NSSAI], PIN ID, U2N Relay Info, [Framed Route Info]) to the NEF if authorization succeeds. The U2N Relay Info includes packet filters that the PEGC is able to relay and may include service requirements for the PINE. If it is the first time for creating the association for the PEGC, the Framed Route Info is included and used by the SMF to configure framed routing as described in TS 23.501 [2] clause 5.6.14 for the PDU Session of the PEGC.

6.	The NEF may interact with PCF for setting DNAI and may change the Framed Route Info to the PDU Session of the PEGC.

7.	The procedure described in TS 23.502 [x] clause 4.3.5 may be performed to change the serving SMF that supports PIN according to the DNAI. The SMF uses the Framed Route Info to configure framed routing as described in TS 23.501 [2] clause 5.6.14 with the UPF for the PDU Session of the PEGC.

8.	If the PEGC has subscribed to be in a group that includes the member of the PEGC and the PEMC, the SMF initiates SMF-NEF connection setup with NEF similar as described in TS 23.502 [x] clause 4.25.2 and 5.2.6.15, the difference is that the subscription data includes "NEF ID for PIN" instead of "NEF ID for NIDD", and the name of "NIDD information" is changed to "PIN information" that contains same thing, i.e., GPSI and AF ID.

9.	The NEF sends Create/Update/Delete Association Request (SUPI, PIN ID, U2N Relay Info) to the SMF.

10.	The SMF may trigger authentication and authorization procedure between PINE and UDM, or between PINE and external DN-AAA, via the PEGC, i.e., the SMF sends EAP Identity Request to the PINE.

Editor's note:	The authentication and authorization procedure need coordination with SA3.

11.	The SMF interacts with PCF for the flow descriptions in the U2N Relay Info related to the PINE.

12.	The SMF performs PDU Session Modification with the PEGC, may add, update, or remove QoS flows serve the PINE, and delivers the PIN ID and U2N Relay Info to the PEGC via N1 message during the PDU Session Modification.

13-14. The SMF responds to the NEF. The NEF responds to the PINMF.

15-16. The PINMF sends relay response to the PEGC over application layer, and PEGC sends the relay response to the PINE over application layer.

#### 6.Y.2.2	Management of communication via 5GC

Figure 6.Y.2.2-1: Management of communication via 5GC

1.	The PEGC established a PDU Session with SMF supporting PIN.

2.	The PEMC configures a whitelist or blacklist for communications via 5GC, e.g., for visiting internet, for communicating with a PINE from internet or from other PINEs, etc.

3.	The PINMF authorizes the request and sends Create/Update/Delete Association Request (GPSI, [UE address, DNN/S-NSSAI], PIN ID, Routing Info) to the NEF if authorization succeeds. The Routing Info includes the information whether some PINEs are allowed/disallowed to access internet or a specific internet service or other PINEs. The NEF authorize the request.

4.	Steps 6-8 in clause 6.Y.2.1 may be performed to setup the SMF-NEF connection.

5.	The NEF sends Create/Update/Delete Association Request (SUPI, PIN ID, Routing Info) to the SMF.

6.	The SMF interacts with UPF serving the PDU Session of the PEGC for installing, updating, removing a corresponding FWA from the UPF.

7-8. The SMF responds to the NEF. The NEF responds to the PINMF.

#### 6.Y.2.3	Management of communication via PEGC

Figure 6.Y.2.3-1: Management of communication via PEGC

1.	The PEGC established a PDU Session with SMF supporting PIN, and SMF-NEF connection for managing PIN has been setup.

2.	The PEMC configures a whitelist or blacklist for PINEs to communicate via the PEGC.

3.	The PINMF authorizes the request and sends Create/Update/Delete Association Request (GPSI, [UE address, DNN/S-NSSAI], PIN ID, U2U Relay Info) to the NEF if authorization succeeds. The U2U Relay Info includes the information whether some PINEs are allowed/disallowed to communicate with other PINEs via the PEGC.

4.	The NEF sends Create/Update/Delete Association Request (SUPI, PIN ID, U2U Relay Info) to the SMF.

5-6. The SMF sends N1 message (PIN ID, U2U Relay Info) via the AMF to the PEGC.

7-8. The UE responds with a result.

9-10. The SMF responds to the NEF, the NEF responds to the PINMF.

#### 6.Y.2.4	Mobile termination procedure for PINE

Figure 6.Y.2.4-1: Mobile termination procedure for PINE

1.	The PEGC established a PDU Session with SMF supporting PIN.

2.	The PINMF sends Create/Update/Delete Association Request (GPSI, [UE address, DNN/S-NSSAI], PIN ID, Routing Info) to the NEF. The Routing Info includes the type of the PINE, which indicates whether the PINE is notifiable, e.g., when the PINE can be reached by other way (e.g., the PINE has dual registration with different address with the PINMF), or the PINE is a listener and PEGC is announcer over the, e.g., WiFi, BT, etc.

Steps 6-8 in clause 6.Y.2.1 may be performed to setup the SMF-NEF connection.

3.	The NEF sends Create/Update/Delete Association Request (SUPI, PIN ID, Routing Info) to the SMF.

4-5. The SMF responds to the NEF. The NEF responds to the PINMF.

6.	The PIN APP on the PEGC may detect that the communication with the PINE is not available, the PEGC sends information to PINMF over application layer. The PINMF authorizes the request and sends Update Association Request (GPSI, [UE address, DNN/S-NSSAI], PIN ID, Status Info) to the NEF if authorization succeeds. The Status Info includes the information of the PINEs that are unreachable. The NEF sends Update Association Request (SUPI, PIN ID, Status Info) to the SMF. The SMF responds to the NEF. The NEF responds to the PINMF.

The PEGC may detect that the direct connection with the PINE is lost, the PEGC initiates PDU Session Modification indicating the PINE is unreachable.

The PEGC may transit into CM-IDLE state or the PDU Session for relay may be inactive, the PEGC initiates PDU Session Modification.

7.	According to the information received in step 6 and the Routing Info, the SMF interacts with the UPF serving the PDU Session of the PEGC for instructing that DL data notification for the PINE is needed.

8.	When DL data to the PINE is arrived and DL data notification is instructed, the UPF sends DL data notification related to the PINE to the SMF.

9.	According to the type of the PINE in the Routing Info, the SMF sends Message Delivery to the NEF to indicate the data arrival for a PINE.

10.	The NEF sends Message Delivery to the PINMF.

11.	The PINMF instructs the PINE via another address or instructs the PEGC over application layer.

12.	The PINE or PEGC establish the direct connection according to the instruction.

13.	The PEGC requests the PINMF to establish the relay path for the PINE over application layer.

### 6.Y.3	Impacts on Existing Nodes and Functionality

NEF:

-	Support PIN Association operations with PINMF and SMF.

-	Support indicating DNAI related to PIN to PCF.

SMF:

-	Support SMF-NEF connection setup based on PIN subscription.

-	Support PIN Association operations with NEF.

-	Support instructing UPF for rules related to PIN Elements.

-	Support provisioning relay information to PEGC over NAS.

-	Support delivering message related to DL data notification to NEF.

-	Support triggering authentication and authorization procedure between PINE and UDM/external DN-AAA.

UE (support PEGC):

-	Support relay information provisioning for PIN over NAS.

-	Support proxy authentication and authorization procedure between PINE and UDM/external DN-AAA.

Editor's note:	Additional impacts are FFS.

# * * * * End of changes * * * *