3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202481

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	vivo

Title:	New KI and Solution: Identification assignment for PIN Elements

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper includes the almost agreed KI (S2-2200837r07) and proposes a new solution for the KI.

1. Introduction

This paper includes the almost agreed KI “Identification of PIN and PIN Elements”, and proposes solution addresses this KI.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change (S2-2200837r07) * * * *

## X.x1	Key Issue #X1: Identification of PIN and PIN Elements

### x.x1.1	Description

In order to support the management of the PIN, the 5GS needs to be able to identify the PINE of a PIN and/or the traffic generated of the PINE, which will help, e.g., differentiate the policy for a PINE, differentiate QoS requirement for a PINE, etc.

The key issue focuses on potential enhancements needed to support identification of PIN and PIN Elements. The following aspects will be studied as part of the key issue:

-	How to identify a PIN and who manages the PIN identity. Whether and what characteristics of a PIN shall be known to the 3GPP network (e.g. type of PIN (wearable, home automation, factory etc.), max of PIN elements in the PIN etc.).

-	How to support identify PINE, PEGC and PEMC, and whether and how the 5GS manages the identifier;

# * * * * End of changes * * * *