3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202521r01

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	definition and assumption for the group in GMEC study

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: introduce the definition and assumption for the group in GMEC study

1. Introduction/Discussion

Introduce Terms, Architectural Assumptions and Requirements for the group in this study to make the target clear and focused.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.1.0.

# * * * * First change * * * *

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], in TS 23.501 [2] and in TS 23.502 [3] apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

The following definitions of terms are copied from TS 23.501 [2]:

5G LAN-type Service: A service over the 5G system offering private communication using IP and/or non-IP type communications.

5G LAN-Virtual Network: A virtual network over the 5G system capable of supporting 5G LAN-type service.

5G VN Group: A set of UEs using private communication for 5G LAN-type service.

Private communication: A communication between two or more UEs belonging to a restricted set of UEs.

The following definitions of terms are introduced for this study:

5G UE Group: A set of UEs, which an AF wants to handle together with respect to a common set of interest service(s) and their service characteristic(s).

# * * * * Second change * * * *

4	Architectural Assumptions and Requirements

Editor's note:	This clause will document the Architectural Assumptions and Requirements applicable for the study.

4.1	Architectural Assumptions

The NEF exposure framework as in TS 23.501 [2] is the baseline for study of 5G capabilities exposure for industrial and automation applications.

The QoS and policy framework as in TS 23.501 [2] is used for enforcement of QoS applicable to each UE of a given group.

Existing QoS mechanism and service area mechanism are re-used for enforcement of service area or QoS applicable to each UE of a given group, thus neither new QoS nor service area enforcement mechanism will be specified.

Architecture and functions defined for 5G LAN-type service in clauses 4.4.6, 5.8.2.13 and 5.29 of TS 23.501 [2] are used as the baseline for 5G VN group communication enhancements.

Unstructured PDU Session type is out of scope for this study.

The term 5G UE group in the scope of this study represents a set of UEs which can be handled together by an AF with respect to a common set of interest service(s) and their service characteristic(s). 5G UE group includes at least, but not limited to, the 5G VN group as defined in clause 5.29.2 of TS 23.501 [2] with using 5G VN group communication defined in clause 5.8.2.13 of TS 23.501 [2]. For example, 5G UE group may also be a group UE identified by External Group ID, or a group UE identified Internal Group ID, or a group UE identified by address (multicast/broadcast address).

4.2	Architectural Requirements

Solutions about 5G capabilities exposure for industrial and automation applications needs to define the type of 5G UE group the solution is targeting to and the common service(s) used by the 5G UE group.

Solutions about 5G capabilities exposure for industrial and automation applications needs to strive to generalize the existing 5G VN group management for management of any type of 5G UE group.

# * * * * End of changes * * * *