3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202524r01

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Update Solution 5 to complete the procedure

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: It is proposed to provide a concrete call flow and procedure description for Solution #5.

1. Introduction/Discussion

In SA#2 149 meeting, the functional description of Solution #5 is approved while the corresponding procedures are missing, it is proposed to provide a concrete call flow and procedure description for Solution #5.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.1.0.

# * * * * First change * * * *

6.5	Solution #5: Multiple SMFs involved 5G VN group communication

6.5.1	Introduction

This solution is for Key Issue #4 on enhancements of 5G VN group communication.

6.5.2	Functional Description

The main idea of this solution is as below:

-	Each SMF serving the same 5G VN group reports/registers its identifier and the 5G VN group identifier to the Group Session Management Function. Thus, the Group Session Management Function has the knowledge of all the SMF serving the 5G VN group.

-	Each SMF can be aware of other SMFs via the Group Session Management Function as following:

-	A SMF subscribes to the Group Session Management Function for the 5G VN group change events (e.g., when another new SMF starts to serve the same 5G VN group or another SMF stops to serve the 5G VN group). When the 5G VN group change events occurs, the Group Session Management Function sends a notification including the changed SMF’s identifier information to the subscribed SMF; or

-	A SMF can query the SMF information of other group member from the Group Session Management Function.

-	The SMF requests a session with another SMF and these two SMFs can establish the N19 tunnels between UPFs and configure the UPFs via the session. When the SMF detects, allocates or releases an address of the UE within the 5G VN group, the SMF sends update/release requests to the other SMF via the session to update the configuration on the UPF.

Editor’s Note: it is FFS how the SMF(s) collaborate to manage the topology of N19 links (star, ring, tree, etc…)

Figure 6.5.2-1 depicts the architecture for the solution.

Figure 6.5.2-1: Architecture for multiple SMFs involved 5G VN group communication

Editor's note:	The Group Session Management Function is logical function; it is FFS whether the Group Session Management Function should be a standalone function entity or collocated with existing network function entity (e.g. UDM).

6.5.3	Procedures

6.5.3.1 Retrieval of peer SMF

Pre-condition/assumption:

1. UE1 and UE2 belongs to the same 5G VN group.

Figure 6.5.3.1-1: multiple SMFs involved 5G VN group communication

1. The UE1 establishes a PDU session (clause 4.3.2.2.1 of TS 23.502 [3]) for getting access to a 5G VN group. In this figure, the UE1 is served by SMF1 and UPF1.

2. SMF1 reports/registers the 5G VN group identifier (which can be identified by DNN and S-NSSAI) of UE1 and SMF1 identifier to the GSMF (Group Session Management Function). The SMF1 may also registers the identifier of UE1 to the GSMF.

Editor's note:	It is FFS whether step 2 executes for every new PDU Session to 5G VN in SMF1 or just for the first PDU Session to 5G VN in SMF1.

3. In order to enable communication among the group members served by different SMFs, the SMF1 may subscribe to the GSMF for the 5G VN group change events (e.g., when another SMF serves the same 5GVN group or another SMF stops to serve the 5GVN group).

4. Similar as step 1. In this figure, the UE2 is served by SMF2 and UPF2.

5. Similar as step 2.

6. Similar as step 3.

7. The SMF1 retrieves other SMF(s) information via the following two alternatives:

7a. Corresponding to step 3, the GSMF sends a notification including the SMF2 identifier information to the subscribed SMF1 that a new SMF (i.e., the SMF2) starts to serve the 5GVN group.

7b. The SMF1 queries other SMF(s) information of the 5G VN group from the GSMF with the group identifier in the request, or the SMF1 queries the serving SMF information of a certain target UE from the GSMF with the group identifier and UE identifier in the request (e.g., this may be triggered by a data notification from the UPF).

6.5.3.2 N19 tunnel management

Figure 6.5.3.2-1: N19 tunnel management

1. The SMF1 retrieves the SMF2 information as described in clause 6.5.3.1.

2. Upon the received SMF2 information, the SMF1 determines to establish N19 tunnel between UPF1 controlled by SMF1 and UPF(s) controlled by SMF2. The SMF1 initiates a N4 Session Modification to the UPF1 and the UPF1 provides tunnel information to the SMF1.

The SMF1 may establish N19 tunnel between UPF1 and UPF(s) controlled by SMF2 via either option #1 or option #2:

Option #1:

3. The SMF1 sends PDUSession_Create Request (5G VN group identifier, SM Context ID at SMF1, UPF1 tunnel information, UE1 IP address) towards the SMF2 to establish a session with SMF2. The SMF2 would reject the PDUSession_Create Request from the SMF1 if the session already exists.

4. The SMF2 initiates a N4 Session Modification to the UPF2. During this step:

- The SMF2 installs N4 rules(e.g. PDRs and FARs related to UE1) on the UPF2.

- The UPF2 provides tunnel information to the SMF2.

5. The SMF2 responds to the SMF1(SM Context ID at SMF2, UPF2 tunnel information, UE2 IP address). The SMF1 can update the session based on the received SMF Context ID at SMF2 later.

6. The SMF1 initiates a N4 Session Modification to the UPF1 and installs N4 rules (e.g. PDRs and FARs related to UE2) on the UPF1.

7a. If another UE (e.g. UE3) belongs to the same 5G VN group as UE1 is also served by the SMF1 and UPF3 (it can be the same as or different with UPF1). The SMF1 installs N4 rules related to UE1 and UE2 on the UPF3 and receives UPF3 tunnel information from the UPF3. The SMF1 sends PDUSession_Update Request (SM Context ID at SMF2, UPF3 tunnel information, UE3 IP address) towards the SMF2.

7b. The SMF2 initiates a N4 Session Modification to install N4 rules related to UE3 on the UPF2.

7c. The SMF2 sends PDUSession_Update Respond to the SMF1.

Editor's note:	It is FFS whether Nsmf_PDUSession_Create/Update or other (existing or new) SMF service operations can be used in Option #1.

Option #2:

8. The SMF1 sends N19 tunnel establishment request (5G VN group identifier, UPF1 tunnel information, UE1 IP address) to the SMF2 via the GSMF. The GSMF stores this information when receive the request from the SMF1 and forward the request to the SMf2.

9. The SMF2 initiates a N4 Session Modification to the UPF2. During this step:

- The SMF2 installs N4 rules(e.g. PDRs and FARs related to UE1) on the UPF2.

- The UPF2 provides tunnel information to the SMF2.

10. The SMF2 sends respond(UPF2 tunnel information, UE2 IP address) to the SMF1.

11. The SMF1 initiates a N4 Session Modification to the UPF1 and installs N4 rules (e.g. PDRs and FARs related to UE2) on the UPF1.

Editor's note:	It is FFS how SMF1 instructs UPF1 for N19 tunnels towards UPFs controlled by SMF2 (other than UPF2) for the same 5G VN group.

12a. If another UE (e.g. UE3) belongs to the same 5G VN group as UE1 is also served by the SMF1 and UPF3 (it can be the same as or different with UPF1). The SMF1 installs N4 rules related to UE1 and UE2 on the UPF3 and receives UPF3 tunnel information from the UPF3. The SMF1 sends N19 tunnel modification request (5G VN group identifier, UPF3 tunnel information, UE3 IP address) towards the SMF2 via the GSMF. If a UE3 is served by SMF3 (i.e., a SMF different with SMF1), the SMF3 can retrieve the UPF1 and UPF2 tunnel information from the GSMF.

12b. The SMF2 initiates a N4 Session Modification to install N4 rules related to UE3 on the UPF2.

12c. The SMF2 sends respond to the SMF1.

Editor's note:	It is FFS whether and how to separate step 12 as another call flow.

6.5.4	Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *