3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202530r01

Elbonia, April 6 – 12, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	New Solution for timing synchronization status reporting

Document for:	Approval

Agenda Item:	9.7

Work Item / Release:	FS_5TRS_URLLC / Rel-18

Abstract: This paper proposed a solution on how 5GC learns about 5GS network timing synchronization status to be able to inform UEs and AFs

1. Introduction/Discussion

This paper proposes a new solution on how 5GC learn about 5GS network timing synchronization status to be able to inform UEs and AFs.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-25 V0.1.0.

# * * * * First change * * * *

6.X	Solution #x: Timing synchronization resiliency and status reporting

6.X.1	Introduction

This solution aims to address Key issue #1: 5GS network timing synchronization status and reporting. When 5GS provides timing resiliency service e.g. in the smart grid or financial sector, the timing synchronization status (such as divergence from UTC, timing source degradation) needs to be able to inform UEs (e.g. application running in the UE), devices attached to the UE (i.e. that receive time information from 5GS) and AFs. The AF can configure 5GS with the requirements about the timing resiliency service to meet when timing synchronization event happens.

This solution makes the following assumptions:

-	UEs are consuming (g)PTP timing synchronization service from UPF/NW-TT that acts as grand-master based on IEEE Std 802.1AS [7] or IEEE Std 1588 [8];

-	5G GM may have different sources of time/frequency like GNSS signal, Synchronous Ethernet (SyncE), PTP transport network, PPS input, etc. It is assumed that the 5G GM is collocated with UPF or UPF is synchronized with 5G GM by PTP compatible transport, etc.

-	When UPF detects 5GS networking timing synchronization status for original timing source, it sends the reporting to 5GC. Additionally the UPF activates the timing synchronization configuration for PTP GM functionality as configured by TSCTSF in case of degradation for original timing source, so as to provide seamless time synchronization service for the UEs; or the UPF de-activates the timing synchronization configuration for PTP GM functionality as configured by TSCTSF in case of recovery for original timing source.

Editor's note:	Reporting of 5GS network timing synchronization status to UE is FFS.

Editor's note:	It is FFS how to address the case where time synchronization service is offered based on 5G Access Stratum timing distribution.

6.X.2	Functional Description

This solution uses the following principles:

-	TSCTSF collects timing synchronization capability of UPF via PMIC or UMIC:

-	The timing synchronization capability can indicate holdover time or traceability capability information. For example, clock class, synchronization accuracy, holdover time parameters for the smart grid scenario, or UTC divergence scale and synchronization accuracy parameters for financial scenario;

-  TSCTSF creates timing synchronization configuration on the UPF upon reception of AF request:

-  TSCTSF receives timing synchronization requirement from AFs. The timing synchronization requirement includes timing resiliency service KPIs that 5GS needs to meet, e.g. AF requests for holdover time for 1h for ms sync accuracy for the smart grid scenario, or AF requests for 1ms maximum divergence from UTC for financial scenario;

Editor's note:	It is FFS whether and what additional information is needed in AF request for time synchronization service and subscription for 5GS networking timing synchronization status.

-  TSCTSF confirms that the 5GS can meet the requirement based on UPF reported timing synchronization capability;

-  TSCTSF creates a timing synchronization configuration on UPF;

-  With the timing synchronization configuration, the UPF detects and reports the networking timing synchronization status, and (de)activates the timing synchronization configuration for PTP GM functionality accordingly.

-	UEs are consuming (g)PTP timing synchronization service from UPF/NW-TT that acts as grand-master based on IEEE Std 802.1AS [7] or IEEE Std 1588 [8];

-	When UPF detects networking timing synchronization status for original timing source, it sends the status reporting to TSCTSF, and the TSCTSF forwards the reporting to AFs

-  When UPF detects degradation or unavailable for original timing source, the UPF activates the timing synchronization configuration for PTP GM functionality as configured by TSCTSF, and the status reporting contains original timing source disabled status: 5GS timing synchronization enabled status, the timing source clock class, time validity;

-  When UPF detects recovery for original timing source, the UPF de-activates the timing synchronization configuration for PTP GM functionality, and status reporting contains original timing source enabled status: 5GS timing synchronization disabled status, the timing source clock class, time validity

6.1.3	Procedures

Existing PDU Session establishment procedures are reused in timing synchronization capability configuration. Existing timing distribution procedures are reused for UEs using (g)PTP messages

6.1.4	Impacts on services, entities and interfaces

UPF:

Support of signaling the timing synchronization capability to TSCTSF

Support of reporting time synchronization status (e.g., holdover time or UTC divergence status) to TSCTSF

TSCTSF:

Support of receiving timing synchronization requirement from AFs

Support of timing synchronization capability configuration

Support of timing synchronization status report to AFs

AF:

Support of providing timing synchronization requirement (e.g. holdover time and UTC divergence requirement) to TSCTSF, and receiving status reporting from TSCTSF

# * * * * End of changes * * * *