3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202549r01

Elbonia, April 6th – 12th, 2022

Source:	OPPO

Title:	Solution for KI#2 5GC awareness of URSP enforcement

Document for:	Approval

Agenda Item:	9.22

Work Item / Release:	FS_eUEPO / Rel-18

Abstract: Propose a solution for KI#2.

1. Introduction/Discussion

Currently the UCU procedure is used to provision UE policy (including URSP policy) to UE. However, the result in the UCU Response message only indicates whether the UE supports the type of UE policy per Policy Section.

Meanwhile, if UE supports URSP but not support a URSP rule (e.g. due to not recognize the TD parameter in a URSP rule), the UE will ignore the URSP rule without any notification to network side. This implicit ignoring will incur the 5GC is unable to realize whether the specific Application traffic will be associated to the expected slice described in a RSD of a URSP rule.

Therefore, it is proposed to let UE provides the supported URSP rule in the result in UCU response.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-85.

# * * * * First change (all text is new) * * * *

6.X	Solution #X: 5GC awareness of URSP recognition by UE

6.X.1	Description

Currently the UCU procedure is used to provision UE policy (including URSP policy) to UE. However, the result in the UCU Response message only indicates whether the UE supports the type of UE policy per Policy Section.

Meanwhile, if UE supports URSP but not support a URSP rule (e.g. due to not recognize the TD parameter in a URSP rule), the UE will ignore(skip) the URSP rule without any notification to network side. This implicit ignoring will incur the 5GC is unable to realize whether the specific Application traffic will be associated to the expected slice as described in an RSD of a URSP rule.

Therefore, it is proposed to use the result in UCU response to indicate if and which URSP rule is not recognized/supported by UE.

6.X.2	Procedures

Figure 6.x.2-1 Result of the delivery of UE policies indicate if and which URSP rule is supported by UE

The procedure above is the same as the UCU procedure in clause 4.2.4.3 in TS 23.502, with the extension that

-  When receiving the UE policies in step-3, the UE will evaluate the each URSP rule in the URSP, and identify which URSP rule is not recognized (not supported) by UE.

-  The UE indicate the additional information in the result in step-4: whether and which URSP rule is not recognized (not supported) by the UE

NOTE: the Traffic Descriptor can be used to indicate a specific URSP rule

Based on the feedback from UE, the PCF may update the parameters (e.g. if some parameters in TD are wrongly configured) and provision the updated URSP to UE again. But whether and how to determine the URSP based on the feedback is per implementation pecific.

6.X.3	Impacts on services, entities and interfaces

UE:

- Check the URSP rules when it is provisioned in “Delivery of UE policies” message; and

- Provide feedback about which URSP Rule/RSD cannot be recognized in the UE policy container conveyed in “Result of the delivery of UE policies” message

# * * * * End of changes * * * *