3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202551

Elbonia, April 6 – 12, 2022

Source:	Huawei, HiSilicon

Title:	Update solution for redundancy steering mode

Document for:	Approval

Agenda Item:	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract: This paper updates the solution for redundancy steering mode.

1. Introduction/Discussion

This paper solves the following two ENs.

Editor's note: Whether threshold values other than packet loss rate, such as RTT, are applicable, or whether and how multiple threshold values can be used together, is FFS.

Editor's note: Whether the redundancy transmission solution can be triggered during the traffic switching phase to avoid packet loss during handover procedure is FFS.

- For the first one, except the packet loss rate, no other threshold values seem to be beneficial to the redundancy transmission. So this EN is removed directly.

- For the second one, a NOTE is added to clarify that redundancy transmission can be applied to any steering mode for MPTCP during the traffic switching phase between the 3GPP and non-3GPP accesses in 5GS or the handover procedure from 5GS to EPS for 3GPP access.

2. Text Proposal

It is proposed to capture the following changes into the TR 23.700-53 v0.1.0.

# * * * * First change * * * *

6.1	Solution #1: New steering mode - Redundancy steering mode with packet loss rate

6.1.1	Introduction

This solution addresses KI#3 on support of redundant traffic steering.

6.1.2	High-level Description

The redundancy steering mode was discussed during both Rel-16 and Rel-17 ATSSS study phase, as described in clause 6.3.1.1, 6.4.1 of TR 23.793 [26] for the loss rate sensitive traffic, such as IMS singling, video, and some TCP-based traffic, and the solution 4 in clause 6.4 of TR 23.700-93 [5]. It allows the traffic transmitted via 3GPP and non-3GPP accesses in a redundant way to achieve the lowest latency and to lower the loss rate.

It is proposed to further enhance the redundancy steering mode as described in TR 23.793 [26], with which the traffic will always be transmitted over both accesses once applied, to make it possible that the traffic transmission goes via both accesses, if necessary, or via only one access to save the transport resource. In details, when the traffic is allowed on both accesses and the packet loss rate (PLR) is provided with the redundancy steering mode:

-	The UE and the UPF can select any one of the accesses (based on implementation) to transport the traffic, if both accesses can satisfy the PLR requirement.

-	If only one access can satisfy the PLR value, this access is applied to transport the traffic.

-	Otherwise, redundant transmission over both accesses is triggered.

For example, if the loss rate on one access does not exceed the PLR threshold, then only this one access is applied, otherwise, redundant transmission is triggered.

When the traffic is allowed on both accesses and the redundancy steering mode is provided without a PLR threshold value, both accesses are always applied for the redundant transmission.

See below Figure 6.1.2-1 for details, where UL packet flow is taken as an example. The DL packet flow shares the same mechanism.

This redundancy steering mode can be applied by the MPTCP functionality with new requirements to be derived from this study/solution.

NOTE 1:	The redundant transmission algorithm required by the redundancy steering mode with packet loss rate relies on the implementation of the MPTCP by the OS today.

NOTE 2:	The redundancy steering mode with packet loss rate does not apply to the ATSSS-LL steering functionality.

Editor's note:	How the receiver will treat duplicated packets in case of MP-QUIC or MP-DCCP is FFS.

Editor's note:	The redundant traffic steering mode for the GBR traffic is FFS.

Editor's note:	Whether threshold values other than packet loss rate, such as RTT, are applicable, or whether and how multiple threshold values can be used together, is FFS.

NOTE:	For the MA PDU session, when the UE performs traffic switching between the 3GPP and non-3GPP accesses in 5GS or the handover procedure from 5GS to EPS for 3GPP access, and the MPTCP functionality is applied for the traffic, the UE and the UPF may trigger the redundant transmission via the 3GPP and non-3GPP access during the traffic switching or handover from 5GS to EPS for 3GPP access, based on the UE and UPF implementation. This redundant transmission can be applied to all the MPTCP traffic with any steering mode.

Editor's note:	How the UPF knows handover procedure from 5GS to EPS is to be started so that the UPF can decide to trigger redundant transmission for DL traffic is FFS.

Figure 6.1.2-1: Redundancy steering mode

# * * * * End of changes * * * *