SA WG2 Meeting #150e	S2-2202585r01

# April 6th – 12th, 2022               		       (revision of S2-2202585)

Source: 	China Telecom

Title: 	New solution for KI#2: Support UPF to expose usage report to other NFs

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF event exposure service for SMF

1 Discussion

The following aspects should be studied in KI#2 to support UPF event exposure service(s):

-	How and what specific information the UPF can expose to NWDAF so NWDAF can provide existing (R16-R17) data analytics as specified in TS 23.288 [5]. Support of New R18 data analytics per the R18 FS_eNA_Ph3 may also be considered in alignment with that study

-	How and what specific information the UPF can expose to NEF/Local NEF/trusted AF, e.g. the information which can be exposed in R18 SA2 studies such as FS_EDGE_Ph2 and FS_XRM, or information which has been justified for exposure.

NOTE:	The Relevant Event IDs of the UPF event exposure services above can be introduced.

-	Whether PCF, CHF, and other NFs need to invoke UPF event exposure service. If yes, how and what specific information the UPF can expose to these NFs.

This solution proposal presented here addresses the bullet 3 in KI#2.

The solution propose UPF to expose usage report to other NFs. In the current 5G core network architecture, SMF and UPF exchange information through the N4 interface. The PFCP protocol is used in N4 interface, which has no universality in other interface of core network. This solution intends to migrate the service of the original N4 interface to the UPF event exposure service of the Nupf interface.

2 Proposal

It is proposed to update TR 23.700-62 as follows.

Start of change

6.X	Solution: Support UPF expose information to other NFs (SMF)

6.X.1	Key Issue mapping

Editor's Note:	This clause lists the key issue(s) addressed by this solution.

6.X.2	Description

Editor's Note: This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

The solution introduces UPF usage report exposure service. Usage report is the usage data collected for analysic or charging purposes, and is required by other NFs (such as SMF, CHF, PCF, NWDAF). In the current core network architecture, PFCP is used as an protocol of N4 interface between SMF and UPF. SMF use PFCP’s URR to interact with UPF for usage report. This solution intends to use the UPF event exposure service to obtain usage report. It adopts a service-based interface and uses the HTTP2 protocol, which is more general than the PFCP protocol of N4 interface. Other NFs can transmit usage report with UPF directly, which will reduce repeated data transmission and the pressure on SMF.

6.X.3	Procedures

Editor's Note: This clause describes high-level procedures and information flows for the solution.

6.X.3.1	UPF Event Exposure service for usage report

The following Figure 6.X.3.1-1 depicts the UPF Event Exposure service for usage report procedure.

Figure 6.X.3.1-1: UPF Event Exposure service for usage report procedure

- NF sends the Nupf_UsageReport_Request message to URF to request the usage report.

- The UPF acknowledge whether request sucessfully or not via Nupf_UsageReport_Response. If requst real-time report, UPF will send usage report immediately via this message. If NF request condition-based triggered report, UPF will send usage report when trigger the time or volume conditions by another message.

End of change