3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202609

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Revision of Solution #7 a PDU Session with multiple groups for KI#5

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing revision of a PDU Session with multiple groups.

1. Introduction/Discussion

It is proposed to revise the solution #7 to solve ENs.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

6.7	Solution #7: A PDU Session with multiple groups

6.7.1	Introduction

This solution is Key Issue #5 on 5G Smart Energy and Infrastructure.

Figure 6.7.1-1 depicts the concept of the solution.

Figure 6.7.1-1: Group communication enhancements for SEI (Smart Energy and Infrastructure)

Target scenario of this solution is a smart grid scenario. For example, a UE shares its phase info with the UEs in Regsion.1 for synchronization and with the UEs in Area.1 for monitoring. Region.1 is located in Aera.1. For that, a UE belongs to Region.1 group and to Area.A group. When the UE sends data to Region.1 group in higher QoS and to Area.A group in lower QoS. The other UEs which belong to Region.1 group and to Area.A group receive data in higher QoS. The UEs which belong to only Region.1 group receive the data in higher QoS. The UEs which belong to Area.A group receive the data in lower QoS. A UE which belongs to Region.1 only, sends data to UEs which belong to Region.1 group and to Area.1 group and to UEs which belong to Region.1 group. A UE which belongs to Area.A group olny sends data to UEs which belong to Region.1 group and to Area.1 group and to UEs which belong to Area.A group.

6.7.2	Functional Description

Figure 6.7.2-1 depicts the concept of the solution.

Figure 6.7.2-1: A PDU session with multiple groups

The principles of this solution are like the following. The numbers of each bullet refer to those in Figure 6.7.2-1.

- A UE or an AF requests 5GS to configure new groups with different QoS’s (0). UDM/UDR stores the group QoS information. SMF retrieves the information during PDU Session setup. AF can pre-configure the group and the QoS per group.
A UE belongs to Group1 and Group2 sends data to the UEs which belong to at least one of Group1 and Group2. A UE belongs to Group1 only sends data to UEs which belong to Group1. A UE belongs to Group2 only sends data to UEs which belong to Group2.

- A UE establish a PDU Session for the groups with explicit priorities or implicit priorities (e.g. the former of the list means the higher priority) (1). The UE provides list of group IDs with explicit priorities. For this, the UE provides an indication when it send PDU Session request to 5GC. It is assumed that the UE knows the group id via pre-configuration or via any other means.
For each group the PDU Session is accessing, there should have one QoS flow. For that, priorities among QoSs among groups are needed. It is also possible with implicit priorities when the QoS information includes priorities. Explicit priorities (e.g. using Priority Levels) are provided when the AF pre-configure the group and the QoS per group. If explicit priorities are not provided more strict requirements can be considered to have higher priorities if the same types of requirements like max-latency or min-rate. If the same types of requirements are not used, the groups specified in the earlier order of the PDU Session Request from the UE can be considered to have higher priorities.

- The SMF sets up the UPF forwarding table based on the groups and the SMF sets up the QoS flows based on the QoS requirements (2, 3, 4). For example, an UPF can be configured to forward the highest group traffic first, and then to forward the next-higher group traffic not overlapped with the previous traffic. Each UE is verified if it is allowed to communicate with a group when the PDU Session is established.

- The UE sends the traffic to the PSA UPF with the highest QoS of the groups. The UPF forwards the traffic to the UEs of a group with the QoS requirements of the group and to the other UEs of another group with the dedicated QoS for the group. (5) For the UL packet, the UE just needs to send one copy to the PDU session on the QoS flow with most strict QoS requirements, and UPF is responsible for distribution of packet to different groups with different QoS. The usage of QoS rules may also need to know priorities among QoS’s among groups. The traffic from the UE belongs to multiple groups with in the PDU Session of multiple DNNs/S-NSSAIs mapped to 5G VN groups is identified as group traffic, and is forwarded as the SMF configured the UPF for the group communication. The UE sends the traffic to the PSA UPF. Each PSA UPF of a UE with a single group forwards the traffic to the UE with the QoS of the group. Each PSA UPF of a UE with multiple groups forwards the traffic to the UE with the highest priority QoS of the groups.

Editor’s NOTE:	As different 5G VN groups are mapped to different (DNN, S-NSSAI)(s), the solution means that inter UPF communication needs to be set by SMF(s) controlling different DNN/S-NSSAI(s)between UPF(s) serving different DNN/S-NSSAI(s). it is FFs how this is realized.

6.7.3	Procedures

Figure 6.7.3.1-1 outlines the procedure for support of a PDU Session with multiple groups when there can be one UPF.

Figure 6.7.3-1: Procedure for a PDU session with multiple groups

0.	The AF configures Group1 and Group2. It also configures QoS requirements1 for Group1 and QoS requirements2 for Group2. Then, the PCF maps QoS1 and QoS2 based on the QoS requirements1 and QoS requirements2.

1.	UE1, which belongs to Group1 and Group2, sends PDU Session Request with Group1 and Group2 to the SMF. In this step, the AMF needs to select the SMF based on the information including Group1 and Group2. UE should include multiple DNNs/S-NSSAIs for a PDU Session.

NOTE: Configuration shall ensure that a SMF / UPF can support all DNNs/S-NSSAIs of the group of groups

2.	The SMF checks subscription of the UE1 with the UDM and retrieves group information for Group1 and Group2 from UDM.

2a.	The SMF associates with PCF and retrieves QoS information like QoS1 for Group1 and QoS2 for Group2.

3.	The SMF exchanges N4 controls with the UPF for Group1 and Group2.

4.	The UPF setup the forwarding rules for Group1 and Group2.

5.	The SMF sends PDU Session Establish Response for Group1 and Group2 to UE1. The SMF sets QoS for the traffic for Group1 and Group2 to QoS1 if QoS1 has higher priority than QoS2.

5a.	UE2, which belongs to Group1, establishes PDU Session for Group1 with QoS1. This includes similar steps similar to steps 1 through 5.

5b.	UE3, which belongs to Group2, establishes PDU Session for Group2 with QoS2. This includes similar steps similar to steps 1 through 5.

6.	UE1 sends traffic for Group1 and Group2 with QoS1 to the UPF.

Editor’s NOTE:	it is FFS whether any data sent to such a group (e.g. group 1) shall always be sent to the other group (e.g. group 2). if this is the case, it is questionable to create different groups for this purpose.

7.	The UPF forwards the traffic from UE1 for Group1 and Group2 to UEs belongs to (Group1 and Group2) or to Group1.

8.	The UPF sends traffic from UE1 for Group1 and Group2 to UE2 for Group1 with QoS1.

9.	The UPF forwards the traffic from UE1 for Group1 and Group2 to UEs belongs to Group2 but not belongs to Group1.

10.	The UPF sends traffic from UE1 for Group1 and Group2 to UE3 for Group2 with QoS2.

6.7.4	Impacts on existing entities and interfaces

The impacts on the existing entities are like the following.

-	UE shall include multiple DNNs/S-NSSAIs for a PDU Session.

-	AMF shall select the SMF based on the multiple DNNs/S-NSSAIs.

-	SMF shall control the UPF(s) based on the multiple DNNs/S-NSSAIs and QoS setup considering priorities among multiple groups.

-	UPF shall forwards traffic for multiple groups.

-	AMF (including NAS) PCF/CHF/RAN shall support PDU Sessions with multiple DNNs/S-NSSAIs.

# * * * * End of changes * * * *