3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-220xxxx

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	Samsung

Title:	Revision of Solution #5 Multiple SMFs involved 5G VN group communication for KI#4

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing revision of Multiple SMFs involved 5G VN group communication.

# 1. Introduction/Discussion

It is proposed to revise the solution #5 to solve ENs.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-74.

# * * * * First change * * * *

## 6.5	Solution #5: Multiple SMFs involved 5G VN group communication

### 6.5.1	Introduction

This solution is for Key Issue #4 on enhancements of 5G VN group communication.

### 6.5.2	Functional Description

The main idea of this solution is as below:

-	Each SMF serving the same 5G VN group reports/registers its identifier and the 5G VN group identifier to the Group Session Management Function. Thus, the Group Session Management Function has the knowledge of all the SMF serving the 5G VN group.

-	Each SMF can be aware of other SMFs via the Group Session Management Function as following:

-	A SMF subscribes to the Group Session Management Function for the 5G VN group change events (e.g. when another new SMF starts to serve the same 5G VN group or another SMF stops to serve the 5G VN group). When the 5G VN group change events occurs, the Group Session Management Function sends a notification including the changed SMF's identifier information to the subscribed SMF; or

-	A SMF can query the SMF information of other group member from the Group Session Management Function.

-	The SMF requests a session with another SMF and these two SMFs can establish the N19 tunnels between UPFs and configure the UPFs via the session. When the SMF detects, allocates or releases an address of the UE within the 5G VN group, the SMF sends update/release requests to the other SMF via the session to update the configuration on the UPF.

Editor's note:	it is FFS how the SMF(s) collaborate to manage the topology of N19 links (star, ring, tree, etc.).

Figure 6.5.2-1 depicts the architecture for the solution.

Figure 6.5.2-1: Architecture for multiple SMFs involved 5G VN group communication

The Group Session Management Function is supported by the UDM , as UDM already has information on SMF ID and DNN/S-NSSAI for each PDU Session. updates to Nudm will be required

### 6.5.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

### 6.5.4	Impacts on existing entities and interfaces

Editor's note:	This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *