3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202635r01

Elbonia, April 6th – 12th, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	New solution on LMF selection based on client or AF identifier

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes new solution for KI#3 Local Area Restriction for an LMF and GMLC.

1. Introduction/Discussion

This contribution is related to KI#3: Local Area Restriction for an LMF and GMLC.

LMF selection functionality is used by the AMF to determine an LMF for location estimation of the target UE. In some local scenarios, operators may deploy specific LMF and GMLC for enterprises for some customized and privacy requirement. But UE of different enterprises may share the same AMF. In order to select the right LMF, AMF shall use some new identifiers which can map the relationship between enterprises and the specific LMF.

2. Text Proposal

It is proposed to agree the following solution into TR 23.700-71.

# * * * * First change * * * *

6.X	Solution #X: LMF selection based on client or AF identifier

6.X.1	Introduction

This solution is to address KI#3: Local Area Restriction for an LMF and GMLC.

LMF selection functionality is used by the AMF to determine an LMF for location estimation of the target UE. In some local cases, operators may deploy specific LMF and GMLC for enterprises for some customized or privacy requirements. But the UEs of different enterprises may share the same AMF or AMF pool. In order to select the right LMF for specific enterprise, AMF could use new identifiers which can map between enterprise and specific LMF.

6.X.2	Functional Description

Based on the scenarios mentioned above, AMF may use client identifiers or AF identifiers as one of the additional factors during the LMF selection, as specified in clause 5.1 in TS 23.273. The client and AF request the location information of the target UE, whose identifiers are assigned by the operator. The selection policy can be configured locally at AMF, or by querying NRF.

If the LMF selection is performed at the AMF by querying NRF, LMF shall register with the client or AF identifier(s) that the LMF can serve.

6.X.3	Procedures

N/A

6.X.4	Impacts on services, entities, and interfaces

AMF:

- take AF or client identifier(s) into account during the LMF selection;

AF:

- provide the AF identifier(s) in location service procedures;

UE:

- provide the client or AF identifier(s) to AMF in location service procedures;

LMF:

- Register in NRF with serving AF or client identifier(s).

# * * * * End of changes * * * *