3GPP WG SA WG2 Meeting #150E e-meeting	S2-2002638r01

Elbonia, April 6th – 12th, 2022	(revision of S2-2xxxxxx)

Source:	China Mobile

Title:	New Solution on NWDAF-assisted application detection

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3/ Rel-18

Abstract of the contribution: This contribution proposes a new Solution to address Key Issue #2: NWDAF-assisted application detection.

# Discussion

This contribution proposes a new solution for Key Issue #2: NWDAF-assisted application detection.

# 2.	Proposal

It is propose a new solution on Key Issue #2: NWDAF-assisted application detection.

* * * * First Change * * * *

## 6.0 Mapping Solutions to Key Issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions |  | 2 NWDAF-assisted application detection |  |  |
| X |  | X |  |  |

* * * * Second Change * * * *

## 6.X	Solution #X: NWDAF-assisted application detection

### 6.X.1	Description

This solution is proposed to address Key Issue #2: NWDAF-assisted application detection.

To differentiate an application traffic handling, the first step is to distinguish application traffic from other traffics. In 5G network, an application can be distinguished by a set of packet headers (SDFs) or application IDs. The most common approach to detect traffic is a set of packet headers, which usually contains source and destination address. The application ID can be used for referring the UPF's specific application detection filter. For managing detection information associated with application IDs, AF may provide PFDs according to clause 4.18 of TS 23.502 [3] to update application detection filter information. A PFD contents could contain flow description (service-side 3-tuple), URL, and domain name/protocol information. When an AF deliver PFD to NEF (PFDF), it will be distributed to SMFs and UPFs to enable flow detection according to clause 5.8.2 of TS 23.501 [2].

However, as indicated by the KI description, the ASP may provide an initial PFD information but does not update it in time or does not update it anymore, then it is unknown how the UPF detect the application traffic.

The objective of this solution is to automate application detection method using PFDs for newly/updated applications by extracting network traffic characteristics using the network data analytics. To realize this solution, one of the main features is 1) collect measurement of an application to extract statistical characteristics, 2) collect payload of packets to extract payload characteristics (such as domain name contained in the payload). The baseline usage of the generated analytics is to store the captured application characteristics as a PFD, and the PFD is used by SMF and UPF to detect an application defined as TS 23.502 [3].

In details, usually, although the IP 3-tuple for an application is changeable, Traffic Characteristics /Feature for the application is unchangeable. The NWDAF could collect the historical PFD information (including Application ID, historical IP 3-tuple, historical URL, historical Domain name information) from the NEF (PFDF) for applications.

Based on historical PFD information from ADRF and traffic information from UPF, new PFD information (including Application ID, new IP 3-tuple, new URL, new Domain name information) for the existing Application ID could be derived by the NWDAF and provisioned to the NEF as analytics output. The new PFD information provided by the NWDAF should be also stored into the UDR. Then the new PFD information could be used by the NEF to be used for PFD provisioning.

Editor's note:	It is FFS on how to handle the overlapping issue between the PFD information provided by NWDAF and the PFD information provided by AF.

The below is the principles of the solution:

-	A consumer (e.g. NEF) requests to the NWDAF to provide PDFs for newly detected  applications,.

-	The NWDAF requests to provide SDF that does not match a installed PDR at the SMF/UPF.

-	The NWDAF analyses the collected data to generate unique traffic patterns, and provides it to the consumer (i.e. NEF).

-	The consumer i.e NEF stores PFD associated with an application ID to enable the detection of application traffic in the future (out of scope).

### 6.X.2	Input Data

NWDAF collects QoS flow related data from SMF for a specific S-NSSAI, DNN, and UE. TThe detailed data are described in Table 6.x.2-1.

Table 6.x.2-1: input data to detect new application from NFs

| Information | Source | Description |
| --- | --- | --- |
| SUPI | SMF / UPF | UE ID for the UE who uses the application. |
| S-NSSAI | SMF | Identifies the Network Slice for which analytics information is provided. |
| DNN | SMF | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Start/end time | UPF | Start and end time of traffic detection |
| Data volume | UPF | Measured data traffic volume (per UL/DL) |
| Data duration | UPF | Measured average data traffic (packets) duration (per UL/DL) |
| QoS flow Bit Rate | UPF | The observed bit rate (per UL/DL) |
| Packet transmission | UPF | The observed average number of packet transmission (per UL/DL) |
| IP 3-tuple | UPF | to Identify a service flow of the UE who uses the application |
| Packet's URL | UPF | URL is extracted from the inspected user plane packets. |
| Packet's Domain Name | SMF/UPF | The domain name is extracted from the inspected user plane packets in a QoS flow |
| Size of packets | SMF/UPF | Average size of packets |
| PFD Information (NOTE 1) | ADRF | Historical PFD Information stored in the NEF (PFDF) as defined in clause 6.1.2.3.2, TS 23.503 [4] |
| > Application ID | ADRF | Identification of the application that refers to one or more application defection filters. |
| > IP 3-tuple | , ADRF | Including protocol, server side IP address and port number |
| > URL | ADRF | the significant parts of the URL to be matched, e.g. host name |
| > Domain Name | ADRF | a Domain name matching criteria and information about applicable protocol(s) |

NOTE:	The PFD Information could be used to train an Application Classifier for all the Applications or extract one or multiple Traffic Characteristics/Feature per Application ID. Then the Application Classifier or the Traffic Characteristics/Feature could be used to derive new PFD information for the existing Application ID in the PFD information. However, these implementation details are out of 3GPP scope.

### 6.X.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.x.3-1.

The output analysis can be used to provision new PFDs for existing applications, or define new PFDs for new applications, not known to NEF yet. With the knowledge on an application's URL, domain name, and IP-address, the NEF can assign an PFD ID for the unknown application, and the assigned PFD ID can be used to identify/audit/report the unknown application.

Table 6.x.3-1: Output for new application detection

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided |
| Flow descriptor | Flow descriptor containing 3-tuple, server side (destination address, port, and protocol) |
| URL | the significant parts of the URL to be matched, e.g. host name defined in |
| Domain name information | a Domain name matching criteria and information about applicable protocol(s). |

### 6.X.4	Procedures

For the procedure to extract the unique characteristic of application traffic such as traffic pattern, URL, and/or domain name related information. The procedure depicted in 6.x.4-1 shows new application detection analytics and store as PFDs by NEF.

Figure 6.x.4-1: A procedure to extract application characteristics

1.	The consumer NF subscribes to the NWDAF to obtain application detection information. This subscription maybe is triggered by local configuration or OAM. The Analytics Filter Information may optionally including the UE ID, S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information from NEF(PFD) ad historical data from ADRF (not shown in the figure)

3.	The NWDAF collects session related information from the UPF about URL, Domain name part, and IP 3-tuples of packets from the SDF not matching installed PDRs. .

4.	The NWDAF derives PFD analytics.

5.	The NWDAF notifies the analytics consumer NF with outputs.Nnwdaf_AnalyticsSubscription_Subscribe

### 6.X.3	Impacts on Existing Nodes and Functionality

NWDAF:

-	Needs to provide a new analytic for application detection information.

-	Needs to process user plane data for extracting traffic characteristics.

UPF:

-	Needs to report URL, Domain name part, and IP 3-tuples of packets from unknown application to NWDAF.

-	Needs to provide requested sessions statistics including data volume, data duration, QoS flow bit rate and packet transmission.

NEF (PFDF):

-	Supports to consume new PFD information from NWDAF.

* * * * End of change * * * *