SA WG2 Meeting #150e	S2-2202647r01

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	New Solution on UPF event exposure to NWDAF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF event exposure to NWDAF.

1 Discussion

The solution proposal presented here addresses the key issue proposal in S2-2201823.

This paper proposes to support the UPF data exposure to NWDAF. NWDAF first subscribes to SMF on UPF data with Target Notification Address referring to NWDAF address, then UPF report to NWDAF directly on the requested information.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: UPF event exposure service to NWDAF |  | X |

****************************Second change  ****************************

6.X	Solution #X: UPF event exposure service to NWDAF

6.X.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

6.X.2	Description

The Annex A in TR 23.700-62 v0.1.0 has analysed the NWDAF requirements of UPF event exposure, which contains seven aspects of information. In this solution, the NWDAF subscribes to the UPF data via the SMF, and the UPF directly sends the collected UPF data to NWDAF.

6.X.3	Procedures

Figure 6.X.3-1: UPF Information Exposure to NWDAF via SMF

1.	The NWDAF invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID and NF type.

2.	The UDM provides a Nudm_UECM_Get response to the NWDAF with the corresponding SMF.

3.	The NWDAF sends the request to the SMF to subscribe UPF data, including the following information:

-	Notification Target Address (NWDAF address);

-	Event Filter Information: S-NSSAI, Application Id, Area of Interest;

-	Target of Event Reporting: a UE or a group of UEs or any UE;

-	Subscription Information:

-  UL/DL Throughput, UL/DL packets or number of connections;

-  N3 delay, N6 delay, E2E delay, UL/DL packet loss, or UL/DL packet retransmission;

-  UL/DL Data Volume;

- The SMF sends the request to the UPF over N4_Session_Modification_Request/Response message including the NWDAF address, Event Filter Information, Target of Event Reporting, and Target Subscription Information.

5.	The UPF responds with the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the NWDAF.

**End of change**