3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202668r02

Elbonia, Apr 6th – 10th, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eNS_Ph3 New Solution to KI#1

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper introduces a new solution to key issue #1

1. Introduction/Discussion

This paper propose a new solution to KI#1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.1.0.

# * * * * First change * * * *

6.X	Solution #X: Additional S-NSSAI associated with the PDU session

6.X.1	Introduction

This solution aims to address the key issues#1: Support of Network Slice Service continuity, particularly it reuses the concept that the PDU session can be associate with two S-NSSAIs. When one S-NSSAI is congested, the network can associate the PDU session with additional S-NSSAI in order to provide service continuity during mobility scenario.

NOTE: This solution may be extended to cover no mobility scenarios 1b and 1c described in clause 5.1.1

6.X.2	Functional Description

In this solution, a network slice instance may be associated with multiple S-NSSAIs in the PLMN. The AMF is configured with which S-NSSAIs are associated with the network slice instance. When the UE establishes PDU session it determines the requested S-NSSAI based on the URSP rules. The PDU Session is initially associated with this requested S-NSSAI. When this requested S-NSSAI become congest, the network may associate the PDU session with an additional S-NSSAI which can be associated with the network slice instance. After the S-NSSAI congestion is mitigated, the network may remove the additional S-NSSAI of the PDU Session.

When the AMF determines to associate the PDU session with an additional S-NSSAI, it provides the additional S-NSSAI to SMF. The SMF uses this additional S-NSSAI in the N2 information and send it to NG-RAN. The SMF may also send the additional S-NSSAI to the UPF.

This additional S-NSSAI is not provided to the UE, so from UE perspective the PDU session is still associated with the original requested S-NSSAI and therefore no impact on the UE side. Since the PDU session is still associated with the original S-NSSAI, the anchor SMF/UPF and the IP address is not changed so service continuity is ensured.

NOTE: When the AMF detects that the original requested S-NSSAI is not congested, the AMF may remove the additional S-NSSAI associated with the PDU session.

6.X.3	Procedures

The AMF is configured that both S-NSSAI#1 and S-NSSAI#2 are associated with one network slice instance.

1.	The UE establishes a PDU session with the requested S-NSSAI#1 via S-RAN.

2.	S-RAN performs a UE measurement and determines that a handover to the T-RAN is needed. The S-RAN sends a handover required message(T-RAN node, a source to target transparent container, an SM N2 information list, PDU session ID(s)) to the AMF via the N2 interface.

3.	If the AMF is notified that the S-NSSAI#1 is under congestion and the service continuity of the PDU session is required, the AMF checks if the S-NSSAI#2 is supported in the T-RAN node. If it is supported the AMF determines to associate the PDU session with additional S-NSSAI#2. In case of inter AMF mobility, the target AMF determines whether to associate this PDU session with the additional S-NSSAI#2.

4.	The AMF sends an Nsmf_PDUSession_UpdateSMContext request message (the PDU session ID, the S-NSSAI#2, N2 SM Information) to the SMF. The SMF associates the S-NSSAI#2 with the PDU session.

5.	The SMF may modify the N4 session to update the network slice information in the UPF.

6.	The SMF sends a Nsmf_PDUSession_UpdateSMContext response message (PDU session ID, N2 SM Information) to the AMF. The N2 SM information includes the S-NSSAI#2 received from the AMF.

7.	The rest steps in N2 based handover procedure are performed.

6.X.4	Impacts on existing entities and interfaces

Editor's Note: This clause lists impacts to existing entities and interfaces.

# * * * * End of changes * * * *