3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202668r09

Elbonia, Apr 6th – 10th, 2022	(revision of S2-220xxxx)

Source:	ZTE

Title:	FS_eNS_Ph3 New Solution to KI#1

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: This paper introduces a new solution to key issue #1

1. Introduction/Discussion

This paper propose a new solution to KI#1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41 V0.1.0.

# * * * * First change * * * *

6.X	Solution #X: Additional S-NSSAI associated with the PDU session

6.X.1	Introduction

This solution aims to address the key issues#1: Support of Network Slice Service continuity, particularly it reuses the concept that the PDU session can be associate with two S-NSSAIs (based on values provided in mapping of Allowed NSSAI). The scenario covers the case where NFs (SMF, UPF...) in a network slice instance support multiple S-NSSAIs. How to split the resource depends on implementation. It is possible that the resource of one S-NSSAI is getting congested, while other S-NSSAI is not (e.g. there is a fixed allocation of resources to each S-NSSAI in the NF instances). This solution proposes that when one S-NSSAI is congested, the network can associate the PDU session with additional S-NSSAI in order to provide service continuity during mobility scenario, so the resource allocated in SMF and/or UPF for one S-NSSAI can be shared by other S-NSSAI if other S-NSSAI is getting congested.

Editor's note:	The validity of the scenario that this solution covers is to be further assessed.

Editor’s Note: It is FFS whether and how a network slice instance is associated with multiple S-NSSAIs.

Editor’s Note: It is FFS whether the PDU session can be associated with more than one S-NSSAIs or not

NOTE: This solution may be extended to cover no mobility scenarios 1b and 1c described in clause 5.1.1

6.X.2	Functional Description

In this solution it is assumed that a network slice instance may be associated with multiple S-NSSAIs in the PLMN, and each S-NSSAI may use different network resources. The AMF,SMF and UPF is configured with which S-NSSAIs are associated with the network slice instance. When the UE establishes PDU session it determines the requested S-NSSAI based on the URSP rules. The PDU Session is initially associated with this requested S-NSSAI. When this requested S-NSSAI become congest, the network may associate the PDU session with an additional S-NSSAI which can be associated with the network slice instance. After the S-NSSAI congestion is mitigated, the network may remove the additional S-NSSAI of the PDU Session.

When the AMF determines to associate the PDU session with an additional S-NSSAI, it provides the additional S-NSSAI to SMF. The SMF uses this additional S-NSSAI in the N2 information and send it to NG-RAN. The SMF may also send the additional S-NSSAI to the UPF.

This additional S-NSSAI is not provided to the UE, so from UE perspective the PDU session is still associated with the original requested S-NSSAI and therefore no impact on the UE side. Since the PDU session is still associated with the original S-NSSAI, the anchor SMF/UPF and the IP address is not changed so service continuity is ensured.

For PDU Session with home routed scenario, the AMF may determine to associate this PDU session with additional S-NSSAI in the VPLMN. The AMF doesn’t update the slice mapping between VPLMN and HPLMN in the UE.

NOTE: When the AMF detects that the original requested S-NSSAI is not congested, the AMF may remove the additional S-NSSAI associated with the PDU session.

6.X.3	Procedures

The target AMF is configured that both S-NSSAI#1 and S-NSSAI#2 are associated with one network slice instance.

1.	The UE establishes a PDU session with the requested S-NSSAI#1 via S-RAN.

2.	S-RAN performs a UE measurement and determines that a handover to the T-RAN is needed. The S-RAN sends a handover required message(T-RAN node information, a source to target transparent container, an SM N2 information list, PDU session ID(s)) to the AMF via the N2 interface.

3.	The source AMF selects a target AMF based on the T-RAN node information and create UE context in the target AMF.

4.	If the target AMF is notified that the S-NSSAI#1 is under congestion in the CN and the service continuity of the PDU session is required, the target AMF checks if the S-NSSAI#2 is supported in the T-RAN node. If it is supported the target AMF determines to associate the PDU session with additional S-NSSAI#2.

5.	The target AMF sends an Nsmf_PDUSession_UpdateSMContext request message (the PDU session ID, the S-NSSAI#2, N2 SM Information) to the SMF. The SMF associates the S-NSSAI#2 with the PDU session.

6.	The SMF may modify the N4 session to update the network slice information in the UPF.

7.	The SMF sends a Nsmf_PDUSession_UpdateSMContext response message (PDU session ID, N2 SM Information) to the target AMF. The N2 SM information includes the S-NSSAI#2 received from the AMF.

8.	The rest steps in N2 based handover procedure are performed.

6.X.4	Impacts on existing entities and interfaces

Editor's Note: This clause lists impacts to existing entities and interfaces.

AMF

SMF

UPF

NG-RAN (TBD)

Editor's note: detailed list of impacts is FFS.

# * * * * End of changes * * * *