3GPP TSG-WG SA2 Meeting #150E e-meeting 	S2-2202717

Elbonia, April 6th – 14th, 2022	(revision of S2-220xxxx)

Source:	CATT

Title:	New Solution for KI#3: Non-GBR QoS Notification Control Report

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: Proposes the Non-GBR QoS Notification Control Report to help the codec changes in the AF

1.  Non GBR QoS Flow for XRM service Stream

In the Pre-R18, the QoS Notification Control (QNC) w/o alternative QoS profile are defined in clause 5.7.2.4 of TS 23.501.

The QNC and alternative QoS profile is only defined for the GBR-type QoS Flow. Since the QNC notification is defined as "GFBR can no longer be guaranteed" or "GFBR can be guaranteed again", only the GBR-type QoS Flow is associated with GFBR, and Non-GBR QoS Flow is not associated with GFBR.

One of the key XRM service is the cloud gaming, cloud gaming is the consumer type application with very high bitrate, and normally, consumer type application with very high bitrate are not allocated with GBR type QoS Flow, because 1) there are too much consumer users, and the 5G network cannot provide such large number of GBR type QoS Flow with high bitrate. 2) GBR type QoS Flow normally needs the 5GS network to reserve some network resources especially for the expensive radio resources. So the GBR type QoS Flow is more expensive than the Non-GBR QoS Flow and the GBR QoS Flow can provide more valuable incoming for the operator, and Business application can offer such expensive GBR type QoS Flow while the normal consumer users are not willing to pay such high rate credit GBR type QoS.

All-in-all, it is assumed that the some XRM service streams are carried on the non-GBR QoS Flows. In such case, the R18 needs to study the QNC and alternative QoS profile mechanism for the GBR QoS Flow to be used in the non-GBR QoS Flow.

2. Non-GBR QoS Notification Control Report

If the non-GBR QoS Flow is used for the XRM service stream, the QNC similar Notification control mechanism is needed for the non-GBR QoS Flow.

Since there is no explicit bitrate defined for the non-GBR Flow, but all the non-GBR Flows share the APN-AMBR and UE-AMBR, i.e. in fact, there is mean bit rate available for the non-GBR flow. The mean bit rate is the bitrate with a time windows, e.g. averaging windows, i.e. the (default) averaging windows is also applicable to the Non-GBR 5QI. If the change of the mean bitrate is over a threshold, the RAN will provide a non-GBR QoS Notification Control Report message that "The mean bitrate has changed to x kbps" to the SMF, PCF and AF (via NEF), the AF can change the codec based on the report x kbps bitrates.

To control the reporting rates, there are two possible way. First one is that the SMF can provide different bitrate span to the RAN, e.g. the three level bitrate span are 0~500kbps, 500~2500kbps, above 2500kpb ~ Ͼ. Only after the mean bitrate is changed from one bitrate level to another bitrate level, the RAN can generate the non-GBR QoS Notification Control Report Message.  The second on is that the SMF can provide different bitrate changes percentage to RAN, only after the mean bitrate is changed UP/Down above the change percentage, the RAN will generate the non-GBR QoS Notification Control Report message.  One example of 3 level bitrate change percentage can be 50%, 100%, 200%.

Like the alternative QoS, AF can provide different level mean bitrate (the absolute mean bitrate value, or the bitrate change percentage) to the PCF, and PCF generates an 5QI with different level mean bitrates and associated average window to the SMF, then the SMF provides the QoS profile to the RAN with an 5QI with different level mean bitrates and associated average window. When the mean bitrate is changed to different bitrate, the RAN generates the non-GBR QoS Notification Control as described above.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-60.

# * * * * First change (all new texts)* * * *

6. X	Solution #x –Mean bit rate change report

6. X.1	Key Issue mapping

This solution is for KI #3 5GS information exposure for XR/media Enhancements.

6. X.2	Description

In the Pre-R18, the QoS Notification Control (QNC) with and without alternative QoS profile are defined in clause 5.7.2.4 of TS 23.501.

The QNC and alternative QoS profile is only defined for the GBR-type QoS Flow. Since the QNC notification is defined as "GFBR can no longer be guaranteed" or "GFBR can be guaranteed again", only the GBR-Type QoS Flow is associated with GFBR, and Non-GBR QoS Flow is not associated with GFBR.

If the non-GBR QoS Flow is used for the XRM service stream, the QNC similar Notification control mechanism is needed for the non-GBR QoS Flow.

Since there is no explicit bitrate defined for the non-GBR Flow, but all the non-GBR Flows share the APN-AMBR and UE-AMBR, i.e. in fact, there is mean bit rate available for the non-GBR flow. The mean bit rate is the bitrate with a time windows, e.g. averaging windows, i.e. the (default) averaging windows is also applicable to the Non-GBR 5QI. If the change of the mean bitrate is over a threshold, the RAN will provide a non-GBR QoS Notification Control Report Message that "The mean bitrate has changed to x kbps" to the SMF, PCF and AF (via NEF), the AF can change the codec based on the report x kbps bitrates.

Editor’s note: how to define the averaging window is FFS. And how the RAN identifies the change of the mean bitrate over a threshold is FFS.

To control the reporting rates, there are two possible way. First one is that the SMF can provide different bitrate span to the RAN, e.g. the three level bitrate span are 0~500kbps, 500~2500kbps, above 2500kpb ~ Ͼ. Only after the mean bitrate is changed from one bitrate span to another bitrate span, the RAN can generate the non-GBR QoS Notification Control Report Message.  The second on is that the SMF can provide different bitrate changes percentage to RAN, only after the mean bitrate is changed UP/Down above the change percentage, the RAN will generate the non-GBR QoS Notification Control Report message.  One example of 3 level bitrate change percentage can be 50%, 100%, 200%.

Like the alternative QoS, AF can provide different level mean bitrate (the absolute mean bitrate value, or the bitrate change percentage) to the PCF, and PCF generates an 5QI with different level mean bitrates and associated average window to the SMF, then the SMF provides the QoS profile to the RAN with an 5QI with different level mean bitrates and associated average window. When the mean bitrate is changed to different bitrate, the RAN generates the non-GBR QoS Notification Control as described above.

6. X.3	Procedures

6. X.3.1	Non-GBR QoS Notification Control Report

Figure 6.x.3.1-1: Non-GBR QoS Notification Control Report

- The PDU Session for the XRM service is established, and a non-GBR QoS Flow is established for the XRM service stream.

- The RAN detects the mean bitrate of the non-GBR QoS Flow to be changed to another level and sends a N2 QoS Notification Message "the mean bitrate of the Non-GBR QoS Flow changes to x kbps" or "the mean bitrate of the non-GBR QoS Flow increases or decreases x%" to the AMF. The multiple levels of mean bitrate can be provisioned to the RAN by the SMF during the Non-GBR QoS Flow Establishment procedure or during the Non-GBR QoS Flow Modification procedure. The multiple levels of mean bitrate can be bitrate value in kbps or can bitrate rate change percentage.

- The AMF sends Nsmf_PDUSession_UpdateSM Context request ("the mean bitrate of the Non-GBR QoS Flow changes to x kbps" or "the mean bitrate of the non-GBR QoS Flow increases or decreases x%") to the SMF.

- The SMF sends Npcf_SMPolicyControl_Update request ("the mean bitrate of the SDF changes to x kbps" or "the mean bitrate of the SDF increases or decreases x %") to the PCF.

- If NEF is used, the PCF sends the Npcf_PolicyAuthorization_Notify ("the mean bitrate of the SDF changes to x kbps" or "the mean bitrate of the SDF increases or decreases x %") to the NEF.

If the AF locates the trusted domain and the NEF is not used, the PCF sends the Npcf_PolicyAuthorization_Notify ("the mean bitrate of the SDF changes to x kbps" or "the mean bitrate of the SDF increases or decreases x %") to the AF.

- After receiving the Npcf_PolicyAuthorization_Notify from the PCF, the NEF sends the Nnef_EventExposure ("the mean bitrate of the stream changes to x kbps" or "the mean bitrate of the stream increases or decreases x %") to the AF.

- After receiving the information from PCF/NEF, the AF can determine to change the codec with indicated bitrate.

6. X.3.1	Non-GBR QoS Notification Control Report

- The PDU Session for the XRM service is established.

- If the XRM AF does not locate at trusted domain, the AF sends the Nnef_AFsessionwithQoS_Create Request (XRM stream information and request QoS and additional multiple QoS levels) to the NEF. The additionl multiple QoS levels include the different bitrates value or different bitrate change percentage.

If the XRM AF locates at trusted domain, the AF sends the Npcf_PolicyAuthorization_Create Request (XRM stream information and request QoS and additional multiple QoS levels) to the PCF.

- If the NEF is used, the NEF sends The AMF sends ends the Npcf_PolicyAuthorization_Create Request (XRM stream information and request QoS and additional multiple QoS levels) to the PCF as described in step 1b.

- The PCF sends Npcf_SMPolicyControl_UpdateNotify request (SDF PCC rule) to the SMF. The SDF PCC rule includes the current mean bitrate and additional multiple mean bitrate levels.

- The SMF creates a new non-GBR QoS Flow based on received PCC rule and sends the Namf_Communication_N1N2MessageTransfer request (N1 Message, N2 Message (QFI, non-GBR QoS profile with the 5QI , ARP and additional multiple QoS levels)). The additionl multiple QoS levels include the different bitrates value or different bitrate change percentage.

- AMF sends the N2 Message (QFI, non-GBR QoS profile with the 5QI, ARP and additional multiple QoS levels)) to the RAN to create a non-GBR QoS Flow. After the Non-GBR QoS Flow is established. The RAN will generate the current available mean bitrate to the AMF as described in clause 6.x.3.1 and the AF may change its codec based on the mean bitrate reported from the 5GS.

After the mean bitrate is changed, the RAN generates the non-GBR QoS Flow Notification message as described in clause 6.x.3.1 and the AF can change its codec based on mean bitrate reported from the 5GS.

6. X.4	Impacts on services, entities and interfaces

AF:

- Provides the stream QoS requirements which include additional multiple QoS levels

- Change its codec based on the mean bitrate reported from 5G network.

PCF:

- The PCC rules for the non-GBR 5QI include additional multiple QoS levels

- Forwards the current mean bitrate from the SMF to the AF (via NEF)

SMF:

- Creates a non-GBR QoS Flow with additional multiple QoS levels based on the PCC rules from PCF.

- Forwards the current mean bitrate from the RAN to the PCF.

RAN:

- Creates a non-GBR QoS Flow with additional multiple QoS levels in the QoS profile.

- Measures and reports the current mean bitrate to the SMF.

# * * * * End of change* * * *