SA WG2 Meeting #150e	S2-2202733r01

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	New Solution: Support UPF event exposure service to NWDAF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for UPF Event Exposure service to NWDAF.

1 Discussion

The solution proposal presented here addresses the key issue proposal in S2-2201823.

This paper proposes to support the UPF exposes information to NWDAF directly to facilitate the NWDAF providing data analytics with low latency. The current mechanism which the NWDAF should subscribe from SMF->UPF will cost a long time.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: UPF event exposure service to NWDAF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: UPF event exposure service to NWDAF

6.X.1	Description

The Annex A in TR 23.700-62 v0.1.0 has analysed the NWDAF requirements of UPF event exposure service, which contains the following seven aspects of information:

1.	QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information), including QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission, and Packet retransmission.

2.	Data collected by NWDAF for UPF load analytics (i.e., Traffic usage report), and service data from 5GC related to UE communication (e.g., UE communications, N4 Session ID, and Inactivity detection time).

3.	Data Collected from the UPF or from the AF related to User Data Congestion Analytics, e.g., Application ID, IP Packet Filter Set, Measurement period, Throughput UL/DL, Throughput UL/DL (peak), Timestamp, and Achieved sampling ratio.

4.	UE data volume dispersion collected from serving UPF, e.g., UE UP address, Timestamp, Data Volume UL/DL, Application ID, IP 5-tuple, Location of Application, and Application duration.

5.	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific serving anchor UPF/DNAI/Edge Application Server Instance.

6.	Data collected by NWDAF for WLAN performance analytics, e.g., UE communications.

7.	Data collected by NWDAF for Redundant Transmission Experience analytics, e.g., UL/DL packet delay measurement per GTP level.

6.X.2	Procedures

6.X.2.1	Procedure of exposing collected data from UPF to NWDAF with information of SUPI, S-NSSAI, and DNN

For the analytics targeting PDU session related information of a specific UE, e.g., QoS flow level Network Data, UE data volume dispersion, User plane performance analytics for a specific Edge Computing application, the NWDAF can find the UPF by using SUPI, S-NSSAI, and DNN via UDM and SMF, as described in 6.1.

For the analytics targeting any UE (possibly for specific DNN and or slices), e.g., Data collected by NWDAF for UPF load analytics, User Data Congestion Analytics, WLAN performance analytics, Redundant Transmission Experience analytics, the NWDAF can find the UPF by using S-NSSAI, DNN, or DNAI from NRF that has the UPF registration information, as described in other solutions.

Figure 6.X.2.1-1: UPF Information Exposure to NWDAF

1.	The NWDAF find the UPF according to the specific use case.

For the analytics targeting PDU session related information of a specific UE, e.g., QoS flow level Network Data, UE data volume dispersion, User plane performance analytics for a specific Edge Computing application, the NWDAF can find the UPF by using SUPI, S-NSSAI, and DNN via UDM and SMF, as described in 6.1.

For the analytics targeting any UE (possibly for specific DNN and or slices), e.g., Data collected by NWDAF for UPF load analytics, User Data Congestion Analytics, WLAN performance analytics, Redundant Transmission Experience analytics, the NWDAF can find the UPF by using S-NSSAI, DNN, or DNAI from NRF that has the UPF registration information, as described in 6.1.

2.	The NWDAF sends the request for requesting the collected data over Nupf_EventExposure_Subscribe service operation to the UPF.

3.	The UPF responds the requested collected data over Nupf_EventExposure_Notify service operation to the NWDAF.

**End of change**