SA WG2 Meeting #150e	S2-2202740r02

April 6th – 12th, 2022	  (revision of S2-220xxxx)

Source: 	China Mobile

Title: 	New Solution: Support Direct PFD Provisioning to UPF

Document for: 	Approval

Agenda Item: 	9.25

Work Item / Release:	FS_UPEAS/ Rel-18

Abstract of the contribution: Solution proposal for provisioning PFD information from the NEF to the UPF directly.

1 Discussion

The solution proposal presented here addresses the key issue proposal in S2-2201823.

This paper proposes to support the NEF provisions PFD to the UPF directly with low latency.

2 Proposal

It is proposed to update TR 23.700-62 v0.1.0 as follows.

****************************First change****************************

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1: UPF event exposure service framework enhancements to support registration, deregistration and discovery via NRF | X |  |
| #2: Direct PFD Provisioning to UPF |  | X |

****************************Second change (all text is new) ****************************

6.X	Solution #X: Direct PFD Provisioning to UPF

6.X.1	Description

The packet flow description (PFD) from NEF to UPF is transmitted transparently via SMF, which is not handled by the SMF. By provisioning this information directly from NEF to UPF, the UPF can obtain the PFD with low latency.

Since the N4 PFD management procedure is a node level procedure, i.e. independent of any PDU Session as described in TS 23.502 [3], the PFD may provisioned to UPF directly from the NEF by passing the SMF which is not aware of the PFD. The UPF may support Nupf_ParameterProvision service operation.

Table 6.X.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_ParameterProvision | Create | Request/Response | NEF, PCF |
|  | Update | Request/Response | NEF, PCF |
|  | Delete | Request/Response | NEF, PCF |
|  | Get | Request/Response | NEF, PCF |

This service is for allowing consumer NFs to provision of information which can be used for the UPF. There are four operations for this service:

-	Nupf_ParameterProvision_Create

-	Nupf_ParameterProvision_Update

-	Nupf_ParameterProvision_Delete

-	Nupf_ParameterProvision_Get

Editor’s note: Whether the UPF Parameter Provision service is in the scope of this SID is FFS.

6.X.2	Procedures

Figure 6.X.2-1: Direct PFD provisioning to UPF

1.	The AF issues an Nnef_EventExposure_Subscribe (Application ID, UE ID) service operation to request QoS profile from the NEF.

2.	The NEF finds the appropriate UPF by using the S-NSSAI and DNN information, as described in other solutions.

3.	The NEF(PFDF) sends a PFD Provision request to the UPF over Nupf_ParameterProvision_Create (UE IP, SUPI/SUCI, PFD, Application Identifier) service operation to provision the PFD(s) to UPF directly.

4.	The UPF updates the PFD(s) according to the request and acknowledges by responding with the Nupf_ParameterProvision_Create response message.

**End of change**