SA WG2 Meeting #150e	S2-2202741r07

# April 6 – 12, 2022; Elbonia

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0056 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Parameter supplement of EDI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Ericsson, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2022-3-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This document adds the DNAI(s) in the EAS Deployment Information to clarify for which DNAI(s) the DNS server(s) can be used. It is assumed that the AF knows the DNAI(s) in which the EAS is reachable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the DNAI(s) in the EAS Deployment Information to clarify for which DNAI(s) thed DNS server(s) can be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear for which DNAI(s) the DNS server(s) can be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional] NOTE: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN or deployed for each DNAI. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(s) of the EASs in the local DN for each DNAI. [optional] |

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF.

-	The procedure for BaselineDNSPattern management in the EASDF.

End of changes