SA WG2 Meeting #150e	S2-2202745r01

# April 06 – 12, 2022; Elbonia               		            (revision of S2-220xxxx)

Source: 	Lenovo, Motorola Mobility, Broadcom

Title: 	Solution #X: MPQUIC steering functionality using IP proxying over HTTP

Document for: 	Approval

Agenda Item: 	9.3

Work Item / Release:	FS_ATSSS_Ph3 / Rel-18

Abstract of the contribution:

# Proposal

It is proposed to add the following solution into TR 23.700-53, associated with KI#2.

Start of Changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.700-93: "Study on access traffic steering, switch and splitting support in the 5G System (5GS) architecture; Phase 2".

[6]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[7]	IETF RFC 9001: "Using TLS to Secure QUIC".

[8]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[9]	IETF RFC 9221:"An Unreliable Datagram Extension to QUIC".

[10]	draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	IETF RFC 4340: "Datagram Congestion Control Protocol (DCCP)".

[12]	draft-ietf-tsvwg-multipath-dccp: "DCCP Extensions for Multipath Operation with Multiple Addresses".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[13]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting (ATSSS); Stage 3".

[14]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3".

[15]	IETF RFC 4336: " Problem Statement for the Datagram Congestion Control Protocol (DCCP)".

[16]	draft-amend-tsvwg-multipath-framework: "A multipath framework for UDP traffic over heterogeneous access networks".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[17]	IEEE AINAW, "Out-of-Order Transmission for In-Order Arrival Scheduling for Multipath TCP", DOI:10.1109/WAINA.2014.122.

[18]	draft-amend-iccrg-multipath-reordering: "Multipath sequence maintenance".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[19]	IETF RFC 4341: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion Control ID 2: TCP-like Congestion Control".

[20]	IETF RFC 4342: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion Control ID 3: TCP-Friendly Rate Control (TFRC)".

[21]	IETF RFC 5622: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion ID 4: TCP-Friendly Rate Control for Small Packets (TFRC-SP)".

[22]	draft-romo-iccrg-ccid5: "Profile for Datagram Congestion Control Protocol (DCCP) Congestion Control ID 5".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[23]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[24]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[25]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[26]	3GPP TR 23.793: "Study on access traffic steering, switch and splitting support in the 5G System (5GS) architecture".

[x1]	draft-ietf-masque-connect-ip: "IP Proxying Support for HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[x2]	draft-ietf-quic-http: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[x3]	draft-ietf-masque-h3-datagram: "Using Datagrams with HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[x4]	draft-ietf-httpbis-h3-websockets: "Bootstrapping WebSockets with HTTP/3".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

Next Change

## 6.X	Solution #X: MPQUIC steering functionality using IP proxying over HTTP

### 6.X.1	Introduction

The solution in clause 6.X specifies a new ATSSS steering functionality, called Multipath QUIC-IP (MPQUIC-IP) steering functionality, and addresses the objective of KI#2 for a QUIC-based steering functionality. It borrows several aspects from solutions studied in TR 23.700-93, mainly from Solution #6 "MPQUIC-LL Steering Functionality" and from Solution #14 "Proxy based solution using MP-QUIC".

The solution is primarily based on the draft-ietf-masque-connect-ip [x1], which specifies how IP traffic can be transferred between a client (UE) and a proxy (UPF) using the HTTP/3 protocol [x2]. The HTTP/3 protocol operates on top of the QUIC protocol [6], which supports simultaneous communication over multiple paths, as defined in draft-ietf-quic-multipath [10].

### The solution supports two modes for transmitting an IP flow: a datagram transport mode or a stream transport mode. The datagram transport mode provides unreliable transport and may result in data delivery with out-of-order packets and/or with duplicated packets. The stream transport mode provides reliable transport and supports data delivery without out-of-order packets and without duplicated packets. The stream transport mode is particularly useful for supporting per-packet splitting, i.e., when the packets of an IP flow are split across multiple accesses and may be received out-of-order. Further details are provided below.

### 6.X.2	High-level Description

The key principles of the solution are summarized below.

-	After the MA PDU Session establishment, the UE creates one or more multipath QUIC connections with the UPF. Each multipath QUIC connection is associated with a QoS flow, i.e., it carries the traffic mapped to a QoS flow.

-	The UE operates as a connect-ip client and the UPF operates as a connect-ip proxy, both defined in draft-ietf-masque-connect-ip [x1]. Therefore, the UE supports an HTTP/3 client and the UPF supports an HTTP/3 proxy, both of them operating over QUIC.

-	On each of the established QUIC connections, the UE sends an extended HTTP CONNECT request to UPF (as defined in [x1]) indicating that IP proxying over HTTP is needed.

-	When the UE wants to transmit an uplink packet of an IP flow, the UE:

-	Selects which QUIC connection will be used for the uplink traffic of the IP flow based on the QoS flow associated with the IP flow;

-	Creates a new bidirectional QUIC stream on the selected QUIC connection;

-	Configures the QUIC stream to apply a steering mode (i.e., the steering mode that should be used for the uplink traffic of the IP flow based on the ATSSS rules); and

-	 Forwards to UPF the uplink packets of the IP flow using multipath QUIC transport.

-	When the UPF wants to transmit a downlink packet of an IP flow, the UPF:

-	Selects which QUIC connection will be used for the downlink traffic of the IP flow (this QUIC connection is the same as the one selected by UE for the IP flow, assuming the QoS flow in UL and DL directions is the same);

-	Selects a bidirectional QUIC stream on the selected QUIC connection (this QUIC stream is the same as the one created by the UE for the UDP flow);

-	Configures the QUIC stream to apply a steering mode (i.e., the steering mode that should be used for the downlink traffic of the IP flow based on the N4 rules); and

-	Forwards to UE the downlink packets of the IP flow using multipath QUIC transport.

The following figure illustrates how the traffic of an IP flow is transferred between the UE and UPF using multipath QUIC transport [9]. In this figure, it is assumed that the uplink traffic and the downlink traffic of the IP flow are mapped to the same QoS flow (QFI-1). Therefore, both the uplink traffic and the downlink traffic of the IP flow use the same multipath QUIC connection.

Figure 6.X.2-1: Using multipath QUIC transport for a IP flow

The bidirectional QUIC stream is established by the UE to enable transmission of uplink IP packets (blue) and downlink IP packets (red) of the IP flow. The UE configures this stream to send uplink traffic with a steering mode determined based on the ATSSS rules in the UE (uplink steering mode). The UPF configures this stream to send downlink traffic with a steering mode determined based on the N4 rules in the UPF (downlink steering mode). The data packets of the IP flow shown in Fig. 6.X.2-1 are transmitted in datagram mode, i.e., they are encapsulated in HTTP datagrams and QUIC DATAGRAM frames, each one carrying header information (see the Quarter Stream ID defined in draft-ietf-masque-h3-datagram [x3]) that associates it with the established bidirectional stream. As discussed below, the IP packets of the IP flow may be transmitted in stream mode (instead of datagram mode), i.e., transmitted directly over the bidirectional stream. In this case, the IP packets of the IP flow are encapsulated in DATAGRAM capsules (see [x3]) and QUIC STREAM frames.

Figure 6.X.2-2 and Figure 6.X.2-3 illustrate the components of the MPQUIC-IP steering functionality used to support data transmission in the uplink and downlink direction respectively. The MPQUIC-IP steering functionality is composed of three components:

1)	QoS flow selection & Steering mode selection: This component in the UE initiates the establishment of one or more QUIC connections, after the establishment of the MA PDU Session and, for each uplink IP flow, it selects a QoS flow (based on the QoS rules), a steering mode (based on the ATSSS rules) and a transport mode (see further details below). This component in the UPF selects, for each downlink IP flow, a QoS flow (based on the N4 rules), a steering mode (based on the N4 rules) and a transport mode (see further details below)

In the UE, this component is only used in the uplink direction, while, in the UPF, this component is only used in the downlink direction.

2)	HTTP/3 layer: Supports the HTTP/3 protocol defined in draft-ietf-quic-http [x2] and the extensions defined in:

-	draft-ietf-masque-connect-ip [x1] for supporting IP proxying over HTTP; and

-	draft-ietf-masque-h3-datagram [x3] for supporting HTTP datagrams.

The HTTP/3 layer selects a QUIC connection to be used for each IP flow and allocates a new QUIC stream on this connection that is associated with the IP flow. It also configures this QUIC stream to apply a specific steering mode (further details are provided below).

In the UE, the HTTP/3 layer implements an HTTP/3 client, while, in the UPF, it implements an HTTP/3 proxy.

3)	QUIC layer: Supports the QUIC protocol as defined in the applicable IETF specifications (RFC 9000 [6], RFC 9001 [7], RFC 9002 [8]) and the extensions defined in:

-	RFC 9221 [9] for supporting QUIC datagram transport; and

### -	draft-ietf-quic-multipath [10] for supporting QUIC connections using multiple paths simultaneously.

Figure 6.X.2-2: Components of MPQUIC-IP steering functionality used for UL data transmission

Figure 6.X.2-3: Components of MPQUIC-IP steering functionality used for DL data transmission

The protocol stack of the solution is depicted in Figure 6.X.3-4 below.

Figure 6.X.2-4: Protocol stack of the solution

### 6.X.3	Procedures

Figure 6.X.3-1 below depicts the key steps of the procedure that enables data traffic to be exchanged between the UE and UPF using the MPQUIC-IP steering functionality.

Figure 6.X.3-1: Procedure for enabling data traffic using the MPQUIC-IP steering functionality

1.	The UE establishes a MA PDU Session with the 5G core (5GC) network. During the MA PDU Session establishment:

-	In the PDU Establishment Request message, the UE indicates that it supports the MPQUIC-IP steering functionality. This indication can be used by the network (a) to select a UPF that also supports the MPQUIC-IP steering functionality and (b) to decide whether the ATSSS/N4 rules for the MA PDU Session may use the MPQUIC-IP steering functionality.

-	The UE receives MPQUIC-IP proxy information, i.e., one IP address of UPF, one UDP port number and the proxy type (e.g., "connect-ip"). This information is used by the UE for establishing QUIC connections with the UPF, which is also referred to as "MPQUIC-IP proxy".

-	The UE receives one IP address/prefix for the MA PDU Session and two additional IP addresses/prefixes, called "link-specific multipath QUIC" addresses; one associated with 3GPP access and another associated with the non-3GPP access. These two addresses can be used by the UE to create two paths in a multipath QUIC connection.

-	The UE receives QoS rules and ATSSS rules to be applied for the MA PDU Session, for QoS enforcement and traffic steering enforcement respectively. Similar rules (N4 rules) are received by UPF.

2.	After the MA PDU Session is established and the UE identifies that one or more ATSSS rules require traffic steering using the MPQUIC-IP steering functionality, the UE determines the number of multipath QUIC connections to be established with the UPF (MPQUIC-IP proxy). For example, the UE may determine to establish as many multipath QUIC connections, as the number of QoS flows of the MA PDU Session, i.e., one multipath QUIC connection per QoS flow. The QoS rules provided to UE include downlink QoS information and the UE applies the downlink QoS information to establish QUIC connections for the QoS flows used for downlink traffic only.

3.	The UE establishes the number of multipath QUIC connection with the UPF (MPQUIC-IP proxy) determined in the previous step. This results into several multipath QUIC connections between the UE and UPF, each one composed of multiple paths, e.g., one path over 3GPP access and another path over non-3GPP access.

NOTE 1:	Data transmitted over a multipath QUIC connection must be encrypted according to RFC 9001 [7]. However, encryption might not be necessary when the multipath QUIC connection is established between UE and UPF, because the underlying 5G security mechanisms can be applied. It is up to SA3 to investigate this issue and to determine whether and how encryption in the QUIC layer can be omitted.

During a QUIC connection establishment, the UE and UPF negotiate QUIC transport parameters and indicate (a) support of QUIC Datagram frames and (b) support of multipath. They indicate support of QUIC Datagram frames by providing the "max_datagram_frame_size" transport parameter with a non-zero value (see RFC 9221 [9]) and they indicate support of multipath by providing the "enable_multipath" transport parameter (see draft-ietf-quic-multipath [10]).

After a QUIC connection establishment, the HTTP/3 client and the HTTP/3 proxy negotiate HTTP settings and indicate support of HTTP Datagrams (see draft-ietf-masque-h3-datagram [x3]) and support of Extended CONNECT (see draft-ietf-httpbis-h3-websockets [x4]).

The QoS flow associated with a QUIC connection is also negotiated between the UE and UPF. This is done either by using a new QUIC transport parameter (defined by 3GPP) when the QUIC connection is established, or by using a new HTTP/3 SETTINGS parameter (defined by 3GPP) when the HTTP settings are exchanged between the HTTP/3 client in the UE and the HTTP/3 proxy in the UPF.

Editor’s note: It is FFS whether the QoS flow associated with a QUIC connection is also negotiated with a new QUIC transport parameter (defined by 3GPP) or with a new HTTP/3 SETTINGS parameter (defined by 3GPP).

4.	On each of the established QUIC connections, the UE sends an extended HTTP CONNECT request to UPF (as defined in [x1]) indicating that IP proxying over HTTP is needed. If this is accepted by UPF, it responds with a 200 status.

5.	The UE generates a new IP packet (IP packet #1) that should be sent via the MA PDU Session. This IP packet initiates a new IP flow, i.e., a sequence of IP packets using the same 5-tuple. For this new IP flow (and for each new IP flow):

-	The UE selects a QoS flow (QFI) over which the IP flow should be transmitted. This is selected by using the received QoS rules.

-	The UE selects a steering mode that should be applied for the IP flow. This is selected by using the received ATSSS rules.

-	The UE selects a transport mode, either a datagram transport mode or stream transport mode. The datagram transport mode provides unreliable transport and may result in data delivery with out-of-order packets and/or with duplicated packets. In this transport mode, the packets of the IP flow are encapsulated in QUIC DATAGRAM frames [9]. The stream transport mode provides reliable transport and supports data delivery without out-of-order packets and without duplicated packets. In this transport mode, the packets of the IP flow are encapsulated in DATAGRAM capsules (see [x3]) and in QUIC STREAM frames. The stream transport mode is particularly useful for supporting per-packet splitting, i.e., when the packets of a UDP flow are split across multiple accesses and may be received out-of-order.

Editor's note: It is FFS how the transport mode is selected. It is possible to include the transport mode in the ATSSS/N4 rules, in which case the transport mode is selected by the PCF.

Editor’ note: To avoid out-of-order / duplication delivery, the HTTP/3 proxy/client can prefix the IP packet with a sequence number before passing it to the QUIC layer and use the datagram transport mode. It is FFS whether this alternative can be applied instead of the stream transport mode.

6.	The UE selects a multipath QUIC connection to be used for the new IP flow (e.g., based on the selected QFI) and the UE allocates a new QUIC stream (e.g., stream 40) in this multipath QUIC connection. This new stream is associated with the new IP flow. The UE configures the new stream to transmit data traffic using the selected steering mode for this IP flow.

9. The UE sends the IP packet using the allocated new stream on the selected QUIC connection.

When the datagram transport mode is selected, the UE encapsulates the IP packet within an HTTP DATAGRAM frame [x3], which is transferred inside a QUIC DATAGRAM frame [9]. The header of the HTTP DATAGRAM indicates that this datagram is associated with stream 40 (i.e., the Quarter Stream ID is set to 10).

When the stream transport mode is selected, the UE encapsulates the IP packet within an HTTP DATAGRAM frame [x3] that is further encapsulated in a DATAGRAM capsule (see clause 4.4 in [x3]), which is transferred inside a QUIC STREAM frame [6].

In the example procedure shown in Figure 6.X.3-1, the datagram transport mode is selected. The UE sends a QUIC DATAGRAM frame to UPF that encapsulates the IP packet, which is forwarded by UPF to the remote host.

11.	When an IP packet is received by UPF (MPQUIC-IP proxy) from the remote host (IP packet #2), this IP packet is transferred to the UE using the established context information for the IP flow, i.e., using the selected multipath QUIC connection, the selected stream on this connection, the selected steering mode and the selected transport mode. Such context information is stored in the UPF and in the UE and is applied for all the IP packets of the IP flow.

12.	Similarly, when another IP packet is generated by the UE app (IP packet #3), this IP packet is transferred to UPF (MPQUIC-IP proxy) using again all the stored context information for the IP flow.

Note that the context information for an IP flow in the UE and in the UPF is created when the initial IP packet (i.e., IP packet #1) of this IP flow is transferred. All subsequent IP packets of the same IP flow are transferred between the UE and the UPF using this context information.

When the UE identifies that the context information for an IP flow is no longer needed, the UE deletes this context information and releases the associated QUIC stream, which cause the UPF to delete the context information stored in UPF.

The following figure 6.X.3-2 illustrates an example of how the uplink traffic of various IP flows is transferred from the UE to UPF over QUIC multipath transport, and how the UPF relays this traffic to a final destination (remote host). Note that for each IP flow there is an associated QUIC connection and an associated bidirectional QUIC stream, which is configured to apply a specific steering mode for the uplink traffic. For example, the blue IP flow is associated with the multipath QUIC connection #1 and with the blue Stream X, which is configured to apply the Active-Standby steering mode in the uplink direction.

All UDP flows shown in this figure, except the blue IP flow, are transferred with the datagram transport mode, i.e., their data packets are transferred inside QUIC DATAGRAM frames. The blue IP flow is transferred with the stream transport mode, i.e., its data packets are transferred inside QUIC STREAM frames.

The downlink traffic of IP flows is transferred between the UE and UPF in a similar way.

Figure 6.X.3-2: Example of user-plane operation using the MPQUIC-IP steering functionality (UL direction)

### 6.X.4	Impacts on Existing Nodes and Functionality

### Editor's note: FFS.

End of Changes